# GEORGIA SURFACE WATER AND GROUNDWATER QUALITY MONITORING AND ASSESSMENT STRATEGY

White County Georgia, Double Culvert Creek			PHOTO:  Roy Burke

Georgia Department of Natural Resources

Environmental Protection Division

Watershed Protection Branch

2 Martin Luther King Jr. Drive
Suite 1152, East Tower
Atlanta, GA 30334

# PREFACE

The Georgia Environmental Protection Division (GAEPD) of the Department of Natural Resources (DNR) developed this document entitled “Georgia Surface Water and Groundwater Quality Monitoring and Assessment Strategy”.  As a part of the State’s Water Quality Management Program, this report focuses on GAEPD’s water quality monitoring efforts to address key elements identified by the U.S. Environmental Protection Agency (USEPA) monitoring strategy guidance entitled “Elements of a State Monitoring and Assessment Program, March 2003”.

This report updates the State’s water quality monitoring strategy as required by the USEPA’s regulations addressing water management plans of the Clean Water Act, Section 106(e)(1).

Georgia Department of Natural Resources

Environmental Protection Division

Watershed Protection Branch

2 Martin Luther King Jr. Drive
Suite 1152, East Tower
Atlanta, GA 30334

# TABLE OF CONTENTS

TABLE OF CONTENTS	1

INTRODUCTION	1

1.  MONITORING PROGRAM STRATEGY OVERVIEW	3

Assessment of Water Quality	5

History of Georgia’s Water Quality Monitoring Programs	5

Data Management, Assessment, and Reporting	6

Future Issues and Challenges	7

Strategy Implementation Challenges	8

2.  MONITORING OBJECTIVES	9

3.  MONITORING DESIGN	11

State-wide Trend Monitoring.	11

Targeted Monitoring	11

Probabilistic Monitoring	12

Lake/Reservoir Monitoring	12

Estuary Monitoring	12

DNR State Park Beach Monitoring	13

Biological Monitoring	13

Fish Tissue Monitoring	14

Aquatic Toxicity Monitoring	14

Toxic Substance Monitoring	15

Intensive Surveys Monitoring	16

Groundwater Well Monitoring	16

Facility Compliance Monitoring	16

4.  CORE AND SUPPLEMENTAL WATER QUALITY INDICATORS	18

5.  QUALITY ASSURANCE	20

Laboratory	20

Quality Assurance	20

6.  DATA MANAGEMENT	21

7.  DATA ANALYSIS/ASSESSMENT	22

Designated Uses and Water Quality Standards	22

8.  REPORTING	24

9.  PROGRAMMATIC EVALUATION	25

10.  GENERAL SUPPORT AND INFRASTRUCTURE PLANNING	26

Resources	26

Future Studies and Actions	27

LIST OF TABLES

TABLE 1. GEORGIA WATER RESOURCES ATLAS……………………………………5

TABLE 2. WATER QUALITY INDICATORS......……….………………………………..19

APPENDICES

APPENDIX A: SURFACE WATER QUALITY MONITORING STATIONS AND GROUNDWATER WELLS

1a. Calendar Year 2025 Monitoring Stations – Rivers/Streams, Lakes/Reservoirs,                                      Estuaries/Sounds

1b. Calendar Year 2026 Monitoring Stations – Rivers/Streams, Lakes/Reservoirs,                                      Estuaries/Sounds

Coastal Shellfish Monitoring Stations

- Coastal Shellfish Monitoring Stations

- Coastal Beach Monitoring Stations

- DNR State Parks Lake Beach Monitoring Stations

- Mercury in Fish Trend Monitoring Stations

- Parameters For Fish Tissue Testing

- Groundwater Monitoring Wells

APPENDIX B: WATER USE CLASSIFICATIONS AND WATER QUALITY STANDARDS APPROVED BY EPA

# INTRODUCTION

The purpose of the Georgia Surface Water Quality Monitoring and Assessment Strategy (Strategy) is to outline the State’s ambient water quality monitoring program, which addresses the 10 Elements recommended by the USEPA for a State Water Monitoring and Assessment Program.  Currently, the Georgia water quality monitoring and assessment program includes a number of different aspects including: baseline or trend monitoring;  planning monitoring or intensive surveys; effectiveness monitoring; probabilistic stream monitoring; lake monitoring; coastal shellfish monitoring; estuary monitoring; coastal and freshwater beach monitoring; toxic substance monitoring; fish tissue monitoring; periphyton, macroinvertebrate and fish community assessment; habitat assessment; and facilities monitoring. These monitoring tools provide Georgia with a comprehensive, long-term monitoring program that serves the water quality management needs and addresses all water body types designated as State waters, including rivers, streams, lakes, reservoirs, estuaries, wetlands, groundwater, and coastal areas.

Water Quality Monitoring and Assessment Strategy Vision, Mission Statement and Goals Vision: To gather information essential to develop indicators and standards to protect human health, aquatic life, and the environment in Georgia. Mission Statement: To implement a monitoring strategy that includes assessment of water quality conditions within Georgia, leads to the development of water quality standards and corrective actions to restore impacts identified through monitoring initiatives taken, and effectively communicates this information to both internal and external customers. Goals: - Measure the physical, chemical, and biological conditions of waters in all river basins within Georgia and identify causes responsible for water quality impairments. - Assess the impact from human and other activities within the watersheds and the effects these activities are having on the overall ecosystem. - Identify and recommend corrective action measures to restore waters to meet designated uses. - Report water quality assessments in support of the management program to customers and stakeholders.

Challenges in fully implementing the Strategy include obtaining sufficient personnel to accomplish the monitoring and assessment goals of the program; refining our database system as needed in order to enhance its storage, retrieval, and analysis capabilities; and coordinating and managing internal and external information and data gathering and assessment.

Key environmental issues and challenges facing the State currently and in future years include: (1) controlling toxic substances in water; (2) ensuring a sustainable and safe supply of potable water; (3) managing nutrient discharges; (4) reducing nonpoint source pollution; and, (5) increasing public involvement in water quality improvement projects.

# 1.  MONITORING PROGRAM STRATEGY OVERVIEW

Surface water and groundwater resources are extremely important to the life, health, and economy of Georgia.  According to USEPA estimates based on the U.S. Geological Survey 1:100,000 Digital Line Graph, the State has 44,056 miles of perennial streams, 23,906 miles of intermittent streams, and 603 miles of ditches and canals for a total of 70,150 stream miles.  The State also has 4.8 million acres of wetlands (9% tidally affected), 425,582 acres of public lakes and reservoirs, 854 square miles of estuaries, and 100 miles of coastline.  Water resources estimates for Georgia are summarized in Table 1.

TABLE 1. GEORGIA WATER RESOURCES ATLAS

| State Population | 10,097,340 |
| --- | --- |
| State Surface Area | 57,906 square miles |
| Number of Major River Basins | 14 |
| Number of Perennial River Miles | 44,056 miles |
| Number of Intermittent River Miles | 23,906 miles |
| Number of Ditches and Canals | 603 miles |
| Total River Miles | 70,150 miles |
| Number of Lakes Over 500 Acres | 48 |
| Acres of Lakes Over 500 Acres | 265,365 acres |
| Number of Lakes Under 500 Acres | 11,765 |
| Acres of Lakes Under 500 Acres | 160,017 acres |
| Total Number of Lakes & Reservoirs, Ponds | 11,813 |
| Total Acreage of Lakes, Reservoirs, Ponds | 425,382 acres |
| Square Miles of Estuaries | 854 square miles |
| Miles of Coastline | 100 |
| Acres of Freshwater Wetlands | 4,500,000 acres |
| Acres of Tidal Wetlands | 384,000 acres |

Georgia has 14 major river basins within the State.  These are the Altamaha, Chattahoochee, Coosa, Flint, Ochlockonee, Ocmulgee, Oconee, Ogeechee, St. Marys, Satilla, Savannah, Suwannee, Tallapoosa, and Tennessee River Basins. The rivers in Georgia provide the water needed by aquatic organisms, animals, and humans to sustain life.  These waters also provide significant recreational opportunities, are used for industrial purposes, drive turbines to provide electricity, and assimilate wastes.

There are nine major aquifer systems in Georgia including the Cretaceous, Providence, Clayton, Clairborne, Jacksonian, Floridan, Miocene, Piedmont/Blue Ridge, and Valley and Ridge unconfined aquifer systems. Groundwater makes up 22 percent (based on 2015 estimates) of the public water supply, 100 percent of rural drinking water sources, 76 percent of the irrigation use, and 42 percent of the industrial and mining use.  Total groundwater withdrawals in 2015 were approximately 1.15 billion gallons per day.  For practical purposes, outside the larger cities of the Piedmont, groundwater is the dominant source of drinking water.  Additional information on groundwater monitoring and management can be found in the Georgia Groundwater Management Plan.

Managing these resources requires up-to-date data and information to develop long-range planning strategies to safeguard water quality and quantity for future needs.  The Watershed Protection Branch of GAEPD, in cooperation with many local, State, and Federal agencies, coordinates programs to address most aspects of water pollution control.  These include: water quality modeling to develop wasteload allocations (WLAs) and total maximum daily loads (TMDLs); TMDL implementation planning; comprehensive water management planning; water quality standards development; local watershed assessment and watershed protection planning; nonpoint source management; erosion and sedimentation control; storm water controls; National Pollutant Discharge Elimination System (NPDES) permit and enforcement program administration for municipal and industrial point sources; industrial pretreatment permitting; and land application of treated wastewater permitting.

Water quality monitoring and assessment is the foundation for the measurement of success for the various water protection programs.  The Monitoring and Assessment Strategy encompasses development of: (1) monitoring objectives; (2) assessment tools for attainment of water quality standards; (3) evaluation measures for state-wide water quality; (4) procedures for establishing, reviewing, and revising water quality standards; (5) measures to support water management programs; (6) Quality Assurance protocols and procedures; and, (7) programmatic data management and reporting procedures.

Georgia’s comprehensive monitoring program and strategy is designed to serve the State’s water quality management needs and to address all State waters including rivers, streams, lakes, reservoirs, estuaries, wetlands, groundwater, and coastal areas.  The monitoring program includes baseline or trend monitoring; planning monitoring or intensive surveys; effectiveness monitoring; probabilistic stream monitoring; lake monitoring; coastal shellfish monitoring; estuary monitoring; coastal and freshwater beach monitoring; toxic substance monitoring; fish tissue monitoring; periphyton, macroinvertebrate and fish community assessment; habitat assessment; and facilities monitoring.  The monitoring program is long-term in nature.

Monitoring program changes and enhancements occur throughout the year, as needed, to address specific acute issues. Larger programmatic changes are considered annually, along with available resources, and are implemented, as appropriate, in conjunction with the annual change in focus.  These annual changes provide milestones or progress markers that are discussed in the State/EPA Performance Partnership Agreements (PPA). The annual planning process in preparing the PPA provides an opportunity for annual review of implementation priorities in-line with available resources to address the priorities.  In addition, the overall strategy for monitoring and assessment is reviewed and updated every three to five years.

This strategy along with the biennial report, “Water Quality in Georgia” (CWA 305(b) Report), and annual State/EPA Performance Partnership Agreements provide a process for communication of monitoring priorities to other State and Federal organizations and the public.  The strategy herein addresses goals, objectives, design, indicators, quality assurance, data management, data analysis, reporting, program evaluation, and general support and infrastructure needs.

## Assessment of Water Quality

Assessment of water quality requires a baseline for comparison.  Water quality data is collected and assessed against Georgia’s water quality standards, which contain designates uses that establish the environmental use of the waterbody, narrative and numeric criteria for both general and chemical constituents, and anti-degradation policies for water quality.  Georgia’s waters are currently categorized as one of the following designated uses: drinking water, recreation, fishing, coastal fishing, wild river, or scenic river.  Specific water quality standards are assigned to support each designated use.  The quality of Georgia’s waters is judged by the extent to which the waters support the uses (comply with standards set for the designated uses) for which they have been designated.

## History of Georgia’s Water Quality Monitoring Programs

In the 1960s, one of the first major efforts in Georgia to combat water pollution was the initiation of monitoring programs to document water quality conditions, assess compliance with water quality standards, and collect data for use in enforcement actions. In the 1970s, the monitoring programs focused on municipal and industrial point source issues and studies to determine the treatment levels required to meet water quality standards.  In the 1980s, GAEPD intensified toxic substance monitoring across the State.  The expanded toxic substance program included facility effluent, stream, sediment, and fish sampling at sites downstream of selected industrial and municipal discharges.  Georgia also initiated biomonitoring or aquatic toxicity testing.  All major industrial and municipal discharges were tested.  Where toxic substances were identified in a treated discharge or impacts documented in a stream, GAEPD incorporated specific limitations in the NPDES permit.

The 1990s saw the initiation of a number of comprehensive lake studies performed in publicly owned lakes in excess of 1000 acres, which culminated in the establishment of water quality standards for pH, chlorophyll a, total nitrogen, and  total phosphorus loadings for a number of lakes across Georgia. In addition, total phosphorus limits were also established for major tributary streams to these lakes.  Fish tissue monitoring was significantly expanded.  The first risk-based fish consumption guidance (Georgia Freshwater and Saltwater Sport Fishing Regulations and Guidelines for Eating Fish for Georgia Waters) was published in 1995.  In the mid-1990s, Georgia implemented a rotating basin approach to chemical water quality monitoring.  Georgia also intensified biological monitoring in the late 1990s with assessments of fish and macroinvertebrate communities on an ecoregion basis.   Georgia completed one full river basin rotation cycle in 2000 with targeted monitoring in each of the five major river basin groups.

Since 1999, DNR’s Coastal Resources Division (CRD) has conducted census bacteria monitoring of Georgia’s popular swimming beaches on Tybee, St. Simons, Jekyll, and Sea Island.  In response to EPA’s Beaches Environmental Assessment and Coastal Health (BEACH) Act (PL 106-284), Georgia expanded its monitoring efforts with the development of the Coastal Beach Monitoring Program implemented by CRD in coordination with county health departments of each Georgia coastal county.  CRD sampling teams began collecting of samples from Georgia beaches for bacterial analysis and developed a public notification system based on EPA’s recommended levels of enterococcus for marine recreational waters.

In 2004, the Georgia General Assembly passed the Comprehensive State-wide Water Management Planning Act, which called for the preparation of a comprehensive state-wide water plan and provided fundamental goals and guiding principles.  This resulting Georgia Comprehensive State-wide Water Management Plan (State Water Plan) was adopted by the General Assembly in 2008.  Part of this plan included expanding monitoring and information gathering including the acquisition of additional stream gages, personnel, and equipment for water quality monitoring.  In November 2011, ten Regional Water Plans were officially adopted by GAEPD. Beginning in late 2015, the Councils began reviewing their plans based on updated water and wastewater demand forecasts for the Municipal, Agricultural and Energy sectors, as well as updated resource assessment information. Based on this review, the Councils updated their Regional Water Plans, which were adopted by EPD in July 2017. The plans were again revised in June 2023 based on new forecast and resource assessment information. All Regional Water Plans are subject to periodic review and revision on a 5-year cycle, and the Councils are currently scheduled to update their Plans again in 2027. These Regional Water Plans outlined management practices to meet future water needs, including calls for additional environmental monitoring.  In the 2000s, GAEPD significantly expanded water monitoring efforts to support regional water planning efforts, including hiring 11 new monitoring staff and establishing field offices in Atlanta, Augusta, Brunswick, Tifton, and Cartersville.

## Data Management, Assessment, and Reporting

Data collected by GAEPD and its cooperators are stored in a centralized database known as the Georgia EnvirOnmental Monitoring and Assessment System (GOMAS).  GOMAS is a web-accessible repository of chemistry, physical, and biological data collected by GAEPD’s Watershed Protection Branch, as well as outside entities under contract and/or agreement with GAEPD. GOMAS currently houses the following information: surface and ground water chemical and physical data collected by GAEPD’s Ambient Monitoring and Facilities Monitoring Units; biological data collected by GAEPD’s Ambient Monitoring Unit; visual assessment and other descriptive metadata (such as land use information) that contextualize conditions during GAEPD monitoring activities; surface water chemical and physical data collected by USGS, Columbus Water Works, and various counties and municipalities as specified via contract or terms contained within watershed protection plans; and information pertaining to waters on the 305(b) and 303 (d) lists.  In addition, GOMAS contains an interactive map that allows users to quickly find active and historic monitoring locations using a multitude of search criteria.  In 2019, a public portal to GOMAS was established to provide data access to the general public through GAEPD’s website at https://gomaspublic.gaepd.org/Home/GOMAS_Home. Physical, chemical and biological data collected by GAEPD are uploaded to the USEPA Water Quality Exchange (WQX) database.  The USEPA WQX database provides an alternative electronic Internet portal to GAEPD data.  Some GAEPD data and data from outside sources are maintained in paper files and are available for public review at any time.

A number of monitoring programs provide data for assessing attainment of water quality standards in rivers, streams, lakes, estuaries, coastal waters, and beaches in Georgia.  Existing and readily available data and information are reviewed every two years and the Georgia 305(b)/303(d) list of waters is updated and publicly noticed for comment.  In addition to data collected by GAEPD and its cooperators, data from third parties (such as municipalities and environmental groups) are used as long as the quality control requirements found in Chapter 391-3-6-.03(13) are met. Data and information that does not meet quality control requirements are used as screening information and may be used during the process of selecting sites for GAEPD or cooperator monitoring.

Georgia produces reports and lists in accordance with CWA requirements in a timely and complete manner.  The CWA [Section 305(b)] requires states to assess and characterize the condition and trends of all waters within the State.  The CWA [Section 303(d)] requires states to identify impaired waters for which TMDLs are needed.  Georgia has integrated the two reporting requirements since the late 1980s.  The Section 305(b) Report and the Section 303(d) list are due in even numbered years.  Assessment data is transferred from GOMAS to EPA’s ATTAINS database via the Exchange Network.    The most current integrated 305(b)/303(d) list of waters and report (also known as the “Water Quality in Georgia”) is available on the GAEPD website along with an interactive Story Map of the report that can be found at: https://gaepd.maps.arcgis.com/apps/MapSeries/index.html?appid=dea4c9c319d4461c8d5cef8e68957b1b.

## Future Issues and Challenges

The key issues and challenges to be addressed now and in future years include (1) the control of toxic substances including those currently not regulated; (2) a sustainable and safe supply of potable water; (3) the management of nutrient discharges; (4) the reduction of nonpoint source pollution; and (5) the need to increase public involvement in water quality improvement projects.

The reduction of toxic substances in rivers, lakes, sediment and fish tissue is extremely important in protecting both human health and aquatic life.  The sources are widespread.  The most effective method to reduce the releases of toxic substances into rivers is pollution prevention that consists primarily of eliminating or reducing the use of toxic materials or at least reducing the exposure of toxic materials to drinking water, wastewater, and storm water.  It is very expensive and difficult to reduce low concentrations of toxic substances in wastewaters by treatment technologies.  And it is virtually impossible to treat large quantities of storm water and reduce toxic substances.  Therefore, toxic substances must be controlled at the source.

The dramatic increase in growth and population within Georgia is making considerable demands on Georgia’s groundwater and surface water resources.  The problems and issues are further complicated by the fact that surface water resources are limited in south Georgia and groundwater resources are limited in north Georgia.  In some locations, resources are approaching their sustainable limits.  Water management planning based on Georgia’s Comprehensive State-Wide Water Plan provides for management of water resources in a sustainable manner to support the State’s economy, to protect public health and natural systems, and to enhance the quality of life for all citizens.

Nutrient over-enrichment is defined as the accumulation of nutrients from human activities and natural sources that impairs the beneficial uses of a waterbody.  Historically, Georgia has addressed nutrient issues on a site-specific basis in response to documented water quality impairments.  The implementation of the supplemental lake water quality standards for the six major publicly owned lakes has led to nutrient control strategies in their respective watersheds.  Georgia has also been proactive in managing nutrients discharged from permitted surface water discharges to potentially nutrient sensitive waters.  In 2011, GAEPD developed a Strategy for Addressing Phosphorus in NPDES Permitting to limit phosphorus from new or expanding wastewater treatment plants that discharge phosphorus. This Strategy was updated in June 2025. As part of the strategy all POTWs have been given total phosphorus limits. In addition, GAEPD developed a Nutrient Roadmap outlines GAEPD’s comprehensive Nutrient Reduction Strategy for point and nonpoint source discharges.

GAEPD will ultimately develop and adopt numeric nutrient criteria for all major lakes and estuaries in Georgia.  Monitoring to provide the scientific basis for the development of these standards and quantifying biological response to nutrient over-enrichment is an on-going focus.

Nonpoint source pollution affects Georgia’s streams and consists of sediment, litter, bacteria, pesticides, fertilizers, metals, oils, surfactants, and a variety of other pollutants discharged into rivers and lakes by storm water.  As with toxic substance control, nonstructural techniques such as pollution prevention and best management practices must be significantly expanded.  These include both watershed protection through planning, zoning, buffer zones, and appropriate building densities, as well as increased use of green infrastructure, storm water retention ponds, street cleaning, and limitations on pesticide and fertilizer usage.

GAEPD will continue to work aggressively to emphasize public involvement, not only in decision-making, but also in direct programs of water body improvement.  Georgia has active public outreach programs within the Watershed Protection Branch.  Staff within the Branch’s Nonpoint Source Program promote Adopt-A-Stream programs, which includes providing extensive training to volunteer groups, and Project WET (Water Education for Teachers) a program for bringing water conservation and pollution prevention education to the classrooms.

## Strategy Implementation Challenges

Challenges in implementing the Strategy are tied directly to funding.  Georgia’s monitoring programs are designed and operated to allow full implementation within the current GAEPD budget.  The budget cycle for Georgia is one year. The budget may change from year to year and programs are increased or reduced, as appropriate.

To accomplish the monitoring and assessment goals of the program, obtaining sufficient personnel is a critical issue.  Without sufficient personnel and resources, the program will be limited in the scope of evaluation and assessment that can be accomplished.  In addition, to manage the data collected and to statistically analyze data for trends, an expanded database management system is essential.  Coordination and management of internal and external information and data gathering and assessment also requires a staffing commitment by GAEPD to accomplish the goals and objectives of the project.

When additional resources become available, GAEPD expands the monitoring programs to include additions to the scope of work, adding sites for monitoring and/or implementing different types of monitoring to complement existing programs.  At that time, equipment and other resource needs are evaluated and additions to the data management capabilities are considered.

# 2.  MONITORING OBJECTIVES

The State’s monitoring program integrates physical, chemical, and biological monitoring to provide information for water quality management needs and addresses all State waters and water body types.  For the State to be efficient and effective in generating data that serve its management decision needs and to be consistent with the objectives of the Clean Water Act, Georgia has identified the following monitoring objectives:

- Establishing, reviewing, and revising water quality standards in accordance with Section 303(c) of the Clean Water Act.

- Determining water quality standards attainment in accordance with Section 305(b) of the Clean Water Act.

- Identifying impaired waters in accordance with Section 303(d) of the Clean Water Act.

- Identifying causes and sources of water quality impairments in accordance with Sections 303(d) and 305(b) of the Clean Water Act.

- Supporting the implementation of water management programs in accordance with Sections 303, 314 and 319 of the Clean Water Act.

- Supporting the evaluation of program effectiveness in accordance with Sections 303, 305, 402, 314, and 319 of the Clean Water Act.

GAEPD uses baseline, planning, and effectiveness monitoring to meet the objectives of the Strategy.  To fulfill these monitoring objectives, GAEPD utilizes multiple monitoring programs including: state-wide trend monitoring, probabilistic monitoring, TMDL monitoring, intensive surveys monitoring, lake monitoring, coastal monitoring, biological monitoring, fish tissue monitoring, toxic substance monitoring, facility compliance sampling, and groundwater monitoring.

A brief description of the monitoring programs is provided below.

- Baseline:  Probabilistic, targeted ambient, and long-term trend sampling of state-wide waterbodies at fixed stations.  The data from these stations provide an historic record of water quality.  Monitoring at these locations may be repeated annually. Monitoring state-wide allows for comparison of similar sites within basins during different hydrologic and climatological conditions (i.e. drought, normal, and high rain years).

- Planning: short-term, intensive surveys designed to gather data necessary for the development, calibration and/or refinement of water quality models, TMDLs, and wasteload allocations (WLAs).

- Effectiveness:  focused sampling of a select group of sites located state-wide to measure the status of water quality.  This targeted sampling is for waterbodies currently on the 303(d) list. Data is used to determine whether waterbodies meet their designated use once a TMDL has been completed and/or implemented. In addition, sampling may be conducted on a waterbody with prior monitoring data to determine whether it still meets its designated use or continues to be considered impaired (as applicable).

These monitoring programs are applied to all waters of the State in a manner that yields scientifically defensible results and meets the needs of the decision makers in GAEPD.  Many of our monitoring efforts are long-term in nature and are expected to be used in the future to the extent that resources are available.

# 3.  MONITORING DESIGN

Georgia has developed multiple monitoring designs for selecting sampling sites and gathering data that best serves the monitoring objectives.   Each of the monitoring types described below are a component of the monitoring programs discussed in section 2.

## State-wide Trend Monitoring.  Trend monitoring collects baseline data to document existing conditions,  determine water use impairment, and assess the environmental effectiveness of required pollution control programs and nonpoint source programs and projects.

The state-wide trend monitoring is long-term monitoring of streams at strategic locations throughout Georgia.  Trend monitoring is conducted by GAEPD associates and through cooperative agreements with Federal, State, and local agencies, which collect samples from groups of stations at specific, fixed locations throughout the year. Although there have been a number of changes over the years, much of the trend monitoring is still accomplished through cooperative agreements.  The lists of the sampling stations that make up the State’s Trend monitoring network are presented in Appendix A.

In addition to monthly stream sampling, GAEPD and its contractors manage several continuous monitoring stations throughout the State in support of baseline and planning monitoring efforts.  The list of continuous monitoring sites currently in operation is presented in Appendix A.

In recent years, GAEPD has incorporated a biological component to its trend monitoring program. Macroinvertebrates and periphyton are collected annually at specified locations to assess biological responses to various environmental changes over time.

## Targeted Monitoring.  The targeted monitoring program collects data used for planning to  develop TMDLs, WLAs, and study the impacts of specific discharges on water quality.  Targeted monitoring measures effectiveness by collecting data used to determine improvements resulting from upgraded water pollution control plants and implementation of nonpoint source best management practices (BMPs).

Each year new or repeat monitoring stations are selected state-wide based on needs and priorities.  State-wide selection allows for the collection of data during different climatic conditions in each basin.  Selection of these sites tends to be targeted.  Locations in minimally impacted areas, urban areas, agricultural and forested areas, along with stations downstream of wastewater treatment plant discharges are included each year as a part of the monitoring network to provide data and information on new locations and to extend the coverage of the monitoring program. Sites are often selected upstream and downstream from small publicly owned treatment works (POTWs) to determine compliance with the narrative toxicity criteria.  Temperature and pH of the stream are monitored along with instream ammonia concentrations to determine if the ammonia chronic and acute criteria recommended by EPA are being met.

Targeted sampling stations are often located on 303(d) listed segments where TMDLs and TMDL implementation plans have been prepared and/or 319 projects have been implemented  to determine if improvements in water quality have occurred.  Often this monitoring is contracted through grants with the Regional Development Centers or through cooperative endeavors by local municipal governments assisted by University projects.  Data obtained from TMDL monitoring efforts is used to assess water quality conditions in 303(d) listed waters and to measure the success of local restoration efforts.

## Probabilistic Monitoring. This type of monitoring design is used for making a statistically valid inference about the condition of various water types.  The sampling sites are randomly selected and a sufficient number of data points are collected to make a statistically based assessment of water quality within a region with similar land use and population characteristics.

## Lake/Reservoir Monitoring.  Lake baseline monitoring is conducted to assess the compliance of the six major lakes with site specific water quality criteria. The six major lakes with established water quality standards are Lake Lanier, Lake Walter F. George, West Point Lake, Lake Jackson, Lake Allatoona, and Carters Lake. These lakes are monitored monthly during the growing season from April through October. Field measurements are taken and include secchi depth and depth profiles of dissolved oxygen, temperature, pH and conductivity. Composite water quality samples are collected within the photic zone and analyzed for chlorophyll a, nutrients,  bacteria, and other standard chemical parameters. In addition, field measurements are collected, along with flow, and water quality samples in the major tributaries feeding these lakes.  These data are to protect both public health and aquatic life.

GAEPD also conducts planning monitoring in 28 publicly owned lakes greater than 500 acres annually from April through October. Sufficient data needs to be collected for standard development of numeric nutrient and chlorophyll criteria for these 28 basin lakes. The data collected on these lakes includes: secchi disk transparency, Li-Cor photic zone measurements, chlorophyll a, total phosphorus, nitrogen compounds, and turbidity. Depth profiles for temperature, dissolved oxygen, pH, and specific conductance are also measured at each monitoring location.

If additional resources become available, the lake and reservoir monitoring network may be expanded to include assessment of smaller publicly owned lakes and reservoirs in the State.

## Estuary Monitoring.  GAEPD annually conducts planning monitoring of eight estuaries during the growing season to determine their water quality status.  This monitoring supports the protection the public health and documents existing conditions. The data collected will be used to establish numeric nutrient criteria for the appropriate cause and response parameters and to determine appropriate dissolved oxygen criteria for these water bodies that have naturally low dissolved oxygen levels.

Coastal Monitoring.  The CRD conducts baseline water quality monitoring in estuarine and near-shore coastal waters through its Public Health Water Quality Monitoring Program. This program includes the Shellfish Sanitation and Beach Water Quality Monitoring Programs that are concerned with public health.  The CRD administers the Shellfish Sanitation Program under the guidance of the United States Food and Drug Administration’s (FDA) National Shellfish Sanitation Program (NSSP) standards that requires States to show that shellfish harvest areas are “not subject to contamination from human and/or animal fecal matter.” The CRD collects water samples and performs bacterial analysis to ensure that the area has fecal coliform levels below the established threshold of 14 MPN/100 mL.  Currently, the CRD monitors 84 stations for fecal coliform bacteria with site selection focusing on monitoring around harvest areas. Chatham, Liberty, McIntosh, Glynn, and Camden counties all have waterbodies designed as potential shellfish harvest areas and stations that are monitored.  These stations are monitored once a month at random tidal stages.

The CRD developed the Beach Monitoring Program to protect swimmer health. CRD does baseline beach monitoring and public notification based on EPA’s recommended levels of enterococcus for marine recreational waters. CRD has worked in partnership with local governments, the Jekyll Island Authority, and the Public Health Districts to develop procedures to notify the public about elevated bacteria levels. Public advisory signage has been installed at beach access points on Jekyll, St. Simons, and Tybee Islands. The Health Districts have prepared templates for press releases to issue health advisories in the event of elevated bacteria levels. CRD has placed beach information on the DNR website https://coastalgadnr.org/HealthyBeaches and has partnered with Earth 911 to show current beach conditions on their web site.  The CRD Coastal Beach Monitoring Program is ongoing and a list of beaches with Advisory Zones is provided in Appendix A.

## DNR State Park Beach Monitoring.  The DNR Parks, Recreation and Historic Sites Division (PRHSD) conducts baseline bacteria monitoring of their freshwater beaches to support the protection of human health.  The PRHSD operates public beaches on small lakes and reservoirs at several State parks in Georgia. State park beach monitoring of bacteria was conducted on a periodic park-by-park basis prior to 1996. Beginning in 1996, beach monitoring was conducted at the beginning of the season at the census State park freshwater inland beaches by DNR personnel. In 2020, the Georgia State Parks and Historic Sites Division began weekly E coli monitoring of the 33 State Park freshwater beaches during the recreational season from mid-April through Labor Day. A table of the DNR State Parks Lake Beach monitoring sites is provided in Appendix A.

## Biological Monitoring.  Biological monitoring is performed to assess the biological integrity of the State’s waters. Baseline, planning, and effectiveness biological monitoring is conducted by both GAEPD and the regulated communities. Regulated communities will report over time changes to the biological communities due to new development and/or reservoirs, and the effect of water quality protection measures.

Biological communities are sensitive to a wide array of direct stresses, including the effects of sedimentation, habitat loss, riparian zone disruption, flow modification and chemical pollution.  An Index of Biotic Integrity (IBI) is used to assess fish and macroinvertebrate community health in individual ecoregions in Georgia.  This index provides a direct and quantitative assessment of the biotic integrity of an aquatic community based on an overall evaluation of its fish and/or macroinvertebrate community in wadeable streams. In some cases, macroinvertebrates are a more sensitive community and reflect changes in stream quality before an impact of the fish community occurs.

In the 1990s, DNR’s Wildlife Resources Division (WRD) developed a fish community assessment that identified waters for the State’s 305(b)/303(d) listing, which ranked streams from very good to very poor as indicators of stream health. GAEPD has worked extensively for the last several years to develop a similar ranking assessment utilizing macroinvertebrates as an indicator organism.  This ranking will provide a broader picture of what is happening within Georgia’s waters and the resulting effects of pollution. Macroinvertebrate sampling is conducted in the fall/winter in wadeable rivers and streams. GAEPD conducts periphyton community sampling during spring/summer in wadeable rivers and streams.   Zooplankton community sampling occurs during the growing season in lakes and reservoirs.   Additional information collected along with the  biological monitoring include habitat information, pebble counts, general reach and /watershed characteristics, visual algae/aquatic vegetation surveys, and flow measurements. These data collected are primarily used in determining a biological response to nutrients and developing numeric nutrient criteria.

Approximately 100 stations are sampled once per year for fish, approximately 30 stations are sampled once per year for macroinvertebrate, and approximately 30 stations are sampled once per year for periphyton (diatoms).  Historically GAEPD sampled approximately 50 stations for zooplankton. However, GAEPD ceased zooplankton sampling in 2017.  In addition, targeted monitoring sites are also evaluated to assess waters undergoing restoration project improvements, and to correlate water chemistry with biological responses at trend monitored locations.

## Fish Tissue Monitoring.  Baseline fish tissue monitoring supports the protection of public health.  It allows for assessing the spatial impact from potential contaminant sources and supports water quality management programs.

GAEPD began their fish tissue monitoring in 1992, with collection of fish fromlarge lakes. The first risk-based consumption guidance for fish was published in 1995. Each year GAEPD increases its fish tissue sampling program to include small lakes, rivers, and estuaries. Sampling sites and fish species and size are selected based on fishing pressure and/or where more information is required for a particular species. The sampling is conducted by either WRD or CRD, depending on whether the site is freshwater (WRD), or estuarine/marine waters (CRD). In 2024, GAEPD, WRD, and CRD worked to revise the fish consumption sampling plan to ensure that fish from a given site were resampled at least every 10 years.   Site-specific sampling in Georgia lakes and rivers occurs every spring and fall and site-specific sampling in estuaries occurs between the spring and fall. Samples are catalogued and transported to GAEPD or UGA laboratories.  The list of the general contaminants analyzed for in the fish tissue is provided in Appendix A. Results are reported to GAEPD the following late summer or early fall. The data are assessed in the fall and winter and the consumption guidance is updated each spring. The data assessments are incorporated annually into the Guidelines for Eating Fish for Georgia Waters and Georgia’s Freshwater and Saltwater Sport Fishing Regulations, which is available on the GAEPD website http://epd.georgia.gov/fish-consumption-guidelines.

As part of the Georgia Clean Air Mercury Rule (CAMR) development, it was recognized that a more rigorous monitoring program of mercury in fish tissue would be required to support trend analysis and the efficacy of future reductions in air mercury emissions.  The Mercury in Fish Trend project was designed and implemented in 2006.  The project consists of 22 fish mercury trend stations that are monitored annually.  Fish from each location consist of a single species of similar age. Nineteen stations are freshwater and three are estuarine. The 22 fish mercury trend stations are listed in Appendix A.  Sample collection for the Mercury in Fish Trend project was completed in the fall 2020.

## Aquatic Toxicity Monitoring.  Aquatic toxicity monitoring supports protection of aquatic life, determines the impact of specific discharges, documents improvements resulting from upgraded water pollution control plants, supports enforcement actions, and verifies water pollution control plant compliance.

In the 1980s and 1990s, Georgia incorporated biomonitoring or aquatic toxicity testing in NPDES permits and initiated a comprehensive aquatic toxicity testing program.  Over the course of the decade from 1985 to 1995, GAEPD conducted (acute or chronic) aquatic toxicity tests on effluents from major municipal and industrial wastewater treatment facilities and minor facilities with a reasonable potential for having toxic substances.   This work identified potential problem areas across the State and resulted in NPDES permit modifications to include monitoring requirements and facility upgrades to remove toxic substances. In January 1995, GAEPD issued approved NPDES Reasonable Potential Procedures that further delineated required conditions for conducting whole effluent toxicity (WET) biomonitoring for municipal and industrial discharges.  As a result of funding and redirection issues, GAEPD laboratory testing was phased out in 1997. Currently, aquatic toxicity monitoring requirements are addressed in all municipal and industrial NPDES permits and WET testing is incorporated into permits where needed.

## Toxic Substance Monitoring.  The original objective of the toxic substance monitoring program was to identify potential problem areas across the State.  This resulted in NPDES permit modifications, including monitoring requirements and facility upgrades, to remove toxic substances and ensure compliance with water quality standards.  The current objective of the toxic monitoring program is to collect data to support 305(b)/303(d) listing assessments, TMDL development, and evaluation of point and nonpoint sources.

GAEPD targeted agricultural sites for legacy pesticide sampling. Pesticide samples were collected twice a year.  However, GAEPD discontinued this sampling because no legacy pesticides were found. Each year a select number of stream sites are sampled quarterly for heavy metals, these include all probabilistic and trend sites.  In addition, some targeted sampling is conducted.

Additional information is gathered through the NPDES permitting program where requirements are in place for periodic collection and analysis of effluent samples for toxic substances, including the State’s list of priority pollutants contained in the Rules and Regulations for Water Quality Control, Chapter 391-3-6.

PFAS Monitoring. Perfluoroalkyl and polyfluoroalkyl substances (PFAS) are a group of man-made chemicals that have strong carbon-fluorine bonds, which cause them to be highly persistent in the environment and in animals, including fish and human beings. These chemicals do not break down and they can accumulate over time.

There is evidence that exposure to PFAS can lead to adverse human health effects. . PFOA and PFOS are part of a larger group of PFAS chemicals. Peer-reviewed studies of laboratory animals and epidemiological studies of human populations indicate that exposure to PFOA and PFOS over certain levels may result in adverse health effects. EPA has a MCL for PFAS of

In the winter of 2021, EPD initiated a targeted PFAS monitoring project to assess the level of PFAS in drinking water across Georgia. EPD started monitoring finished drinking water in the Coosa and neighboring Tennessee basins due to the documented presence of PFAS and PFAS sources in the Coosa basin. EPD sent sample kits to all surface water public drinking water systems and all groundwater public drinking water systems serving populations of 500 or more.

This first round of monitoring identified no detectable GenX in finished drinking water. PFBS was found in 13 public water systems, all significantly below the health advisory. PFOA and PFOS were found in detectable levels in 10 public water systems.

Following the completion of the first phase of monitoring, EPD conducted a second round of monitoring of finished drinking water, which focused on large public water systems that serve populations of 100,000 or more, as well as those systems located in close proximity to significant Department of Defense installations.

In 2023, EPA initiated monitoring under UCMR 5. This monitoring includes 29 species of PFAS, and all public water systems serving populations of 3,300 or more are required to participate. To complement this effort, EPD will initiate a third and final round of monitoring in finished drinking water. This third round of monitoring will focus on public water systems not included in UCMR 5. Specifically, EPD will conduct monitoring of finished drinking water from very small public water systems that rely on groundwater in areas where groundwater is highly susceptible to pollution . The results of this monitoring as well as historic surface water and dirinking water PFAS monitoring data can be found on the EPD’s PFAS StoryMap, which can be found here:  https://gaepd.maps.arcgis.com/apps/MapSeries/index.html?appid=e8f2c6a51c1c41088002350f1eabe598

## Intensive Surveys Monitoring. The intensive survey planning monitoring is used to calibrate water quality models to develop TMDLs, establish wasteload allocations for new and existing facilities, study the impacts of specific discharges, and support enforcement actions.

Intensive surveys complement fixed station monitoring, as these studies focus intensive areal monitoring on a particular issue or problem over a shorter period of time. These surveys can be used to monitor and assess all waters of the State including rivers, streams, lakes, reservoirs, estuaries coastal areas, wetlands, and groundwater. Several types of intensive surveys are conducted, including model calibration surveys and impact studies.  Models are used for wasteload allocations and/or TMDL development and as tools for making regulatory decisions.  Impact studies are conducted where information on the cause and effect relationships between pollutant sources and receiving waters is needed.

Intensive surveys may include time of travel dye studies, flow measurements, bathymetry, long-term BOD studies, sediment oxygen demand measurements, photosynthesis respiration studies, water quality field measurements, continuous monitoring, and chemical analysis of water samples.  In many cases, biological information is collected along with chemical data for use in assessing environmental impacts. Intensive survey locations are selected based on the needs and priorities of GAEPD.

## Groundwater Well Monitoring.  In January 2011, GAEPD’s Regulatory Support Program reinstated a state-wide ambient groundwater monitoring network similar in design to that which existed within the Georgia Geologic Survey prior to 1998.  The network consists of wells and springs located throughout the State such that broad characterizations may be drawn regarding the general water quality of all major aquifers found within Georgia.  Water samples are analyzed for dissolved oxygen, pH, specific conductance, presence of radiation, VOCs, chloride, fluoride, sulfate, total phosphorus, nitrate/nitrite, and metals.  Pesticide analyses are conducted on certain samples (mainly from the Coastal Plain), when and if possible.  Monitoring personnel collect quarterly samples at 22 stations and single annual samples at approximately 143 well locations. The list of the ground water wells monitored is provided in Appendix A.

## Facility Compliance Monitoring.   GAEPD performs Compliance Sampling Inspections (CSIs) and Technical Evaluations of municipal, industrial, and private wastewater treatment facilities with NPDES permits. CSIs are also performed at wastewater Land Application Systems and all State-permitted industrial wastewater pretreatment facilities (“industrial users”). During CSIs, 24-hour effluent composite and/or grab samples are collected and split with the facility’s laboratory as part of the self-monitoring program validation process. Permittee sampling and flow monitoring procedures are also evaluated for compliance with the NPDES permit.

GAEPD compliance specialists perform between 70 and 100 CSIs annually, depending on staff levels. Inspections are targeted based on input from compliance personnel and the District Offices. Compliance staff and District Office associates also perform Integrated Compliance Information System (ICIS) and Performance Partnership Agreement (PPA) reportable inspections including CSIs, Operation & Maintenance Inspections, Laboratory Audits, and Facility Reconnaissance. Findings of all types of inspections are used to assess facility treatment efficiency, NPDES permit compliance, and self-monitoring effectiveness, and are available for use in enforcement actions, if necessary.

# 4.  CORE AND SUPPLEMENTAL WATER QUALITY INDICATORS

As described in the individual monitoring program designs above, a variety of indicators are used to assess compliance with water quality standards and support of designated uses.  A common set of water quality criteria including pathogen indicators ( enterococci, E. coli), dissolved oxygen, pH, temperature, and toxic substances apply to all water uses in Georgia including recreation, drinking water, fishing and aquatic life, wild river, scenic river, and coastal fishing.  In assessing lake water quality, additional indicators such as nutrients, secchi depth, and chlorophyll a are included.

Core and supplemental indicators are shown in Table 2. Core indicators are those sampled in all water bodies of a given type every time that water body type is monitored.  Supplemental indicators are parameters that are sampled as part of the targeted monitoring program. Those parameters that are marked with a “*” either have water quality criteria or are used for assessment purposes.

TABLE 2.  WATER QUALITY INDICATORS

| INDICATOR TYPE | FISHING - AQUATIC LIFE | DRINKING WATER | RECREATION |
| --- | --- | --- | --- |
| Core | Dissolved oxygen* pH* Water Temperature* Specific Conductance BOD5 Alkalinity Hardness Suspended solids Ammonia (Narrative Toxicity)* Nitrate-nitrite Kjeldahl nitrogen Total nitrogen* Total phosphorus* Total Organic Carbon Lake trophic status | Turbidity VOCs, Chloride Sulfate Calcium Nitrate-nitrite Phosphorus Potassium Magnesium Manganese Sodium Chromium Nickel Copper Zinc Arsenic* Selenium Molybdenum Silver Cadmium Tin Antimony Barium Thallium Lead Uranium Aluminum Beryllium Cobalt Iron Titanium Vanadium Fluorine | pH* Chlorophyll a* Secchi Disk Transparency Photic Zone Depth Algal blooms* Aesthetics* Objectionable scums* Objectionable sheens* Objectionable debris* Objectionable deposits* Color* Turbidity Water level |
| Supplemental | Toxic pollutants* (e.g., priority pollutants, pesticides) Metals (aquatic life)* Fish Tissue analysis* Macroinvertebrate community* Fish community * Periphyton/Phytoplankton Habitat Flow Sediment chemistry Organism condition factor Non-native species Land-use/% impervious cover Pollutant loadings Fish kills | Human Health Criteria* E. coli* Enterococci* | E. coli Enterococci Sediment quality Macrophyte density |

The supplemental indicators may also be  used when there is a reasonable expectation that a specific pollutant may be present in a watershed, when core indicators indicate impairment, or to support a special study such as screening for pollutants of concern.  The process for identifying supplemental indicators to monitor is based on which type of designated use has not been met and then selecting the appropriate indicators to measure.  Basic water chemistry including turbidity may be used as an initial screening tool prior to running more expensive analytical procedures.

# 5.  QUALITY ASSURANCE

## Laboratory

All samples collected by GAEPD and its cooperators, as part of the Surface Water Quality Monitoring Program, are sent to laboratories operating under formalized Quality Assurance Program Plans (QAPP) that are reviewed by GAEPD prior to sample submission. All laboratory tests are conducted in accordance with USEPA approved methods. These laboratories follow standard laboratory Quality Control procedures and participate in both internal and external blind proficiency sample programs.  Accepted results reported are within the 95% confidence interval.  Each laboratory is required to have a comprehensive QAPP document on file with GAEPD. Sample integrity, from time of collection to time of laboratory receipt, is maintained through use of Chain of Custody documentation. Sample integrity is maintained within the laboratories through extensive sample tracking and documentation procedures.  All laboratory analyses are performed and reported in compliance with the comprehensive quality assurance plans of each laboratory.

Final sample results from each laboratory are maintained in validated database systems. These results are reported to GAEPD via electronic data transfer files. This data is ultimately combined and stored in a GAEPD database GOAMS and/or the USEPA WQX.  A review and feedback system between GAEPD and the laboratories is maintained to ensure that data quality is maintained.

## Quality Assurance

Georgia monitoring work is conducted in accordance with approved methods and documented in the Watershed Protection Branch QAPP and Standard Operating Procedures (SOPs).  The QAPP provides the details of the quality assurance procedures employed by GAEPD.  The standard quality assurance procedures used by GAEPD were developed to ensure and document the validity of measurements and analysis, and the representativeness of samples collected.  Enforcement activities by GAEPD require full documentation of the particulars of data collection, including the equipment used.  All GAEPD field personnel who collect samples or field data are trained to implement the procedures and have a full understanding of all sampling SOPs.

USEPA requirements pertaining to specifics of sample collection for States receiving grant funds are specified in federal regulations under the authority of the CWA and the NPDES permitting program.  The most widely applicable guidance at this level is Title 40 of the Code of Federal Regulations (40 CFR).  The procedures and techniques given in 40 CFR are periodically updated. In accordance with these regulations, state-wide water quality monitoring data collections are covered by an USEPA approved Quality Management Plan (QMP) and a QAPP.  These plans along with SOPs are maintained in GAEPD files. Updates to the SOPs, QMP, and QAPP are submitted to the USEPA by GAEPD when any changes in the documents occur (for example, monitoring site list revised, use of new sampling equipment, changes in sampling parameters or analytical methods used, etc.).

# 6.  DATA MANAGEMENT

Georgia uses GOMAS, an electronic accessible database, for storage of water quality, fish tissue, habitat, biological, and facility monitoring data collected by GAEPD and its cooperators.  Data are entered into GOMAS  in a timely manner and the data are available to the public through a public portal.

Additionally, these water quality data are uploaded to the USEPA’s WQX database.  The USEPA WQX database provides an electronic internet portal to GAEPD data and provides Georgia the opportunity to assess waters beyond state boundaries, as appropriate.  All data are collected and stored using appropriate metadata and State/Federal geo-locational standards.

GAEPD’s 305b/303d assessment information is also contained within GOMAS.  The assessment data is transmitted to EPA’s ATTAINS database every two years with the submitral of the Integrated Report.  Supporting documentation such as our Listing Assessment Methodology, comments received and responses, and the GIS coverage of our assessed waters, is uploaded to ATTAINS via the web portal.

# 7.  DATA ANALYSIS/ASSESSMENT

## Designated Uses and Water Quality Standards

Georgia has a methodology for assessing attainment of water quality standards based on analyses of various types of data (chemical, physical, biological, land use) from various sources for all water body types in the State.  Assessment of water quality requires a baseline for comparison. A state-wide baseline is provided by Georgia’s water quality standards, which contain designated uses, numeric and narrative criteria, and an antidegradation policy.  The GAEPD is responsible for setting and enforcing water quality standards.

The purposes and intent of the State in establishing water quality standards are to provide enhancement of water quality and prevention of pollutions; protect the public health and welfare in accordance with the public interest for drinking water supplies, conservation of fish, wildlife and other beneficial aquatic life, recreational, and other reasonable and necessary uses; and maintain and improve the biological integrity of the waters of the State.  Georgia’s waters are currently classified as one of the following designated uses: drinking water, recreation, fishing, coastal fishing, wild river, or scenic river.

Specific water quality criteria are assigned to support each designated use.  The quality of Georgia’s waters is judged by the extent to which the waters support the uses for which they have been designated. Appendix B provides a summary of designated uses and specific water quality criteria for each water use.  Georgia also has general narrative water quality standards that apply to all waters. These narrative standards are also summarized in Appendix B.

In 1989, the Board of Natural Resources adopted 31 numeric standards for protection of aquatic life and 90 numeric standards for the protection of human health.  In addition, during the early to mid-1990’s, the DNR Board began adopting standards for six major lakes and tributaries. All general criteria for waters are outlined in Georgia’s Rules and Regulations for Water Quality Control (Chapter 391-3-6-.03(5), lake criteria are given in Georgia’s Rules and Regulations for Water Quality Control (Chapter 391-3-6-.03(17).

Different sections of the CWA require States to assess water quality [Section 305(b)], to list waters with water quality standards violations for which no actions have been initiated and therefore a TMDL is needed [Section 303(d)], and to document waters with nonpoint source problems (Section 319). All existing and readily available data is compiled and analyzed.  In addition to data collected by GAEPD and its cooperators, data from third parties (such as municipalities and environmental groups) are used as long as the quality control requirements found in Chapter 391-3-6-.03(13) are met.  Data and information that does not meet quality control requirements are used as screening information and may be used during the selection process regarding sites GAEPD or our cooperator will monitor.

GAEPD assesses water quality data to determine if water quality standards are met and if the water body supports its designated use.  Waterbodies are placed in one of five tiers that were developed by EPA indicating whether it is supporting its designated use or not, if more information is needed to make a determination, and if a TMDL is required.  This tiered approach provides a mechanism to track all waters of the State and the range of assessments made to determine if the waterbody meets its designated uses(s).  Georgia’s 305(b)/303(d) Listing Methodology is a dynamic document that is updated with each listing cycle to reflect current guidance by the USEPA and to incorporate new information made available during the listing cycle. The assessment information is maintained in an electronic web-accessible database.

Other information is integrated with available data and a report prepared for the USEPA and the public every two years.  These integrated reports are Georgia’s 305(b)/303(d) list of waters and “Water Quality in Georgia Report.”  The list and report are updated and publicly noticed for comment prior to submittal to the USEPA for final approval.  This is done to engage and secure public input on the listing, TMDL prioritization, and reporting process.  Georgia assesses streams, coastal streams, lakes, freshwater beaches, marine beaches, and sounds/harbors as part of the 305(b)/303(d) process.  The 305(b)/303(d) list of waters includes information about the waters designated use(s), whether use(s) are being met, and location information such as county and river basin.  For waters assessed as impaired, the criterion violated and potential sources of pollution are also included along with information about when a TMDL will be drafted.  If a TMDL has already been written, this information is also included.  Georgia’s current and historic 305(b)/303(d) lists and reports are available for public review on the GAEPD web page at:  https://epd.georgia.gov/watershed-protection-branch/watershed-planning-and-monitoring-program/water-quality-georgia.   Geographic Information System (GIS) coverages are also maintained that depict the waters on the lists and can be found on GAEPD’s webpage at: https://epd.georgia.gov/geographic-information-systems-gis-databases-and-documentation.

# 8.  REPORTING

Georgia produces reports and lists in accordance with CWA requirements in a timely and complete manner.  The CWA [Section 305(b)] requires states to assess and characterize the condition and trends of waters within the State.  The CWA [Section 303(d)] requires States to identify impaired waters for which TMDLs are needed.  The Section 305(b) Report and the Section 303(d) list are due in even numbered years.  Georgia integrates the two reporting requirements of Sections 305(b) and 303(d) of the CWA.  Final reports are submitted to the USEPA by April 1st of every even numbered year for the State to remain eligible for Section 106 grant funding assistance for the water quality monitoring program.  The integrated report is submitted to EPA’s ATTAINS database.   Annual updates of water quality data and information are provided to the USEPA during odd numbered years to provide a status of water quality monitoring efforts between 305(b)/303(d) listing cycles.

Georgia also prepares a GIS coverage to illustrate the location of the waters on the integrated list.  The GIS coverage, lists, and reports are placed on the GAEPD website for easy access for the public.  The GIS coverage is also uploaded to ATTAINS as part of the submittal of our Integrated report.

In addition, information required under Sections 314 and 319 are covered in the Georgia 305(b) Report (also known as the “Water Quality in Georgia” report).  The CRD provides information  on monitoring and notification programs for coastal recreation waters in accordance with CWA Section 406 (BEACHES Act).   Georgia also provides a CWA Section 106 monitoring update (in odd numbered years) by uploading of monitoring data to the national STORET data warehouse.  A Story Map of the 2020 “Water Quality in Georgia” report can be found  on the GAEPD website at: https://gaepd.maps.arcgis.com/apps/MapSeries/index.html?appid=dea4c9c319d4461c8d5cef8e68957b1b.

# 9.  PROGRAMMATIC EVALUATION

The Georgia Monitoring and Assessment Strategy as described in this document represents a comprehensive approach to address the goals and objectives of the water quality monitoring program.  The monitoring program is long-term in nature.

Monitoring program evaluations occur throughout the year with enhancements implemented as needed to address specific acute issues. The Program Managers in the Watershed Protection Branch meet throughout the year and monitoring issues and needs are regularly discussed. Often needs arise, such as monitoring to support enforcement actions, impact studies, TMDL modeling, and/or monitoring in response to citizen input, that require changes to the monitoring programs. Minor program changes can be implemented quickly and efficiently in response to localized needs, at any time during the year. Larger programmatic changes are considered annually, along with available budgets, and implemented, as appropriate.  The Watershed Planning and Monitoring Program (WPMP) monitoring staff works with the other Programs within the Watershed Protection Branch to determine the sites that should be monitored.  Integration of monitoring activities between the Assessment Coordinator, TMDL Modeling and Development Unit, Wastewater Regulatory Program, Nonpoint Source Program, and the Ambient Monitoring Units allows GAEPD to effectively and systematically prioritize waters for assessment, restoration, and protection.

Any major annual changes provide milestones or progress markers that are negotiated and documented in the State/EPA PPA. The annual planning process in preparing the PPA provides an opportunity for annual review of implementation priorities, in line with available resources to address the priorities. This also provides for a periodic review of each aspect of the monitoring program to determine how well the program serves its water quality data and decision needs. In addition, this Monitoring and Assessment Strategy will be reviewed and updated every three to five years.

# 10.  GENERAL SUPPORT AND INFRASTRUCTURE PLANNING

The Georgia monitoring program depends primarily on funds from the State budget with some funding from Federal sources.  Georgia works closely with the USEPA and the USGS on a number of monitoring projects to maximize monitoring efficiencies.  The USEPA provides some grant funding for monitoring projects in Georgia and the USGS provides some limited cooperative project matching funds for monitoring projects in Georgia.  As a part of the ongoing planning process, monitoring needs are discussed with the USEPA during the negotiation process for the State/EPA PPA that includes CWA Section 106 funds.  The USEPA also provides direct support for monitoring projects in Georgia through its Science and Ecosystem Support Division in Athens, Georgia.  Each year, Georgia and other states in the Region provide the USEPA with a list of technical assistance needs for the following year.  The USEPA reviews and prioritizes the state requests and supports the states, as resources allow.

Training is an important element of ongoing monitoring programs in Georgia.  GAEPD takes advantage of USEPA sponsored training in all aspects of monitoring including: field techniques, laboratory analysis, and data management and analysis.  In addition, GAEPD conducts an annual internal training for all monitoring personnel, to ensure that sampling standards and practices are accurate and consistent with established protocols.

Georgia will continue to review and assess monitoring programs and seek additional resources to enhance them, as needed.

## Resources

The Watershed Protection Branch has identified the Watershed Planning and Monitoring Program (WPMP) as the lead program for implementing and maintaining the State’s Surface Water and Ground Water Quality Monitoring and Assessment Strategy.  This involves coordination with outside agencies and monitoring groups to assist in the collection of data needed to fulfill the management goals of the program.  Staffing resources, within the WPMP to fulfill the responsibilities of data gathering, assessment, report preparations, and TMDL development, include eighteen (18) field staff positions within the WPMP for collection of physical, chemical, and biological data from rivers, streams, lakes, estuaries, and groundwater; three (3) field staff positions within the WPMP for conducting compliance evaluation inspections and sampling of permitted facility effluents; one (1) water quality standards coordinator; one (1) data management and QA/QC position; one (1) 305(b)/303(d) data assessment, report preparation and Sampling Quality Assurance Plan review position; and four (4) TMDL modeling and development positions.  Additional resources are provided through contracted monitoring assistance from the USGS, Phinizy Center for Water Sciences, and Columbus Water Works.

The monitoring programs in Georgia are supported by a full service GAEPD laboratory located in Norcross, Georgia.  In addition, some laboratory work is contracted with EPA, the University of Georgia (UGA), and/or USGS.  In some cases, in conjunction with technical assistance requests, the USEPA provides laboratory support at its facilities in Athens, Georgia.  Biological work on macroinvertebrate identification is conducted at the Watershed Protection Branch laboratory facilities in Atlanta, Georgia, and fish identification work is conducted by the WRD in Social Circle, Georgia.  Contractors also assist with these identification efforts.

In addition to staffing and analytical services, the contractual services provided by the USGS and Columbus Water Works for water quality sample collection and by UGA for water quality analyses amounts to over $1,500,000 per year.  An assessment of current funding and staffing resources as opposed to the level of effort to achieve the goals of the State’s monitoring strategy indicates the funding and staffing resources are minimal to what is needed to meet the goals and objectives of the strategy.

For approximately ten years, the Wetlands Unit has been engaged in various wetlands monitoring activities across the state, initially as a state-level participant in EPA’s National Wetlands Condition Assessment (NWCA), and then conducting independent ecoregion focused wetland monitoring that employed the established NWCA field methodology.  Recently, the wetland monitoring has focused on documenting wetland hydrology/hydric soils characteristics at selected wetland reference sites, as well as assessing the performance and condition of long-established GDOT wetland mitigation sites.  GAEPD will need to

Additional monitoring programs or enhancement/expansion of already implemented programs requires additional resources in manpower and laboratory analytical services.  Some of the new or enhanced monitoring projects to fully implement the State’s Monitoring and Assessment Strategy include:

- In 2022, open office in Augusta to house two monitoring staff who will be responsible for monitoring the eastern portion of Georgia.

- Development and implementation of a probabilistic monitoring program for streams to increase the number of assessed waters over a 10-year monitoring period.

- Recruit and retain staff with expertise in wetland science and come up with a viable balance between wetland monitoring efforts and ongoing 401 Water Quality Certification obligations.

- Continue monitoring of groundwater well stations installed as part of wetland grant work.

## Future Studies and Actions

The State’s Monitoring and Assessment Strategy is a dynamic document and should be flexible to incorporate shifting priorities in monitoring goals and objectives. Some suggested future studies and actions are listed below as part of the Strategy review process.  These suggestions will enhance or improve on the data quality, quantity and assessment strategies already in place requiring a relatively neutral budget change.

- Increase coordination and collaboration with other programs within GAEPD and GADNR.

- Expand working with citizen volunteer monitoring groups to provide technical assistance and training to ensure improved quality of data to build on the State’s assessed water database.

There are benefits to be gained by using external data and promoting stewardship warranting the investment of resources on the part of GAEPD.  Integrating other data is a process that requires enhanced program coordination (i.e. communication about sampling plans and goals, planning useful/equivalent measures, modeling/assessment procedures, QA and data sharing).

While State agencies are collecting water quality information to address specific needs, water quality monitoring efforts are being conducted by other agencies, universities, local governments, organizations and/or citizen volunteer groups.  Although some of these efforts are implemented to address goals similar to the State, these efforts may also have a non-management focus, for example, addressing research, education and supporting other programs.  Data and information provided by these outside programs can serve to fill gaps within the State’s monitoring program.

Future success in making sound professional judgments about the quality of Georgia’s waters depends on the proper direction and training to staff, providing the tools and skills to accomplish the tasks and the technical resources to support the assessment process.

APPENDIX A

SURFACE WATER MONITORING STATIONS AND GROUNDWATER WELLS

.

1a. 2025 SURFACE WATER MONITORING STATIONS

Rivers/Streams, Lakes/Reservoirs, Estuaries/Sounds

Rivers and streams stations are sampled monthly for field and chemical parameters for one calendar.  Bacterial samples are collected monthly

Lakes, reservoirs, and estuaries are sampled once a month during the growing season (April-October).

| Georgia Station Number | Sampling Site | River Basin | Sampling Organization1 | Waterbody Type/Project | Latitude | Longitude | Routine2 | Enterococcus | E. coli | Orthophosphate s | Metals | Macroinvertebrates3 | Periphyton3 | Discharge | Chlorophyll |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LK_01_10 | Lake Rabun - Dampool (aka Tallulah River - Upstream From Mathis Dam) | Savannah | Cartersville AMU | Lake Trend Monitoring | 34.764722 | -83.417778 | X |  | X |  |  |  |  |  | X |
| LK_01_11 | Lake Hartwell at Interstate 85 | Savannah | Augusta AMU | Lake Trend Monitoring | 34.484167 | -83.029833 | X |  | X |  |  |  |  |  | X |
| LK_01_22 | Lake Hartwell - Dam Forebay | Savannah | Augusta AMU | Lake Trend Monitoring | 34.358733 | -82.824417 | X |  | X |  |  |  |  |  | X |
| LK_01_27 | Lake Russell Between Markers 42 and 44 (Mid Lake) | Savannah | Augusta AMU | Lake Trend Monitoring | 34.127778 | -82.673611 | X |  | X |  |  |  |  |  | X |
| LK_01_29 | Lake Richard B. Russell - Dam Forebay | Savannah | Augusta AMU | Lake Trend Monitoring | 34.026333 | -82.594167 | X |  | X |  |  |  |  |  | X |
| LK_01_38 | Clarks Hill Lake- Savannah River At U.S. Highway 378 | Savannah | Augusta AMU | Lake Trend Monitoring | 33.857861 | -82.399583 | X |  | X |  |  |  |  |  | X |
| LK_01_39 | Clarks Hill Lake- Savannah River At Dordon Crk. | Savannah | Augusta AMU | Lake Trend Monitoring | 33.765861 | -82.271778 | X |  | X |  |  |  |  |  | X |
| LK_01_40 | Clarks Hill Lake - Dam Forebay | Savannah | Augusta AMU | Lake Trend Monitoring | 33.662694 | -82.198528 | X |  | X |  |  |  |  |  | X |
| LK_01_67 | Lake Tugalo - u/s Tugalo Lake Rd (aka Bull Sluice Rd.) | Savannah | Atlanta AMU | Lake Trend Monitoring | 34.737805 | -83.340555 | X |  | X |  |  |  |  |  | X |
| LK_01_68 | Lake Tugalo - Upstream From Tugaloo Dam | Savannah | Atlanta AMU | Lake Trend Monitoring | 34.715 | -83.351694 | X |  | X |  |  |  |  |  | X |
| LK_01_7 | Lake Burton - 1/4 mile South of Burton Island (aka Tallulah River) | Savannah | Cartersville AMU | Lake Trend Monitoring | 34.835233 | -83.553817 | X |  | X |  |  |  |  |  | X |
| LK_01_71 | Clarks Hill Lake - Little River At Highway 47 | Savannah | Augusta AMU | Lake Trend Monitoring | 33.692722 | -82.338805 | X |  | X |  |  |  |  |  | X |
| LK_01_8 | Lake Burton - Dampool (aka Tallulah River u/s Lake Burton Dam) | Savannah | Cartersville AMU | Lake Trend Monitoring | 34.795317 | -83.5401 | X |  | X |  |  |  |  |  | X |
| LK_01_9 | Lake Rabun - Approx. 4.5 mi u/s Dam (Mid Lake) | Savannah | Cartersville AMU | Lake Trend Monitoring | 34.763533 | -83.455817 | X |  | X |  |  |  |  |  | X |
| LK_03_17700 | Lake Oconee - Lick Creek Cove near Old Phoenix Rd. | Oconee | Augusta AMU | Lake Trend Monitoring | 33.403819 | -83.272422 | X |  | X |  |  |  |  |  |  |
| LK_03_520 | Lake Oconee At Highway 44, Oconee River Arm | Oconee | Augusta AMU | Lake Trend Monitoring | 33.431394 | -83.265734 | X |  | X |  |  |  |  |  | X |
| LK_03_525 | Lake Sinclair - Little River & Murder Creek Arm, U/S U.S. Hwy 441 | Oconee | Augusta AMU | Lake Trend Monitoring | 33.189 | -83.2953 | X |  | X |  |  |  |  |  | X |
| LK_03_526 | Lake Sinclair - 300 Meters U/S Dam (Dam Forebay) | Oconee | Augusta AMU | Lake Trend Monitoring | 33.142817 | -83.202617 | X |  | X |  |  |  |  |  | X |
| LK_03_530 | Lake Sinclair - Midlake, Oconee River Arm | Oconee | Augusta AMU | Lake Trend Monitoring | 33.1968 | -83.2742 | X |  | X |  |  |  |  |  | X |
| LK_03_538 | Lake Oconee 300 Meters U/S Wallace Dam (Dam Forebay) | Oconee | Augusta AMU | Lake Trend Monitoring | 33.351667 | -83.160833 | X |  | X |  |  |  |  |  | X |
| LK_03_545 | Lake Oconee - Richland Creek Arm | Oconee | Augusta AMU | Lake Trend Monitoring | 33.3947 | -83.1767 | X |  | X |  |  |  |  |  | X |
| LK_03_600 | Lake Sinclair- Little River/Oconee River Confluence | Oconee | Augusta AMU | Lake Trend Monitoring | 33.1875 | -83.275 | X |  | X |  |  |  |  |  | X |
| LK_04_893 | Lake Jackson at confluence of Alcovy River and Yellow/South River Branch | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.36314 | -83.86131 | X |  | X |  |  |  |  |  | X |
| LK_04_897 | Lake Jackson - Dam Forebay | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.322 | -83.8409 | X |  | X |  |  |  |  |  | X |
| LK_05_2076 | High Falls Lake - Midlake | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.1973 | -84.031 | X |  | X |  |  |  |  |  | X |
| LK_05_2078 | High Falls Lake - Dam Forebay | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.1799 | -84.0209 | X |  | X |  |  |  |  |  | X |
| LK_05_2131 | Lake Juliette - Midlake | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.0464 | -83.8106 | X |  | X |  |  |  |  |  | X |
| LK_05_2132 | Lake Juliette - Dam Forebay | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.0338 | -83.7572 | X |  | X |  |  |  |  |  | X |
| LK_05_2144 | Lake Tobesofkee - Midlake | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 32.8346 | -83.8161 | X |  | X |  |  |  |  |  | X |
| LK_05_2146 | Lake Tobesofkee - Dam Forebay | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 32.8215 | -83.7706 | X |  | X |  |  |  |  |  | X |
| LK_09_3199 | Banks Lake - Near Lakeland, GA | Suwannee | Tifton AMU | Lake Trend Monitoring | 31.026667 | -83.105555 | X |  | X |  |  |  |  |  | X |
| LK_11_3467 | Lake Blackshear - Midlake | Flint | Tifton AMU | Lake Trend Monitoring | 31.9665 | -83.9342 | X |  | X |  |  |  |  |  | X |
| LK_11_3520 | Lake Blackshear - Dam Forebay | Flint | Tifton AMU | Lake Trend Monitoring | 31.8479 | -83.9394 | X |  | X |  |  |  |  |  | X |
| LK_11_3534 | Flint River Reservoir - Midlake, Flint River Arm | Flint | Tifton AMU | Lake Trend Monitoring | 31.6085 | -84.119 | X |  | X |  |  |  |  |  | X |
| LK_11_3535 | Flint River Reservoir (Lake Worth) - Dam Forebay | Flint | Tifton AMU | Lake Trend Monitoring | 31.6033 | -84.1365 | X |  | X |  |  |  |  |  | X |
| LK_11_3551 | Lake Worth (original) - Above Hwy 91 Bridge / Diversion Dam (aka Lake Chehaw) | Flint | Tifton AMU | Lake Trend Monitoring | 31.6109 | -84.15 | X |  | X |  |  |  |  |  | X |
| LK_11_3569 | Lake Seminole - Flint River Arm at Spring Creek | Flint | Tifton AMU | Lake Trend Monitoring | 30.7627 | -84.8171 | X |  | X |  |  |  |  |  | X |
| LK_12_3913 | Lake Sidney Lanier - Little River Embayment, Betw M1WC & 3LR | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.355 | -83.8427 | X |  | X |  |  |  |  |  | X |
| LK_12_3995 | Lake Sidney Lanier at Boling Bridge (State Road 53) on Chestatee River | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.31235 | -83.950103 | X |  | X |  |  |  |  |  | X |
| LK_12_3998 | Lake Sidney Lanier at Lanier Bridge (State Road 53) on Chattahoochee River | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.32195 | -83.880171 | X |  | X |  |  |  |  |  | X |
| LK_12_4001 | Lake Sidney Lanier at Browns Bridge Road (State Road 369) | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.261666 | -83.950662 | X |  | X |  |  |  |  |  | X |
| LK_12_4005 | Lake Sidney Lanier - Flat Creek Embayment, 100' U/S M7FC | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.2587 | -83.9198 | X |  | X |  |  |  |  |  | X |
| LK_12_4007 | Lake Sidney Lanier - Balus Creek Embayment, 0.34m SE M6FC | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.2504 | -83.9244 | X |  | X |  |  |  |  |  | X |
| LK_12_4010 | Lake Sidney Lanier - Mud Crk Embayment, Betw Marina & Ramp | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.2333 | -83.9373 | X |  | X |  |  |  |  |  | X |
| LK_12_4012 | Lake Lanier upstream from Flowery Branch Confluence (Midlake) | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.200278 | -83.982869 | X |  | X |  |  |  |  |  | X |
| LK_12_4019 | Lake Sidney Lanier - Six Mile Creek Embayment, 300' E M9SM | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.2335 | -84.0287 | X |  | X |  |  |  |  |  | X |
| LK_12_4028 | Lake Sidney Lanier upstream of Buford Dam Forebay | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.162778 | -84.067108 | X |  | X |  |  |  |  |  | X |
| LK_12_4048 | West Point Lake at LaGrange Water Intake near LaGrange, Ga. | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 33.0783 | -85.110833 | X |  | X |  |  |  |  |  | X |
| LK_12_4060 | West Point Lake - Dam Forebay | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.9208 | -85.1834 | X |  | X |  |  |  |  |  | X |
| LK_12_4072 | Lake Harding - Midlake, Main Body | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.7379 | -85.1125 | X |  | X |  |  |  |  |  | X |
| LK_12_4074 | Lake Harding - Dam Forebay (aka Chatt. River US Bartletts Ferry Dam) | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.6633 | -85.090278 | X |  | X |  |  |  |  |  | X |
| LK_12_4078 | Goat Rock Lake - Dam Forebay | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.6112 | -85.0794 | X |  | X |  |  |  |  |  | X |
| LK_12_4080 | Lake Oliver - Dam Forebay | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.516 | -85.0009 | X |  | X |  |  |  |  |  | X |
| LK_12_4097 | Lake Walter F. George at U.S. Highway 82 (aka Chatt. River at Hwy 82) | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 31.891944 | -85.120833 | X |  | X |  |  |  |  |  | X |
| LK_12_4103 | Lake Walter F. George at Dam Forebay | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 31.629167 | -85.0725 | X |  | X |  |  |  |  |  | X |
| LK_12_4107 | Lake Andrews - Dam Forebay | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 31.2632 | -85.113 | X |  | X |  |  |  |  |  | X |
| LK_12_4113 | Lake Seminole - Chattahoochee Arm, Lower | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 30.7662 | -84.9201 | X |  | X | X | X |  |  |  | X |
| LK_12_4115 | Lake Seminole - Dam Forebay | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 30.7115 | -84.8647 | X |  | X |  |  |  |  |  | X |
| LK_14_4494 | Lake Allatoona Upstream from Dam | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.160833 | -84.725845 | X |  | X |  |  |  |  |  | X |
| LK_14_4497 | Lake Allatoona at Allatoona Creek Upstream from Interstate 75 | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.085833 | -84.711389 | X |  | X |  |  |  |  |  | X |
| LK_14_4502 | Lake Allatoona at Etowah River upsteam from Sweetwater Creek | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.19 | -84.577778 | X |  | X |  |  |  |  |  | X |
| LK_14_4523 | Carters Lake (CR1) - Upper Lake, Coosawattee Arm | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.62087 | -84.6212 | X |  | X |  |  |  |  |  | X |
| LK_14_4524 | Carters Lake - Midlake (upstream from Woodring Branch) | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.6076 | -84.638 | X |  | X |  |  |  |  |  | X |
| LK_14_4553 | Lake Allatoona at Little River upstream from Highway 205 | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.158611 | -84.577222 | X |  | X |  |  |  |  |  | X |
| LK_14_4556 | Lake Allatoona downstream from Kellogg Creek | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.138611 | -84.639167 | X |  | X |  |  |  |  |  | X |
| LK_15_4895 | Lake Chatuge LMP 12 at State Line (aka Hiawassee River) | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.983333 | -83.788611 | X |  | X |  |  |  |  |  | X |
| LK_15_4899 | Lake Nottely (LMP15A) at Reece Creek | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.91152 | -84.0506 | X |  | X |  |  |  |  |  | X |
| LK_15_4900 | Lake Nottely - Dam Forebay (aka Nottely River - Upstream From Nottley Dam) | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.957778 | -84.092222 | X |  | X |  |  |  |  |  | X |
| LK_15_4907 | Lake Blue Ridge (LMP18) - 300 Meter Upstream Of Dam | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.881667 | -84.28 | X |  | X |  |  |  |  |  | X |
| LK_15_4908 | Lake Blue Ridge (LMP18A) - 4 miles upsteam Dam | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.84017 | -84.2731 | X |  | X |  |  |  |  |  | X |
| SH_02_317 | Little Ogeechee River at Green Island | Ogeechee | Brunswick AMU | Lake/Estuary Site | 31.88823 | -81.08798 | X | X |  |  |  |  |  |  | X |
| SH_02_364 | St Catherines Sound at Medway River near Midway, GA | Ogeechee | Brunswick AMU | Lake/Estuary Site | 31.715469 | -81.156798 | X | X |  |  |  |  |  |  | X |
| SH_02_372 | Sapelo Sound at South Newport River near Barbour Island | Ogeechee | Brunswick AMU | Lake/Estuary Site | 31.554108 | -81.200361 | X | X |  |  |  |  |  |  | X |
| SH_02_374 | Sapelo River - Mouth of Broro River - 1.4 miles South of Shellman's Bluff | Ogeechee | Brunswick AMU | Lake/Estuary Site | 31.544861 | -81.316027 | X | X |  |  |  |  |  |  | X |
| SH_02_56 | Mouth of Wilmington River - Marker #19 Wassaw Sound | Ogeechee | Brunswick AMU | Lake/Estuary Site | 31.932416 | -80.977111 | X | X |  |  |  |  |  |  | X |
| SH_06_15212 | Doboy Sound | Altamaha | Brunswick AMU | Lake/Estuary Site | 31.394942 | -81.29435 | X | X |  |  |  |  |  |  | X |
| SH_06_2857 | Altamaha River - channel marker #201 off Wolf Island | Altamaha | Brunswick AMU | Lake/Estuary Site | 31.319166 | -81.325 | X | X |  |  |  |  |  |  | X |
| SH_07_15209 | St. Simons Sound | Satilla | Brunswick AMU | Lake/Estuary Site | 31.12568 | -81.411953 | X | X |  |  |  |  |  |  | X |
| SH_07_3008 | St. Andrews Sound at Satilla Riv near | Satilla | Brunswick AMU | Lake/Estuary Site | 30.983162 | -81.453238 | X | X |  |  |  |  |  |  | X |
| SH_07_3029 | Turtle River off Hermitage Island | Satilla | Brunswick AMU | Trend Monitoring | 31.220278 | -81.564167 | X |  |  |  |  |  |  |  |  |
| SH_07_3032 | Turtle River - Georgia Highway 303 | Satilla | Brunswick AMU | Trend Monitoring | 31.186944 | -81.531389 | X |  |  |  |  |  |  |  |  |
| SH_07_3035 | Brunswick Harbor (off East River) - 0.83 miles SW of Brunswick | Satilla | Brunswick AMU | Trend Monitoring | 31.143611 | -81.4975 | X |  |  |  |  |  |  |  |  |
| SH_07_3036 | Brunswick River - U.S. Highway 17 | Satilla | Brunswick AMU | Trend Monitoring | 31.1164 | -81.4858 | X |  |  |  | X |  |  | X |  |
| SH_07_3049 | Cumberland Sound at St. Marys Riv nr St Marys, GA | Satilla | Brunswick AMU | Lake/Estuary Site | 30.728073 | -81.489794 | X | X |  |  |  |  |  |  | X |
| RV_01_109 | Savannah River - Seaboard Coast Line Railway, North of Clyo | Savannah | Brunswick AMU | Trend Monitoring (USGS) | 32.525 | -81.263978 | X |  | X | X |  |  |  |  |  |
| RV_01_12 | Reed Creek at County Road 301 near Hartwell, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 34.45377 | -82.94018 | X |  | X |  |  |  |  |  |  |
| RV_01_120 | Savannah River - US Highway 17 (Houlihan Bridge) | Savannah | Brunswick AMU | Trend Monitoring (USGS) | 32.165833 | -81.1539 | X |  | X | X |  |  |  |  |  |
| RV_01_14 | Eastanollee Creek At Rose Lane In Toccoa, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli, pH) | 34.54393 | -83.3029 | X |  | X |  |  |  |  |  |  |
| RV_01_144 | Kettle Creek at Stone Bridge Rd | Savannah | Augusta AMU | Trend Monitoring | 33.68301 | -82.85747 | X |  | X | X |  | X | X | X |  |
| RV_01_16 | Eastanollee Creek - Yow Mill Road Near Avalon, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 34.52774 | -83.19733 | X |  | X |  |  |  |  |  |  |
| RV_01_17 | Eastanolle Creek at Tower Road nr Avalon, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 34.52633 | -83.1853 | X |  | X |  |  |  |  |  |  |
| RV_01_175 | Broad River at US Hwy 29 | Savannah | Augusta AMU | Targeted Monitoring (Additional data) | 34.24722 | -83.17328 | X |  |  | X | X |  |  |  |  |
| RV_01_18166 | Trib to Van Creek at Amberly Rd near Elberton, GA | Savannah | Augusta AMU | Probabilistic Monitoring | 34.18538 | -82.79846 | X |  | X | X | X |  |  |  |  |
| RV_01_18167 | Dicks Creek at Guard Camp Rd in Lake Russell WMA near Ayersville, GA | Savannah | Augusta AMU | Targeted Monitoring (Tetrachloroetyhlene, vinyl chloride) | 34.52974 | -83.41875 | X |  |  | X |  |  |  |  |  |
| RV_01_19 | Crawford Creek at County Road 118 near Lavonia, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 34.48206 | -83.12227 | X |  | X |  |  |  |  |  |  |
| RV_01_244 | Charlies Creek at Charlies Creek Rd East of Hiawassee, GA | Savannah | Atlanta AMU | Trend Monitoring (SEMN) | 34.95895 | -83.57158 | X |  |  | X | X | X | X | X |  |
| RV_01_248 | Coleman River at Coleman River Rd near Clayton, GA | Savannah | Atlanta AMU | Trend Monitoring (SEMN) | 34.95203324 | -83.51659881 | X |  |  | X | X | X | X | X |  |
| RV_01_266 | Shoal Creek at State Road 77 (Providence Church Rd) at Parkertown, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 34.45261 | -83.04182 | X |  | X |  |  |  |  |  |  |
| RV_01_44 | Hudson River at State Road 15 near Homer, GA | Savannah | Atlanta AMU | Targeted Monitoring (E coli) | 34.337312 | -83.487836 | X |  | X |  |  |  |  |  |  |
| RV_01_53 | Clark Creek at County Road 113 near Tignal, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 33.92666 | -82.81003 | X |  | X |  |  |  |  |  |  |
| RV_01_57 | Falling Creek at Sam Tate Rd / Co. Rd 50 near Fortsonia, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 34.00311 | -82.80827 | X |  | X |  |  |  |  |  |  |
| RV_01_66 | Chattooga River - U.S. Highway 76 near Clayton, GA | Savannah | Atlanta AMU | Trend Monitoring (USGS) | 34.813983 | -83.306433 | X |  | X | X |  |  |  |  |  |
| RV_01_87 | Savannah R BL Spirit Cr nr Augusta, GA | Savannah | Augusta AMU | Trend Monitoring (USGS) | 33.3306 | -81.9153 | X |  | X | X | X |  |  |  |  |
| RV_01_87 | Savannah River - 0.5 Mile Downstream from Spirit Creek | Savannah | Augusta AMU | Trend Monitoring (USGS) | 33.3306 | -81.9153 | X |  | X | X |  |  |  |  |  |
| RV_02_15770 | Ogeechee River at RM 5 at Middle Marsh Islands near Richmond Hill, GA | Ogeechee | Brunswick AMU | Estuary Trend Site | 31.856992 | -81.110452 | X | X |  |  |  |  |  |  | X |
| RV_02_17701 | North Fork Ogeechee at Brooks Rd | Ogeechee | Augusta AMU | Trend Monitoring | 33.58844 | -83.00938 | X |  | X | X |  | X | X | X |  |
| RV_02_18164 | Duhart Creek at Sand Valley Rd near Edgehill, GA | Ogeechee | Augusta AMU | Probabilistic Monitoring | 33.10931 | -82.49084 | X |  | X | X | X |  |  |  |  |
| RV_02_287 | Williamson Swamp Creek - Georgia Highway 231 | Ogeechee | Augusta AMU | Targeted Monitoring (E coli) | 32.97573 | -82.60997 | X |  | X |  |  |  |  |  |  |
| RV_02_296 | Ogeechee Creek at State Road 17 at Oliver, GA | Ogeechee | Brunswick AMU | Targeted Monitoring (E coli) | 32.524444 | -81.539722 | X |  | X |  |  |  |  |  |  |
| RV_02_298 | Ogeechee River - Georgia Highway 24 near Oliver, GA | Ogeechee | Brunswick AMU | Trend Monitoring (USGS) | 32.49475 | -81.555833 | X |  | X | X |  |  |  |  |  |
| RV_02_349 | Wateringhole Branch at Country Club Road | Ogeechee | Brunswick AMU | Targeted Monitoring (E coli) | 32.41486 | -81.84817 | X |  | X |  |  |  |  |  |  |
| RV_02_350 | Lotts Creek and State Road 250 (Nevils-Daisy Road) near Nevils, GA | Ogeechee | Brunswick AMU | Targeted Monitoring (E coli) | 32.264417 | -81.80835 | X |  | X |  |  |  |  |  |  |
| RV_02_457 | Little Lotts Creek at SR46 near Stateboro, GA | Ogeechee | Brunswick AMU | Trend Monitoring | 32.32603 | -81.8024 | X |  | X |  |  |  |  | X |  |
| RV_03_16356 | Kimbro Creek at Lanier Road | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.43653 | -83.12807 | X |  | X |  |  |  |  |  |  |
| RV_03_16783 | Pearson Creek at College Street near Monticello, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.32556 | -83.69175 | X |  | X |  |  |  |  |  |  |
| RV_03_17292 | Whitten Creek at SR 15 near White Plains, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.38682 | -83.02516 | X |  | X |  |  |  |  |  |  |
| RV_03_17706 | Little Bear Creek at Fowler Mill Rd near Athens, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.96587 | -83.51485 | X |  | X |  |  |  |  |  |  |
| RV_03_17797 | Pearson Creek at Maddux Street near Monticello, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.32164 | -83.69867 | X |  | X |  |  |  |  |  |  |
| RV_03_17813 | Turkey Creek at MLK Jr Dr near Eatonton, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.28477 | -83.32305 | X |  | X |  |  |  |  |  |  |
| RV_03_17817 | Trib to White Oak Creek at Jordan Rd near Monticello, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.29575 | -83.66833 | X |  | X |  |  |  |  |  |  |
| RV_03_18130 | Little River at Glades Rd near Eatonton, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.37281 | -83.4773 | X |  | X |  |  |  |  |  |  |
| RV_03_18136 | Flat Creek at US Highway 221 near Mount Vernon, GA | Oconee | Brunswick AMU | Probabilistic Monitoring | 32.207194 | -82.598359 | X |  | X | X | X |  |  |  |  |
| RV_03_18163 | Trib to Beaverdam Creek at Leslie Mill Rd near Greensboro, GA | Oconee | Augusta AMU | Probabilistic Monitoring | 33.49969 | -83.17106 | X |  | X | X | X |  |  |  |  |
| RV_03_18165 | Little Buckeye Creek at New Buckeye Rd near Oconee, GA | Oconee | Augusta AMU | Probabilistic Monitoring | 32.76997 | -82.86158 | X |  | X | X | X |  |  |  |  |
| RV_03_489 | North Oconee River at Georgia Highway 82 near Maysville, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.213983 | -83.546917 | X |  | X |  |  |  |  |  |  |
| RV_03_490 | North Oconee River at State Highway 335 Near Nicholson, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.117667 | -83.474367 | X |  | X |  |  |  |  |  |  |
| RV_03_500 | North Oconee River at Whitehall Road near Whitehall, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.90724 | -83.35999 | X |  | X |  |  |  |  |  |  |
| RV_03_501 | Cedar Creek at Barnett Shoals Drive near Athens, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.89561 | -83.33238 | X |  | X |  |  |  |  |  |  |
| RV_03_502 | Oconee River at Barnett Shoals Road near Athens, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli, USGS) | 33.85561 | -83.32684 | X |  | X |  |  |  |  |  |  |
| RV_03_508 | Town Creek at Cold Springs Road near Greensboro, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.61309 | -83.23891 | X |  | X |  |  |  |  |  |  |
| RV_03_5121 | Trib to Pittman Branch at Brook Hollow Way near Mansfield, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.506 | -83.718 | X |  | X |  |  |  |  |  |  |
| RV_03_541 | Town Creek at Old Covington Road County Road 39 near Greensboro, | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.54953 | -83.20865 | X |  | X |  |  |  |  |  |  |
| RV_03_542 | Richland Creek at Shelby Dreyer Road near Greensboro, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.54688 | -83.22103 | X |  | X |  |  |  |  |  |  |
| RV_03_543 | Richland Creek At Interstate 20 Near Greensboro | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.542222 | -83.223611 | X |  | X |  |  |  |  |  |  |
| RV_03_553 | Little River at Little River Road (Ga. 213) near Godfrey, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.451167 | -83.536633 | X |  | X |  |  |  |  |  |  |
| RV_03_559 | Little River Tributary 2 CR 212 (Glenwood Springs Rd) near Eatonton, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.295283 | -83.41675 | X |  | X |  |  |  |  |  |  |
| RV_03_567 | Big Cedar Creek at U.S. Highway 129 near Eatonton, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.186111 | -83.437222 | X |  | X |  |  |  |  |  |  |
| RV_03_570 | Pond Fork at Wayne Poultry Road near Pendergrass, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.18073 | -83.66086 | X |  | X |  |  |  |  |  |  |
| RV_03_572 | Allen Creek at Wayne Poultry Road near Pendergrass, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.17358 | -83.674 | X |  | X |  |  |  |  |  |  |
| RV_03_576 | Middle Oconee River at Etheridge Road near Arcade, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.04175 | -83.587759 | X |  | X |  |  |  |  |  |  |
| RV_03_585 | McNutt Creek at Jennings Mill Road, Athens, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.92665 | -83.46092 | X |  | X |  |  |  |  |  |  |
| RV_03_589 | Middle Oconee River at Macon Hwy near Athens, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.91859 | -83.3901 | X |  | X |  |  |  |  |  |  |
| RV_03_591 | Middle Oconee River at Whitehall Road at Athens, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.89064 | -83.37647 | X |  | X |  |  |  |  |  |  |
| RV_03_592 | Calls Creek at Hickory Hill Drive near Watkinsville, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.881454 | -83.3895 | X |  | X |  |  |  |  |  |  |
| RV_03_596 | North Oconee River at Diamond Hill Church Road (CR266) near Maysville ,GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.259888 | -83.645699 | X |  | X |  |  |  |  |  |  |
| RV_03_597 | Chandler Creek at Deadwyler Road near Maysville, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.24542 | -83.59101 | X |  | X |  |  |  |  |  |  |
| RV_03_599 | Rooty Creek at Martin Luther King Jr. Drive (County Road 90) near Eatonton, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.315278 | -83.365556 | X |  | X |  |  |  |  |  |  |
| RV_03_640 | Oconee River at Interstate Highway 16 near Dublin, GA | Oconee | Tifton AMU | Trend Monitoring (USGS) | 32.480367 | -82.858217 | X |  | X | X |  |  |  |  |  |
| RV_03_704 | Wolf Creek at Turner Woods Rd | Oconee | Atlanta AMU | Probabilistic Monitoring | 32.995263 | -83.495836 | X |  | X | X | X |  |  |  |  |
| RV_03_763 | Wheeler Creek at Bill Cheek Rd | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.082469 | -83.854552 | X |  | X |  |  |  |  |  |  |
| RV_03_784 | Beaverdam Creek at County Road 66 near Veazey, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.50471 | -83.15567 | X |  | X |  |  |  |  |  |  |
| RV_03_796 | E.T. Creek near Chicopee, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.26 | -83.837222 | X |  | X |  |  |  |  |  |  |
| RV_03_799 | Mulberry River at Old Covered Bridge Road near Hoschton, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 34.07832 | -83.7766 | X |  | X |  |  |  |  |  |  |
| RV_03_803 | Richland Creek at U.S. Hwy 278 / SR 12 near Greensboro ,GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.57682 | -83.21038 | X |  | X |  |  |  |  |  |  |
| RV_03_807 | Town Creek at Ga. Hwy 44 near Greensboro, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.551717 | -83.200433 | X |  | X |  |  |  |  |  |  |
| RV_04_15916 | Garner Creek at Five Forks Trickum Road nr Lawrenceville, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.861944 | -84.097182 | X |  | X |  |  |  |  |  |  |
| RV_04_15918 | Beaver Ruin Creek at Hillcrest Road near Norcross, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.928212 | -84.167021 | X |  | X |  |  |  |  |  |  |
| RV_04_18017 | Tributary to Doolitte Creek at Fontaine Circle in Atlanta, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (Tetrachloroethylene) | 33.72412 | -84.2857 | X |  |  |  |  |  |  |  |  |
| RV_04_18169 | Shetley Creek at Castlerock Drive near Norcross, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.9623 | -84.165804 | X |  | X |  |  |  |  |  |  |
| RV_04_2009 | Upton Creek at SR 42 / Moreland Ave | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.607103 | -84.302357 | X |  | X |  |  |  |  |  |  |
| RV_04_2058 | Bear Creek at McDonald Road near Mansfield ,GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.445923 | -83.812818 | X |  | X |  |  |  |  |  |  |
| RV_04_2059 | Doless Creek at Flat Shoals Road near Decatur, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.705898 | -84.27743 | X |  | X |  |  |  |  |  |  |
| RV_04_816 | Bromolow Creek at Shackleford Road | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.943056 | -84.143056 | X |  | X |  |  |  |  |  |  |
| RV_04_817 | Jackson Creek at Arcado Road near Luxomni, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.894417 | -84.114883 | X |  | X |  |  |  |  |  |  |
| RV_04_848 | South River - Georgia Highway 81 at Snapping Shoals | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 33.45265 | -83.927083 | X |  |  | X |  |  |  |  |  |
| RV_04_853 | South River at Island Shoals Road near Snapping Shoals, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.45265 | -83.927083 | X |  | X |  |  |  |  |  |  |
| RV_04_863 | Jacks Creek at State Road 264 near Centerville, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.820833 | -84.063889 | X |  | X |  |  |  |  |  |  |
| RV_04_869 | Yellow River at Pleasant Hill Road near Lithonia ,GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.733822 | -84.061609 | X |  | X |  |  |  |  |  |  |
| RV_04_873 | Brushy Creek at Rock Bridge Road near Rosebud, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (Special Request) | 33.798889 | -83.97 | X |  |  | X | X |  |  |  |  |
| RV_04_876 | Yellow River - Georgia Highway 212 near Stewart, GA | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 33.454267 | -83.881333 | X |  | X | X |  |  |  |  |  |
| RV_04_888 | Alcovy River - Newton Factory Bridge Road near Stewart | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 33.4494 | -83.8283 | X |  | X | X |  |  |  |  |  |
| RV_04_889 | Rocky Creek at Henderson Mill Road near Monticello, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.431388 | -83.779722 | X |  | X |  |  |  |  |  |  |
| RV_04_892 | Tussahaw Creek at Fincherville Road near Jackson, GA | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 33.378867 | -83.9634 | X |  |  | X |  |  |  |  |  |
| RV_04_899 | Walnut Creek at Elliot Road nr McDonough, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.4823 | -84.1188 | X |  | X |  |  |  |  |  |  |
| RV_04_937 | Big Haynes Creek at Bald Rock Rd / Costley Mill Rd | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.661772 | -83.928042 | X |  | X |  |  |  |  |  |  |
| RV_04_985 | Big Cotton Indian Creek at Homestead Rd | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.584363 | -84.260691 | X |  | X |  |  |  |  |  |  |
| RV_04_994 | Panther Creek at SR42/US23 | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.569167 | -84.262206 | X |  | X |  |  |  |  |  |  |
| RV_05_16777 | Echeconnee Creek at Rock Quarry Rd nr Yatesville | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 32.92305 | -84.11694 | X |  | X |  |  |  |  |  |  |
| RV_05_18159 | Brushy Creek at Pecan Orchard Rd. near Empire, GA | Ocmulgee | Tifton AMU | Probabilistic Monitoring | 32.299963 | -83.341652 | X |  | X | X | X |  |  |  |  |
| RV_05_18160 | Unnamed Trib to Gum Swamp Creek at Lister Cemetery Rd. near Yonkers, GA | Ocmulgee | Tifton AMU | Probabilistic Monitoring | 32.355434 | -83.208814 | X |  | X | X | X |  |  |  |  |
| RV_05_2105 | Wise Creek at Concord Road (County Road 141) near Monticello, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.256389 | -83.799722 | X |  | X |  |  |  |  |  |  |
| RV_05_2112 | Big Sandy Creek at State Road 87 near Sandy, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.195278 | -83.850556 | X |  | X |  |  |  |  |  |  |
| RV_05_2135 | Tobesofkee Creek at Ramah Church Road near Barnesville, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 33.0307 | -84.0768 | X |  | X |  |  |  |  |  |  |
| RV_05_2165 | Ocmulgee River - New Macon Water Intake | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 32.899247 | -83.664064 | X |  | X | X |  |  |  |  |  |
| RV_05_2203 | Ocmulgee River at Hawkinsville, GA | Ocmulgee | Tifton AMU | Trend Monitoring (USGS) | 32.281759 | -83.462773 | X |  | X | X |  |  |  |  |  |
| RV_05_2217 | House Creek at Walker Road near Forest Glen, GA | Ocmulgee | Tifton AMU | Targeted Monitoring (E coli) | 31.848783 | -83.2533 | X |  | X |  |  |  |  |  |  |
| RV_05_2223 | Ocmulgee River - US Highway 341 at Lumber City, GA | Ocmulgee | Brunswick AMU | Trend Monitoring (USGS) | 31.919933 | -82.674267 | X |  | X | X |  |  |  |  |  |
| RV_05_2224 | Gully Creek at Liberty Church Road (CR244) near Hazelhurst, GA | Ocmulgee | Brunswick AMU | Targeted Monitoring (E coli) | 31.91386 | -82.55355 | X |  | X |  |  |  |  |  |  |
| RV_05_2230 | Little Ocmulgee River - U.S. Hwy 280 / SR 30 | Ocmulgee | Tifton AMU | Targeted Monitoring (E coli) | 32.080859 | -82.888138 | X |  | X |  |  |  |  |  |  |
| RV_05_2232 | Alligator Creek at State Road 46 near McRae, GA | Ocmulgee | Tifton AMU | Targeted Monitoring (E coli) | 32.228611 | -82.984722 | X |  | X |  |  |  |  |  |  |
| RV_05_2238 | Sugar Creek at State Road 27 near Lumber City, GA | Ocmulgee | Brunswick AMU | Targeted Monitoring (DOC, Color) | 31.959722 | -82.727222 | X |  |  |  |  |  |  |  |  |
| RV_06_16760 | Big Cedar Creek at Bartow Dublin Hwy near Wrightsville, Ga. | Altamaha | Augusta AMU | Targeted Monitoring (E coli) | 32.73294 | -82.70701 | X |  | X |  |  |  |  |  |  |
| RV_06_18134 | Brazells Creek at Lynntown Road near Reidsville, GA | Altamaha | Brunswick AMU | Targeted Monitoring (pH) | 32.095962 | -82.13335 | X |  |  |  |  |  |  |  |  |
| RV_06_18135 | Trib to Rocky Creek at Rodney Stanley Road near Lyons, GA | Altamaha | Brunswick AMU | Probabilistic Monitoring | 32.102219 | -82.281576 | X |  | X | X | X |  |  |  |  |
| RV_06_2840 | Altamaha River at State Road 121 near Surrency, GA | Altamaha | Brunswick AMU | Probabilistic Monitoring | 31.583889 | -82.094167 | X |  | X | X | X |  |  |  |  |
| RV_06_2846 | Altamaha River - 6 Miles Downstream From Doctortown | Altamaha | Brunswick AMU | Trend Monitoring (USGS) | 31.6233 | -81.7653 | X |  | X | X |  |  |  |  |  |
| RV_06_2860 | Ohoopee River at Harts Ford Road (County Road 239) near Harrison, GA | Altamaha | Augusta AMU | Targeted Monitoring (E coli) | 32.81111 | -82.80645 | X |  | X |  |  |  |  |  |  |
| RV_06_2864 | Big Cedar Crk at Liberty Grove Church Rd (CR 175) near Wrightsville, GA | Altamaha | Augusta AMU | Targeted Monitoring (E coli) | 32.68103 | -82.68735 | X |  | X |  |  |  |  |  |  |
| RV_06_2887 | Ohoopee River at State Road 292 near Lyons, GA | Altamaha | Brunswick AMU | Probabilistic Monitoring | 32.194167 | -82.192222 | X |  | X | X | X |  |  |  |  |
| RV_06_2906 | Brazells Creek at Brazell Street west of Reidsville, GA | Altamaha | Brunswick AMU | Targeted Monitoring (pH) | 32.091318 | -82.148621 | X |  |  |  |  |  |  |  |  |
| RV_07_16397 | Trib to Trib to Seventeen Mile River at 10th Street near Douglas, GA | Satilla | Brunswick AMU | Targeted Monitoring (Req-NH3) | 31.501813 | -82.841701 | X |  |  | X |  |  |  |  |  |
| RV_07_16398 | Trib to Trib to Seventeen Mile River at Gaskin Avenue near Douglas, GA | Satilla | Brunswick AMU | Targeted Monitoring (Req-NH3) | 31.502071 | -82.845428 | X |  |  | X |  |  |  |  |  |
| RV_07_17554 | Trib to Trib to Seventeen Mile River 100 m downstream of McDonald Rd near Douglas, GA | Satilla | Brunswick AMU | Targeted Monitoring (Req-NH3) | 31.501623 | -82.842639 | X |  |  | X |  |  |  |  |  |
| RV_07_17660 | Satilla Creek at Pine Level Church Rd. near Fitzgerald, Ga | Satilla | Tifton AMU | Targeted Monitoring (E coli) | 31.69609 | -83.16842 | X |  | X |  |  |  |  |  |  |
| RV_07_18068 | Big Satilla Creek at US Highway 84 near Screven, GA | Satilla | Brunswick AMU | Trend Monitoring | 31.457226 | -82.056476 | X |  | X | X | X | X |  | X |  |
| RV_07_18137 | Trib to Twenty Mile Creek at Church Street near Douglas, GA | Satilla | Brunswick AMU | Targeted Monitoring (Chrysene and Benzo(a)anthracene) | 31.52079 | -82.8522 | X |  |  |  |  |  |  |  |  |
| RV_07_2892 | Swift Creek at State Road 152 near Lyons, GA (Special Project see comments) | Satilla | Brunswick AMU | Targeted Monitoring (Additional data) | 32.222222 | -82.298889 | X |  |  |  |  |  |  |  |  |
| RV_07_2986 | Satilla River Georgia Highways 15 and 121 | Satilla | Brunswick AMU | Trend Monitoring (USGS) | 31.2167 | -82.1625 | X |  | X | X |  |  |  |  |  |
| RV_07_2993 | Little Hurricane Creek at Highway 1 near Waycross, GA | Satilla | Brunswick AMU | Targeted Monitoring (E coli) | 31.423477 | -82.432838 | X |  | X |  |  |  |  |  |  |
| RV_07_2997 | Alabaha River - SR 203 | Satilla | Brunswick AMU | Targeted Monitoring (E coli) | 31.37547 | -82.28867 | X |  | X |  |  |  |  |  |  |
| RV_07_3001 | Satilla River at US Highway 82 near Atkinson, GA | Satilla | Brunswick AMU | Probabilistic Monitoring | 31.221111 | -81.8675 | X |  | X | X | X |  |  |  |  |
| RV_07_3013 | Big Satilla Creek at State Road 203 near Baxley, GA | Satilla | Brunswick AMU | Probabilistic Monitoring | 31.590833 | -82.311667 | X |  | X | X | X |  |  |  |  |
| RV_09_17661 | Reedy Creek at Bethlehem Church Rd near Ocilla, Ga | Suwannee | Tifton AMU | Targeted Monitoring (E coli) | 31.48904 | -83.17775 | X |  | X |  |  |  |  |  |  |
| RV_09_17673 | Woodyard Creek at Bypass Road near Homerville, GA | Suwannee | Brunswick AMU | Targeted Monitoring (E coli) | 31.03621 | -82.7288 | X |  | X |  |  |  |  |  |  |
| RV_09_18161 | Alapaha River at SR 107 near Rebecca, GA | Suwannee | Tifton AMU | Probabilistic Monitoring | 31.721059 | -83.455564 | X |  | X | X | X |  |  |  |  |
| RV_09_18162 | Pindar Creek at Mayday Rd. near Fruitland, GA | Suwannee | Tifton AMU | Probabilistic Monitoring | 30.826176 | -83.00755 | X |  | X | X | X |  |  |  |  |
| RV_09_3181 | Suwannee River at US Highway 441 near Fargo, GA | Suwannee | Brunswick AMU | Trend Monitoring (USGS) | 30.6806 | -82.5606 | X |  | X | X |  |  |  |  |  |
| RV_09_3182 | Tatum Creek at CR 37 (Clarence Smith Rd) near Homerville, GA | Suwannee | Brunswick AMU | Targeted Monitoring (E coli) | 30.99342 | -82.71747 | X |  | X |  |  |  |  |  |  |
| RV_09_3183 | Tatum Creek at US Highway 441 near Homerville, GA | Suwannee | Brunswick AMU | Targeted Monitoring (E coli) | 30.896389 | -82.665833 | X |  | X |  |  |  |  |  |  |
| RV_09_3190 | Little Brushy Creek at County Road 63 (Harrell Road) near Ocilla, GA | Suwannee | Tifton AMU | Targeted Monitoring (E coli) | 31.5075 | -83.208889 | X |  | X |  |  |  |  |  |  |
| RV_09_3236 | Withlacoochee River at Clyattsville-Nankin Road near Clyattsville, GA | Suwannee | Tifton AMU | Trend Monitoring (USGS) | 30.674722 | -83.394722 | X |  | X | X |  |  |  |  |  |
| RV_09_3242 | Little River at Upper Ty Ty Road near Tifton | Suwannee | Tifton AMU | Probabilistic Monitoring | 31.481667 | -83.584167 | X |  | X | X | X |  |  |  |  |
| RV_10_16316 | Pine Creek at SR3 near Ochlocknee, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Req-NH3) | 30.963491 | -84.045693 | X |  |  |  |  |  |  |  |  |
| RV_10_17558 | Pine Creek U/S Ochlockonnee WPCP near Ochlockonee, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Req-NH3) | 30.964723 | -84.048281 | X |  |  |  |  |  |  |  |  |
| RV_10_3384 | Tired Creek at County Road 151 near Reno, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.763611 | -84.229444 | X |  |  | X |  |  |  |  |  |
| RV_10_3386 | Ochlockonee River @ Hadley Ferry Rd. nr Calvary, GA | Ochlockonee | Tifton AMU | Trend Monitoring (USGS) | 30.731717 | -84.235533 | X |  | X | X |  |  |  |  |  |
| RV_10_3388 | Aucilla River at Twelve Mile Post Rd. near Boston, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (E coli) | 30.712452 | -83.744396 | X |  | X |  |  |  |  |  |  |
| RV_10_3389 | Attapulgus Creek at U.S. Hwy 27 near Attapulgus, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.732778 | -84.453611 | X |  |  | X |  |  |  |  |  |
| RV_10_3390 | Swamp Creek at US Hwy 27 near Attapulgus, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.719444 | -84.411389 | X |  |  | X |  |  |  |  |  |
| RV_10_3422 | Little Attapulgus Creek at Faceville-Attapulgus Rd. near Attapulgus, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.750046 | -84.501333 | X |  |  | X |  |  |  |  |  |
| RV_10_3423 | Little Attapulgus Creek at State Rd 241 near Attapulgus, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.718056 | -84.49 | X |  |  | X |  |  |  |  |  |
| RV_10_4989 | Olive Creek at Cone Rd nr Thomasville, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (E coli) | 30.795108 | -83.904104 | X |  | X |  |  |  |  |  |  |
| RV_11_15907 | Keg Creek at Stallings Road/McIntosh Trail near Senoia, GA | Flint | Atlanta AMU | Probabilistic Monitoring | 33.332198 | -84.56699 | X |  | X | X | X |  |  |  |  |
| RV_11_16473 | Spring Hill Creek at South Street (D/S Marshallville WPCP) near Marshallville, GA | Flint | Tifton AMU | Targeted Monitoring (Additional data) | 32.439369 | -83.94293 | X |  |  |  |  |  |  |  |  |
| RV_11_16474 | Spring Hill Creek at South Street (U/S Marshallville WPCP) near Marshallville, GA | Flint | Tifton AMU | Targeted Monitoring (Additional data) | 32.44107 | -83.942959 | X |  |  |  |  |  |  |  |  |
| RV_11_16802 | Sand Creek at Railroad St. (D/S Ideal WPCP) near Ideal, GA | Flint | Tifton AMU | Targeted Monitoring (Req-NH3) | 32.378023 | -84.188619 | X |  |  |  |  |  |  |  |  |
| RV_11_17565 | Sand Creek at Martin Luther King Ave (U/S Ideal WPCP) near Ideal, GA | Flint | Tifton AMU | Targeted Monitoring (Req-NH3) | 32.38202 | -84.1992 | X |  |  |  |  |  |  |  |  |
| RV_11_18018 | Sullivan Creek at Edison Dr in Atlanta, GA | Flint | Atlanta AMU | Targeted Monitoring (Dieldrin) | 33.62976 | -84.46552 | X |  |  |  |  |  |  |  |  |
| RV_11_18127 | Trib to Shoal Creek at Brady Road | Flint | Atlanta AMU | Probabilistic Monitoring | 32.410761 | -84.470708 | X |  | X | X | X |  |  |  |  |
| RV_11_3485 | Flint River at State Road 92 near Griffin, GA | Flint | Atlanta AMU | Trend Monitoring (USGS) | 33.3089 | -84.393056 | X |  | X | X |  |  |  |  |  |
| RV_11_3507 | Flint River at State Road 26/49 near Montezuma, GA | Flint | Tifton AMU | Trend Monitoring (USGS) | 32.2981 | -84.043889 | X |  | X | X |  |  |  |  |  |
| RV_11_3511 | Flint River at SR 26 near Montezuma | Flint | Tifton AMU | Trend Monitoring (USGS) | 32.29295 | -84.044067 | X |  | X | X |  |  |  |  |  |
| RV_11_3553 | Flint River at State Road 234 near Albany, GA | Flint | Tifton AMU | Trend Monitoring (USGS) | 31.5524 | -84.1463 | X |  | X | X |  |  |  |  |  |
| RV_11_3558 | Flint River at State Road 37 at Newton, GA | Flint | Tifton AMU | Trend Monitoring (USGS) | 31.309444 | -84.335 | X |  | X | X |  |  |  |  |  |
| RV_11_3563 | Flint River at U.S. Highway 27-B near Bainbridge, GA | Flint | Tifton AMU | Trend Monitoring (USGS) | 30.910946 | -84.580514 | X |  | X | X |  |  |  |  |  |
| RV_11_3605 | Line Creek at Rockaway Rd | Flint | Atlanta AMU | Targeted Monitoring (Additional data) | 33.335716 | -84.536739 | X |  |  |  |  |  |  |  |  |
| RV_11_3610 | Bear Creek at Wildwood Rd./ Bear Creek Rd | Flint | Atlanta AMU | Targeted Monitoring (Additional data) | 33.352347 | -84.359427 | X |  |  |  |  |  |  |  |  |
| RV_11_3789 | Flint River at Sprewell Bluff Sprewell Bluff State Park | Flint | Atlanta AMU | Trend Monitoring | 32.855988 | -84.476812 | X |  | X | X | X | X | X |  |  |
| RV_11_3804 | Lime Creek at Springhill Church Road east of Americus, Ga | Flint | Tifton AMU | Trend Monitoring | 32.035 | -83.9925 | X |  | X |  |  |  | X | X |  |
| RV_11_3807 | Little Ichawaynochaway Creek at CR 3 near Shellman, Ga | Flint | Tifton AMU | Trend Monitoring | 31.803532 | -84.640013 | X |  | X | X | X | X | X | X |  |
| RV_12_17578 | Sweetwater Creek at Blairs Bridge Rd near Lithia Springs, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.77454 | -84.61455 | X |  | X | X | X | X | X |  |  |
| RV_12_17688 | Trib to Snake Creek near Newnan, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (Trichlorethylene, pH) | 33.393887 | -84.81949 | X |  |  |  |  |  |  |  |  |
| RV_12_18085 | Camp Creek at Cascade-Palmetto Hwy (Hwy 70), near Campbellton | Chattahoochee | Atlanta AMU | Targeted Monitoring (Revisit) | 33.675528 | -84.630941 | X |  | X | X | X |  |  |  |  |
| RV_12_18103 | Big Creek at Grimes Bridge Rd (Roswell, GA) | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 34.018996 | -84.345133 | X |  | X | X | X | X | X |  |  |
| RV_12_18122 | Foe Killer Creek at Mansell Rd | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.041 | -84.31709 | X |  | X |  |  |  |  |  |  |
| RV_12_18123 | North Utoy Creek at Falls Dr SW | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 33.74052 | -84.50464 | X |  | X |  |  |  |  |  |  |
| RV_12_18124 | Pepperell Creek at Darden Meadows Dr | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 33.05916 | -84.99649 | X |  | X |  |  |  |  |  |  |
| RV_12_18125 | Lee Branch at Forrest Avenue | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 33.01574 | -85.04728 | X |  | X |  |  |  |  |  |  |
| RV_12_18126 | Trib to Sulphur Creek at Old Durand road/Winter Road | Chattahoochee | Atlanta AMU | Probabilistic Monitoring | 32.940977 | -84.765901 | X |  | X | X | X |  |  |  |  |
| RV_12_18128 | Fort Creek at J street U/S of Koch Discharge near Pine Mountain Valley | Chattahoochee | Atlanta AMU | Targeted Monitoring (Req-NH3) | 32.808062 | -84.807744 | X |  | X |  |  |  |  |  |  |
| RV_12_18129 | North Fork Balus Creek at Tumbling Creek Road | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.24274 | -83.85935 | X |  | X |  |  |  |  |  |  |
| RV_12_18131 | Woodall Creek at Chattahoochee Ave NW in Atlanta | Chattahoochee | Atlanta AMU | Targeted Monitoring (Tetrachloroetyhlene, PCBs) | 33.80279 | -84.43166 | X |  |  |  |  |  |  |  |  |
| RV_12_18132 | Woodall Creek at Collier Rd NW in Atlanta | Chattahoochee | Atlanta AMU | Targeted Monitoring (B-BHC) | 33.809371 | -84.434465 | X |  |  |  |  |  |  |  |  |
| RV_12_18133 | Tributary to Woodall creek at Marietta Blvd NW in Atlanta | Chattahoochee | Atlanta AMU | Targeted Monitoring (alpha BHC, beta BHC, copper, zinc) | 33.79586 | -84.43269 | X |  |  |  |  |  |  |  |  |
| RV_12_18158 | Chattahoochee River near Odom Creek Rd. near Blakely, GA | Chattahoochee | Tifton AMU | Probabilistic Monitoring | 31.38688762 | -85.08113213 | X |  | X | X | X |  |  |  |  |
| RV_12_3841 | Chattahoochee River at McGinnis Ferry Road | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 34.050556 | -84.097701 | X |  | X |  | X |  |  |  |  |
| RV_12_3859 | Chattahoochee River - DeKalb County Water Intake | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.9731 | -84.2631 | X |  | X |  |  |  |  |  |  |
| RV_12_3870 | Chattahoochee River at Cobb County Water Intake near Roswell, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.9443 | -84.405 | X |  | X |  |  |  |  |  |  |
| RV_12_3891 | Chattahoochee River - Atlanta Water Intake | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.8278 | -84.455 | X |  | X |  | X |  |  |  |  |
| RV_12_3902 | Chattahoochee River at Belton Bridge Road near Lula, GA | Chattahoochee | Cartersville AMU | Trend Monitoring (USGS) | 34.445145 | -83.68423 | X |  | X | X |  |  |  |  |  |
| RV_12_3925 | Chestatee River at State Road 400 near Dahlonega, GA | Chattahoochee | Cartersville AMU | Trend Monitoring (USGS) | 34.466667 | -83.968889 | X |  | X | X |  |  |  |  |  |
| RV_12_3934 | Chattahoochee River at Bankhead Highway | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.795278 | -84.507778 | X |  | X |  | X |  |  |  |  |
| RV_12_3939 | Prob - Chattahoochee River - Georgia Highway 139 | Chattahoochee | Atlanta AMU | Probabilistic Monitoring | 33.779167 | -84.533611 | X |  | X | X | X |  |  |  |  |
| RV_12_3960 | Chattahoochee River at Capps Ferry Road near Rico, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.5778 | -84.808611 | X |  | X |  |  |  |  |  |  |
| RV_12_4003 | Flat Creek at McEver Road near Gainesville, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 34.265833 | -83.885 | X |  | X | X |  |  |  |  |  |
| RV_12_4039 | New River at State Road 100 near Corinth, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 33.235278 | -84.987778 | X |  | X | X |  |  |  |  |  |
| RV_12_4041 | Chattahoochee River at GA 100 at Franklin, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 33.2897 | -85.088512 | X |  | X | X |  |  |  |  |  |
| RV_12_4049 | Yellow Jacket Creek at Hammet Road near Hogansville, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 33.139167 | -84.975278 | X |  | X | X |  |  |  |  |  |
| RV_12_4094 | Chattahoochee River at Spur 39 near Omaha, Ga. | Chattahoochee | Tifton AMU | Trend Monitoring (USGS) | 32.1436 | -85.045278 | X |  | X | X |  |  |  |  |  |
| RV_12_4110 | Chattahoochee River at State Road 91 near Steam Mill, GA | Chattahoochee | Tifton AMU | Trend Monitoring (USGS) | 30.9775 | -85.005278 | X |  | X | X |  |  |  |  |  |
| RV_12_4123 | Hillabahatchee Creek at CR 210 near Frolona, GA | Chattahoochee | Atlanta AMU | Trend Monitoring | 33.311218 | -85.187675 | X |  | X | X | X | X | X | X |  |
| RV_12_4218 | Town Branch at Brewer Rd | Chattahoochee | Cartersville AMU | Targeted Monitoring (Req-NH3) | 33.75386 | -84.86242 | X |  | X |  |  |  |  |  |  |
| RV_12_4219 | Town Branch at Mirror Lake Pkwy | Chattahoochee | Cartersville AMU | Targeted Monitoring (Req-NH3) | 33.743194 | -84.881337 | X |  | X |  |  |  |  |  |  |
| RV_12_4220 | Town Branch Creek d/s of North WPCP | Chattahoochee | Cartersville AMU | Targeted Monitoring (Req-NH3) | 33.743781 | -84.87916 | X |  | X |  |  |  |  |  |  |
| RV_12_4292 | Dicks Creek at Forest Service Road 144-1 near Neels Gap, GA | Chattahoochee | Cartersville AMU | Trend Monitoring (USGS) | 34.6797 | -83.937222 | X |  |  | X |  |  |  |  |  |
| RV_12_4316 | Peachtree Creek at Northside Dr in Atlanta, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.8194 | -84.407778 | X |  | X | X | X | X | X |  |  |
| RV_12_4317 | Pine Knot Creek at SR 355 near Box Springs, GA | Chattahoochee | Atlanta AMU | Probabilistic Monitoring | 32.439831 | -84.648404 | X |  | X | X | X |  |  |  |  |
| RV_12_5130 | Fort Creek at Hwy 116 D/S of Koch Discharge near Pine Mountain Valley | Chattahoochee | Atlanta AMU | Targeted Monitoring (Req-NH3) | 32.793924 | -84.802056 | X |  | X |  |  |  |  |  |  |
| RV_13_18138 | Baxter Creek at Old Hamilton Mill Road near Bremen, GA | Tallapoosa | Cartersville AMU | Targeted Monitoring (Req-NH3) | 33.74371 | -85.12827 | X |  | X |  |  |  |  |  |  |
| RV_13_18139 | Baxter Creek at Baxter Road near Bremen, GA | Tallapoosa | Cartersville AMU | Targeted Monitoring (Req-NH3) | 33.7489 | -85.1208 | X |  | X |  |  |  |  |  |  |
| RV_13_18140 | Baxter Creek at Hoke Golden Road near Bremen, GA | Tallapoosa | Cartersville AMU | Targeted Monitoring (Req-NH3) | 33.7722 | -85.115 | X |  | X |  |  |  |  |  |  |
| RV_13_4349 | Little Tallapoosa River - Georgia Highway 100 near Bowden, GA | Tallapoosa | Atlanta AMU | Trend Monitoring (USGS) | 33.4927778 | -85.279167 | X |  | X | X |  |  |  |  |  |
| RV_13_4353 | Tallapoosa River - Georgia Highway 8 below Tallapoosa, GA | Tallapoosa | Cartersville AMU | Trend Monitoring (USGS) | 33.740833 | -85.336389 | X |  | X | X |  |  |  |  |  |
| RV_14_15870 | Pinhook Creek at Hwy 411 | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.467173 | -84.700669 | X |  |  |  |  |  |  |  |  |
| RV_14_17273 | Woodward Creek at Gaines Loop near Rome, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.364356 | -85.07319 | X |  |  |  |  |  |  |  |  |
| RV_14_17477 | Ninetynine Branch at Irwin Mill Rd near Fairmount, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.42014 | -84.67966 | X |  |  |  |  |  |  |  |  |
| RV_14_18141 | Lawrence Creek at Head Place near Dallas, GA | Coosa | Cartersville AMU | Probabilistic Monitoring | 33.931605 | -84.835658 | X |  | X | X | X |  |  |  |  |
| RV_14_18142 | Buzzard Flapper Creek at Hube Turner Rd | Coosa | Cartersville AMU | Targeted Monitoring (Additional data) | 34.254045 | -84.290755 | X |  | X |  |  |  |  |  |  |
| RV_14_18143 | Holly Creek at Sweetwater Juno Rd | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.446 | -84.20173 | X |  | X |  |  |  |  |  |  |
| RV_14_18144 | Little Amicacola Creek at Afton Rd | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.50133 | -84.23446 | X |  | X |  |  |  |  |  |  |
| RV_14_18145 | Jacks Creek at Farmville Road | Coosa | Cartersville AMU | Probabilistic Monitoring | 34.45307 | -84.84482 | X |  | X | X | X |  |  |  |  |
| RV_14_18148 | Dick Creek at Manning Mill Road | Coosa | Cartersville AMU | New | 34.60115 | -85.15187 | X |  |  | X |  |  |  |  |  |
| RV_14_18149 | West Armurchee Creek at Manning Mill Road | Coosa | Cartersville AMU | New | 34.60861 | -85.16283 | X |  |  | X |  |  |  |  |  |
| RV_14_18150 | Armurchee Creek at Haywood Valley Road | Coosa | Cartersville AMU | New | 34.45687 | -85.18748 | X |  |  | X |  |  |  |  |  |
| RV_14_18151 | Rocky Creek at Battle Road SW | Coosa | Cartersville AMU | New | 34.43827 | -85.08807 | X |  |  | X |  |  |  |  |  |
| RV_14_18157 | Mount Hope Creek @ Old River Road | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.24883 | -85.40736 | X |  | X |  |  |  |  |  |  |
| RV_14_18168 | Etowah River at Boling Park Kayak Launch | Coosa | Cartersville AMU | Targeted Monitoring (Additional data) | 34.23369 | -84.50594 | X |  |  |  |  |  |  |  |  |
| RV_14_4438 | Conasauga at U.S. Highway 76 near Dalton, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.783 | -84.873014 | X |  | X | X |  |  |  |  |  |
| RV_14_4460 | Conasauga River at Tilton Bridge near Tilton, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.6667 | -84.9283 | X |  | X | X |  |  |  |  |  |
| RV_14_4467 | Tickanetley Creek at Roy Road east of Ellijay, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.644444 | -84.331389 | X |  |  |  |  |  |  |  |  |
| RV_14_4518 | Mountaintown Creek at State Road 282 (US Hwy 76) near Ellijay, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.703378 | -84.539793 | X |  | X | X |  |  |  |  |  |
| RV_14_4520 | Coosawattee River at Georgia Highway 5 near Ellijay, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.6717 | -84.500164 | X |  | X | X |  |  |  |  |  |
| RV_14_4534 | Oostanaula River at Rome Water Intake near Rome, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.2703 | -85.1733 | X |  | X | X |  |  |  |  |  |
| RV_14_4549 | Etowah River at State Road 5 spur near Canton, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.239722 | -84.494423 | X |  |  | X |  |  |  |  |  |
| RV_14_4550 | Shoal Creek at State Road 108 (Fincher Rd.) near Waleska, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.263333 | -84.595556 | X |  | X | X |  |  |  |  |  |
| RV_14_4555 | Little River at Georgia Highway 5 near Woodstock, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.1222 | -84.504254 | X |  | X | X |  |  |  |  |  |
| RV_14_4586 | Etowah River at Hardin Bridge (FAS 829) near Euharlee, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.188859 | -84.925104 | X |  | X | X |  |  |  |  |  |
| RV_14_4594 | Etowah River at SR1 Loop near Rome, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.232222 | -85.116944 | X |  |  |  |  |  |  |  |  |
| RV_14_4599 | Coosa River at Heritage Park at Rome, GA | Coosa | Cartersville AMU | Probabilistic Monitoring | 34.255278 | -85.179444 | X |  | X | X | X |  |  |  |  |
| RV_14_4622 | Coosa River - Georgia/Alabama State Line Monitor near Cave Springs, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.1983 | -85.443928 | X |  |  | X |  |  |  |  |  |
| RV_14_4640 | Chattooga River at Holland-Chattoogaville Road (FAS1363) near Lyerly, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.3356 | -85.4453 | X |  | X | X |  |  |  |  |  |
| RV_14_4735 | Cedar Creek on Littlefield Rd | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.393946 | -84.835649 | X |  |  |  |  |  |  |  |  |
| RV_14_4748 | Little Pine Log Creek at Perry Rd | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.39445 | -84.7845 | X |  |  |  |  |  |  |  |  |
| RV_14_4754 | Sugar Hill Creek at Knuckelsville Rd | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.360448 | -84.711863 | X |  |  |  |  |  |  |  |  |
| RV_14_4819 | Clarks Creek nr Taliaferro Springs Rd nr Lyerly, GA | Coosa | Cartersville AMU | Targeted Monitoring (Additional data) | 34.377127 | -85.393541 | X |  | X |  |  |  |  |  |  |
| RV_14_4820 | Clear Creek at Blackberry Mountain Rd nr East Ellijay, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.619589 | -84.436956 | X |  |  |  |  |  |  |  |  |
| RV_14_4825 | Dozier Creek at Bells Ferry Road near Rome, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.320833 | -85.110278 | X |  |  |  |  |  |  |  |  |
| RV_14_4829 | Dykes Creek at Dykes Creek Crossing | Coosa | Cartersville AMU | Trend Monitoring | 34.29357 | -85.0855 | X |  | X | X | X | X |  | X | X |
| RV_14_4837 | Jones Creek near Jones Creek Rd, Dahlonega, GA | Coosa | Atlanta AMU | Trend Monitoring (SEMN) | 34.602401 | -84.150559 | X |  |  | X | X | X | X | X |  |
| RV_14_4841 | Lick Creek nr Langford Rd NE, Fairmount, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.534829 | -84.796003 | X |  |  |  |  |  |  |  |  |
| RV_14_4851 | Noonday Creek at Georgia Highway 92 near Woodstock, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.08547 | -84.529354 | X |  | X | X |  |  |  |  |  |
| RV_14_5136 | Lick Log Creek at SR 52 | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.6418 | -84.38727 | X |  |  |  |  |  |  |  |  |
| RV_15_18146 | East Chickamauga Creek at Houston Valley Road | Tennessee | Cartersville AMU | New | 34.795832 | -85.07418 | X |  |  | X |  |  |  |  |  |
| RV_15_18147 | Hopkins Branch at Lafeyette Road | Tennessee | Cartersville AMU | New | 34.77998 | -85.06388 | X |  |  | X |  |  |  |  |  |
| RV_15_18152 | Mulky Creek at Mulky Gap Road | Tennessee | Cartersville AMU | Probabilistic Monitoring | 34.790926 | -84.049298 | X |  | X | X | X |  |  |  |  |
| RV_15_18153 | Suches Creek at Hwy 60 | Tennessee | Cartersville AMU | New | 34.70393 | -84.05555 | X |  |  | X |  |  |  |  |  |
| RV_15_18154 | Davis Creek at Hwy 60 | Tennessee | Cartersville AMU | New | 34.73036 | -84.081006 | X |  |  | X |  |  |  |  |  |
| RV_15_18155 | Grizzle Creek at Hwy 60 | Tennessee | Cartersville AMU | New | 34.71895 | -84.06667 | X |  |  | X |  |  |  |  |  |
| RV_15_18156 | Cooper Creek at Cooper Creek Recreation Area | Tennessee | Cartersville AMU | New | 34.76319 | -84.06878 | X |  |  | X |  |  |  |  |  |
| RV_15_4903 | Toccoa River - County Road 165 Near Gaddistown | Tennessee | Cartersville AMU | Targeted Monitoring (Additional data) | 34.711111 | -84.072222 | X |  |  |  |  |  |  |  |  |
| RV_15_4918 | West Chickamauga Creek - Georgia Highway 146 near Ringgold, GA | Tennessee | Cartersville AMU | Trend Monitoring (USGS) | 34.9572 | -85.2056 | X |  | X | X |  |  |  |  |  |
| RV_15_4957 | Cooper Creek at State Road 60 near Suches, GA | Tennessee | Cartersville AMU | Targeted Monitoring (Additional data) | 34.743241 | -84.124627 | X |  |  |  |  |  |  |  |  |
| RV_15_4961 | E. Chickamauga Creek at Lower Gordon Springs Rd | Tennessee | Cartersville AMU | Trend Monitoring | 34.74717 | -85.1243 | X |  | X | X | X | X |  | X | X |

Routine field and chemical parameters include: gage height / tape down or discharge measurement, air temperature, water temperature, dissolved oxygen, pH, specific conductance, turbidity, 5-day BOD, alkalinity, hardness, suspended solids, ammonia, nitrate-nitrite, Kjeldahl nitrogen, total phosphorus, and total organic carbon.

Lakes/estuaries field, chemical and biological parameters include:  water depth, secchi disk transparency, photic zone depth, air temperature, depth profiles for dissolved oxygen, temperature, pH, and specific conductance, and chemical analyses for turbidity, specific conductance, 5-day BOD, pH, alkalinity, hardness, suspended solids, ammonia, nitrate-nitrite, Kjeldahl nitrogen, total phosphorus, total organic carbon, and chlorophyll a.

1 Sampling Organization:  Atlanta AMU = GAEPD Atlanta office; Augusta AMU =  GAEPD Phinizy Center for Water Sciences; Brunswick AMU = GAEPD Brunswick Regional Office; Cartersville AMU = GAEPD Cartersville Regional Office; Tifton AMU = GAEPD Tifton Regional office.

2 Routine field and chemical parameters include: gage height/tape down or discharge measurement, air temperature, water temperature, dissolved oxygen, pH, specific conductance, turbidity, 5-day BOD, alkalinity, hardness, suspended solids, ammonia, nitrate-nitrite, Kjeldahl nitrogen, total phosphorus, total organic carbon, and E. coli.

3 Biomonitoring: conducted for invertebrates and periphyton using Georgia EPD protocols.

1b. 2026 SURFACE WATER MONITORING STATIONS

Rivers/Streams, Lakes/Reservoirs, Estuaries/Sounds

Rivers and streams stations are sampled monthly for field and chemical parameters for one calendar.  Bacterial samples are collected monthly

Lakes, reservoirs, and estuaries are sampled once a month during the growing season (April-October).

| Georgia Station Number | Sampling Site | River Basin | Sampling Organization1 | Waterbody Type/Project | Latitude | Longitude | Routine2 | Enterococcus | E. coli | Orthophosphate s | Metals | Macroinvertebrates3 | Periphyton3 | Discharge | Chlorophyll |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LK_01_10 | Lake Rabun - Dampool (aka Tallulah River - Upstream From Mathis Dam) | Savannah | Cartersville AMU | Lake Trend Monitoring | 34.764722 | -83.417778 | X |  | X |  |  |  |  |  | X |
| LK_01_11 | Lake Hartwell at Interstate 85 | Savannah | Augusta AMU | Lake Trend Monitoring | 34.484167 | -83.029833 | X |  | X |  |  |  |  |  | X |
| LK_01_22 | Lake Hartwell - Dam Forebay | Savannah | Augusta AMU | Lake Trend Monitoring | 34.358733 | -82.824417 | X |  | X |  |  |  |  |  | X |
| LK_01_27 | Lake Russell Between Markers 42 and 44 (Mid Lake) | Savannah | Augusta AMU | Lake Trend Monitoring | 34.127778 | -82.673611 | X |  | X |  |  |  |  |  | X |
| LK_01_29 | Lake Richard B. Russell - Dam Forebay | Savannah | Augusta AMU | Lake Trend Monitoring | 34.026333 | -82.594167 | X |  | X |  |  |  |  |  | X |
| LK_01_38 | Clarks Hill Lake- Savannah River At U.S. Highway 378 | Savannah | Augusta AMU | Lake Trend Monitoring | 33.857861 | -82.399583 | X |  | X |  |  |  |  |  | X |
| LK_01_39 | Clarks Hill Lake- Savannah River At Dordon Crk. | Savannah | Augusta AMU | Lake Trend Monitoring | 33.765861 | -82.271778 | X |  | X |  |  |  |  |  | X |
| LK_01_40 | Clarks Hill Lake - Dam Forebay | Savannah | Augusta AMU | Lake Trend Monitoring | 33.662694 | -82.198528 | X |  | X |  |  |  |  |  | X |
| LK_01_67 | Lake Tugalo - u/s Tugalo Lake Rd (aka Bull Sluice Rd.) | Savannah | Atlanta AMU | Lake Trend Monitoring | 34.737805 | -83.340555 | X |  | X |  |  |  |  |  | X |
| LK_01_68 | Lake Tugalo - Upstream From Tugaloo Dam | Savannah | Atlanta AMU | Lake Trend Monitoring | 34.715 | -83.351694 | X |  | X |  |  |  |  |  | X |
| LK_01_7 | Lake Burton - 1/4 mile South of Burton Island (aka Tallulah River) | Savannah | Cartersville AMU | Lake Trend Monitoring | 34.835233 | -83.553817 | X |  | X |  |  |  |  |  | X |
| LK_01_71 | Clarks Hill Lake - Little River At Highway 47 | Savannah | Augusta AMU | Lake Trend Monitoring | 33.692722 | -82.338805 | X |  | X |  |  |  |  |  | X |
| LK_01_8 | Lake Burton - Dampool (aka Tallulah River u/s Lake Burton Dam) | Savannah | Cartersville AMU | Lake Trend Monitoring | 34.795317 | -83.5401 | X |  | X |  |  |  |  |  | X |
| LK_01_9 | Lake Rabun - Approx. 4.5 mi u/s Dam (Mid Lake) | Savannah | Cartersville AMU | Lake Trend Monitoring | 34.763533 | -83.455817 | X |  | X |  |  |  |  |  | X |
| LK_03_17700 | Lake Oconee - Lick Creek Cove near Old Phoenix Rd. | Oconee | Augusta AMU | Lake Trend Monitoring | 33.403819 | -83.272422 | X |  | X |  |  |  |  |  |  |
| LK_03_520 | Lake Oconee At Highway 44, Oconee River Arm | Oconee | Augusta AMU | Lake Trend Monitoring | 33.431394 | -83.265734 | X |  | X |  |  |  |  |  | X |
| LK_03_525 | Lake Sinclair - Little River & Murder Creek Arm, U/S U.S. Hwy 441 | Oconee | Augusta AMU | Lake Trend Monitoring | 33.189 | -83.2953 | X |  | X |  |  |  |  |  | X |
| LK_03_526 | Lake Sinclair - 300 Meters U/S Dam (Dam Forebay) | Oconee | Augusta AMU | Lake Trend Monitoring | 33.142817 | -83.202617 | X |  | X |  |  |  |  |  | X |
| LK_03_530 | Lake Sinclair - Midlake, Oconee River Arm | Oconee | Augusta AMU | Lake Trend Monitoring | 33.1968 | -83.2742 | X |  | X |  |  |  |  |  | X |
| LK_03_538 | Lake Oconee 300 Meters U/S Wallace Dam (Dam Forebay) | Oconee | Augusta AMU | Lake Trend Monitoring | 33.351667 | -83.160833 | X |  | X |  |  |  |  |  | X |
| LK_03_545 | Lake Oconee - Richland Creek Arm | Oconee | Augusta AMU | Lake Trend Monitoring | 33.3947 | -83.1767 | X |  | X |  |  |  |  |  | X |
| LK_03_600 | Lake Sinclair- Little River/Oconee River Confluence | Oconee | Augusta AMU | Lake Trend Monitoring | 33.1875 | -83.275 | X |  | X |  |  |  |  |  | X |
| LK_04_893 | Lake Jackson at confluence of Alcovy River and Yellow/South River Branch | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.36314 | -83.86131 | X |  | X |  |  |  |  |  | X |
| LK_04_897 | Lake Jackson - Dam Forebay | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.322 | -83.8409 | X |  | X |  |  |  |  |  | X |
| LK_05_2076 | High Falls Lake - Midlake | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.1973 | -84.031 | X |  | X |  |  |  |  |  | X |
| LK_05_2078 | High Falls Lake - Dam Forebay | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.1799 | -84.0209 | X |  | X |  |  |  |  |  | X |
| LK_05_2131 | Lake Juliette - Midlake | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.0464 | -83.8106 | X |  | X |  |  |  |  |  | X |
| LK_05_2132 | Lake Juliette - Dam Forebay | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 33.0338 | -83.7572 | X |  | X |  |  |  |  |  | X |
| LK_05_2144 | Lake Tobesofkee - Midlake | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 32.8346 | -83.8161 | X |  | X |  |  |  |  |  | X |
| LK_05_2146 | Lake Tobesofkee - Dam Forebay | Ocmulgee | Atlanta AMU | Lake Trend Monitoring | 32.8215 | -83.7706 | X |  | X |  |  |  |  |  | X |
| LK_09_3199 | Banks Lake - Near Lakeland, GA | Suwannee | Tifton AMU | Lake Trend Monitoring | 31.026667 | -83.105555 | X |  | X |  |  |  |  |  | X |
| LK_11_3467 | Lake Blackshear - Midlake | Flint | Tifton AMU | Lake Trend Monitoring | 31.9665 | -83.9342 | X |  | X |  |  |  |  |  | X |
| LK_11_3520 | Lake Blackshear - Dam Forebay | Flint | Tifton AMU | Lake Trend Monitoring | 31.8479 | -83.9394 | X |  | X |  |  |  |  |  | X |
| LK_11_3534 | Flint River Reservoir - Midlake, Flint River Arm | Flint | Tifton AMU | Lake Trend Monitoring | 31.6085 | -84.119 | X |  | X |  |  |  |  |  | X |
| LK_11_3535 | Flint River Reservoir (Lake Worth) - Dam Forebay | Flint | Tifton AMU | Lake Trend Monitoring | 31.6033 | -84.1365 | X |  | X |  |  |  |  |  | X |
| LK_11_3551 | Lake Worth (original) - Above Hwy 91 Bridge / Diversion Dam (aka Lake Chehaw) | Flint | Tifton AMU | Lake Trend Monitoring | 31.6109 | -84.15 | X |  | X |  |  |  |  |  | X |
| LK_11_3569 | Lake Seminole - Flint River Arm at Spring Creek | Flint | Tifton AMU | Lake Trend Monitoring | 30.7627 | -84.8171 | X |  | X |  |  |  |  |  | X |
| LK_12_3913 | Lake Sidney Lanier - Little River Embayment, Betw M1WC & 3LR | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.355 | -83.8427 | X |  | X |  |  |  |  |  | X |
| LK_12_3995 | Lake Sidney Lanier at Boling Bridge (State Road 53) on Chestatee River | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.31235 | -83.950103 | X |  | X |  |  |  |  |  | X |
| LK_12_3998 | Lake Sidney Lanier at Lanier Bridge (State Road 53) on Chattahoochee River | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.32195 | -83.880171 | X |  | X |  |  |  |  |  | X |
| LK_12_4001 | Lake Sidney Lanier at Browns Bridge Road (State Road 369) | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.261666 | -83.950662 | X |  | X |  |  |  |  |  | X |
| LK_12_4005 | Lake Sidney Lanier - Flat Creek Embayment, 100' U/S M7FC | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.2587 | -83.9198 | X |  | X |  |  |  |  |  | X |
| LK_12_4007 | Lake Sidney Lanier - Balus Creek Embayment, 0.34m SE M6FC | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.2504 | -83.9244 | X |  | X |  |  |  |  |  | X |
| LK_12_4010 | Lake Sidney Lanier - Mud Crk Embayment, Betw Marina & Ramp | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.2333 | -83.9373 | X |  | X |  |  |  |  |  | X |
| LK_12_4012 | Lake Lanier upstream from Flowery Branch Confluence (Midlake) | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.200278 | -83.982869 | X |  | X |  |  |  |  |  | X |
| LK_12_4019 | Lake Sidney Lanier - Six Mile Creek Embayment, 300' E M9SM | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.2335 | -84.0287 | X |  | X |  |  |  |  |  | X |
| LK_12_4028 | Lake Sidney Lanier upstream of Buford Dam Forebay | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 34.162778 | -84.067108 | X |  | X |  |  |  |  |  | X |
| LK_12_4048 | West Point Lake at LaGrange Water Intake near LaGrange, Ga. | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 33.0783 | -85.110833 | X |  | X |  |  |  |  |  | X |
| LK_12_4060 | West Point Lake - Dam Forebay | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.9208 | -85.1834 | X |  | X |  |  |  |  |  | X |
| LK_12_4072 | Lake Harding - Midlake, Main Body | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.7379 | -85.1125 | X |  | X |  |  |  |  |  | X |
| LK_12_4074 | Lake Harding - Dam Forebay (aka Chatt. River US Bartletts Ferry Dam) | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.6633 | -85.090278 | X |  | X |  |  |  |  |  | X |
| LK_12_4078 | Goat Rock Lake - Dam Forebay | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.6112 | -85.0794 | X |  | X |  |  |  |  |  | X |
| LK_12_4080 | Lake Oliver - Dam Forebay | Chattahoochee | Atlanta AMU | Lake Trend Monitoring | 32.516 | -85.0009 | X |  | X |  |  |  |  |  | X |
| LK_12_4097 | Lake Walter F. George at U.S. Highway 82 (aka Chatt. River at Hwy 82) | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 31.891944 | -85.120833 | X |  | X |  |  |  |  |  | X |
| LK_12_4103 | Lake Walter F. George at Dam Forebay | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 31.629167 | -85.0725 | X |  | X |  |  |  |  |  | X |
| LK_12_4107 | Lake Andrews - Dam Forebay | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 31.2632 | -85.113 | X |  | X |  |  |  |  |  | X |
| LK_12_4113 | Lake Seminole - Chattahoochee Arm, Lower | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 30.7662 | -84.9201 | X |  | X | X | X |  |  |  | X |
| LK_12_4115 | Lake Seminole - Dam Forebay | Chattahoochee | Tifton AMU | Lake Trend Monitoring | 30.7115 | -84.8647 | X |  | X |  |  |  |  |  | X |
| LK_14_4494 | Lake Allatoona Upstream from Dam | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.160833 | -84.725845 | X |  | X |  |  |  |  |  | X |
| LK_14_4497 | Lake Allatoona at Allatoona Creek Upstream from Interstate 75 | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.085833 | -84.711389 | X |  | X |  |  |  |  |  | X |
| LK_14_4502 | Lake Allatoona at Etowah River upsteam from Sweetwater Creek | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.19 | -84.577778 | X |  | X |  |  |  |  |  | X |
| LK_14_4523 | Carters Lake (CR1) - Upper Lake, Coosawattee Arm | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.62087 | -84.6212 | X |  | X |  |  |  |  |  | X |
| LK_14_4524 | Carters Lake - Midlake (upstream from Woodring Branch) | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.6076 | -84.638 | X |  | X |  |  |  |  |  | X |
| LK_14_4553 | Lake Allatoona at Little River upstream from Highway 205 | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.158611 | -84.577222 | X |  | X |  |  |  |  |  | X |
| LK_14_4556 | Lake Allatoona downstream from Kellogg Creek | Coosa | Cartersville AMU | Lake Trend Monitoring | 34.138611 | -84.639167 | X |  | X |  |  |  |  |  | X |
| LK_15_4895 | Lake Chatuge LMP 12 at State Line (aka Hiawassee River) | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.983333 | -83.788611 | X |  | X |  |  |  |  |  | X |
| LK_15_4899 | Lake Nottely (LMP15A) at Reece Creek | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.91152 | -84.0506 | X |  | X |  |  |  |  |  | X |
| LK_15_4900 | Lake Nottely - Dam Forebay (aka Nottely River - Upstream From Nottley Dam) | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.957778 | -84.092222 | X |  | X |  |  |  |  |  | X |
| LK_15_4907 | Lake Blue Ridge (LMP18) - 300 Meter Upstream Of Dam | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.881667 | -84.28 | X |  | X |  |  |  |  |  | X |
| LK_15_4908 | Lake Blue Ridge (LMP18A) - 4 miles upsteam Dam | Tennessee | Cartersville AMU | Lake Trend Monitoring | 34.84017 | -84.2731 | X |  | X |  |  |  |  |  | X |
| SH_02_317 | Little Ogeechee River at Green Island | Ogeechee | Brunswick AMU | Estuary Trend Monitoring | 31.88823 | -81.08798 | X | X |  |  |  |  |  |  | X |
| SH_02_364 | St Catherines Sound at Medway River near Midway, GA | Ogeechee | Brunswick AMU | Estuary Trend Monitoring | 31.715469 | -81.156798 | X | X |  |  |  |  |  |  | X |
| SH_02_372 | Sapelo Sound at South Newport River near Barbour Island | Ogeechee | Brunswick AMU | Estuary Trend Monitoring | 31.554108 | -81.200361 | X | X |  |  |  |  |  |  | X |
| SH_02_374 | Sapelo River - Mouth of Broro River - 1.4 miles South of Shellman's Bluff | Ogeechee | Brunswick AMU | Estuary Trend Monitoring | 31.544861 | -81.316027 | X | X |  |  |  |  |  |  | X |
| SH_02_56 | Mouth of Wilmington River - Marker #19 Wassaw Sound | Ogeechee | Brunswick AMU | Estuary Trend Monitoring | 31.932416 | -80.977111 | X | X |  |  |  |  |  |  | X |
| SH_06_15212 | Doboy Sound | Altamaha | Brunswick AMU | Estuary Trend Monitoring | 31.394942 | -81.29435 | X | X |  |  |  |  |  |  | X |
| SH_06_2857 | Altamaha River - channel marker #201 off Wolf Island | Altamaha | Brunswick AMU | Estuary Trend Monitoring | 31.319166 | -81.325 | X | X |  |  |  |  |  |  | X |
| SH_07_15209 | St. Simons Sound | Satilla | Brunswick AMU | Estuary Trend Monitoring | 31.12568 | -81.411953 | X | X |  |  |  |  |  |  | X |
| SH_07_3008 | St. Andrews Sound at Satilla Riv near | Satilla | Brunswick AMU | Estuary Trend Monitoring | 30.983162 | -81.453238 | X | X |  |  |  |  |  |  | X |
| SH_07_3029 | Turtle River off Hermitage Island | Satilla | Brunswick AMU | Estuary Trend Monitoring | 31.220278 | -81.564167 | X |  |  |  |  |  |  |  |  |
| SH_07_3032 | Turtle River - Georgia Highway 303 | Satilla | Brunswick AMU | Estuary Trend Monitoring | 31.186944 | -81.531389 | X |  |  |  |  |  |  |  |  |
| SH_07_3035 | Brunswick Harbor (off East River) - 0.83 miles SW of Brunswick | Satilla | Brunswick AMU | Estuary Trend Monitoring | 31.143611 | -81.4975 | X |  |  |  |  |  |  |  |  |
| SH_07_3036 | Brunswick River - U.S. Highway 17 | Satilla | Brunswick AMU | Estuary Trend Monitoring | 31.1164 | -81.4858 | X |  |  |  | X |  |  | X |  |
| SH_07_3049 | Cumberland Sound at St. Marys Riv nr St Marys, GA | Satilla | Brunswick AMU | Estuary Trend Monitoring | 30.728073 | -81.489794 | X | X |  |  |  |  |  |  | X |
| RV_01_120 | Savannah River - US Highway 17 (Houlihan Bridge) | Savannah | Brunswick AMU | Trend Monitoring (USGS) | 32.165833 | -81.1539 | X |  | X | X |  |  |  |  |  |
| RV_01_123 | Pipe Makers Canal at US Hwy 21 at Savannah, GA | Savannah | Brunswick AMU | Targeted Monitoring (E coli) | 32.121296 | -81.16763 | X |  | X |  |  |  |  |  |  |
| RV_01_102 | Brushy Creek at State Road 80 nr Wrens, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 33.17698 | -82.3058 | X |  | X |  |  |  |  |  |  |
| RV_01_109 | Savannah River - Seaboard Coast Line Railway, North of Clyo | Savannah | Augusta AMU | Trend Monitoring (USGS) | 32.525 | -81.263978 | X |  | X | X |  |  |  |  |  |
| RV_01_115 | Ebenezer Creek at Long Bridge Rd (CR 307) near Stillwell, GA | Savannah | Brunswick AMU | Probabilistic Monitoring | 32.364583 | -81.23075 | X |  | X | X | X |  |  | X |  |
| RV_01_120 | Savannah River - US Highway 17 (Houlihan Bridge) | Savannah | Brunswick AMU | Trend Monitoring (USGS) | 32.165833 | -81.1539 | X |  | X | X |  |  |  |  |  |
| RV_01_123 | Pipe Makers Canal at US Hwy 21 at Savannah, GA | Savannah | Brunswick AMU | Targeted Monitoring (E coli) | 32.121296 | -81.16763 | X |  | X |  |  |  |  |  |  |
| RV_01_144 | Kettle Creek at Stone Ridge Rd | Savannah | Augusta AMU | Trend Monitoring | 33.683 | -82.857 | X |  | X | X | X | X | X | X |  |
| RV_01_147 | Reedy Creek at Campground Rd near Wrens, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 33.23787 | -82.29745 | X |  | X |  |  |  |  |  |  |
| RV_01_17491 | She Creek at Woods Rd near Clayton, GA | Savannah | Atlanta AMU | Targeted Monitoring (E coli) | 34.83938 | -83.33702 | X |  | X |  |  |  |  |  |  |
| RV_01_17495 | Scott Creek at Shadyside Drive near Clayton, GA | Savannah | Atlanta AMU | Targeted Monitoring (E coli) | 34.87713 | -83.40571 | X |  | X |  |  |  |  |  |  |
| RV_01_17496 | Saddle Gap Branch at Dugan Hill Rd near Clayton, GA | Savannah | Atlanta AMU | Targeted Monitoring (E coli) | 34.87788 | -83.39358 | X |  | X |  |  |  |  |  |  |
| RV_01_18226 | Furnace Creek at SR 98 near Ila, GA | Savannah | Augusta AMU | Probabilistic Monitoring | 34.166032 | -83.277784 | X |  | X | X | X |  |  | X |  |
| RV_01_18227 | Runs Branch at Clyo-Kildare Rd near Clyo, GA | Savannah | Augusta AMU | Probabilistic Monitoring | 32.55231 | -81.3838 | X |  | X | X | X |  |  | X |  |
| RV_01_2 | Panther Creek at Panther Creek Road near Toccoa, GA | Savannah | Atlanta AMU | Targeted Monitoring (E coli) | 34.677926 | -83.351194 | X |  | X |  |  |  |  |  |  |
| RV_01_204 | Chechero Creek at New Hope Church Rd | Savannah | Atlanta AMU | Targeted Monitoring (E coli) | 34.850288 | -83.359956 | X |  | X |  |  |  |  |  |  |
| RV_01_244 | Charlies Creek at Charlies Creek Rd East of Hiawassee, GA | Savannah | Atlanta AMU | Trend Monitoring (SEMN) | 34.95895 | -83.57158 | X |  |  | X | X | X | X | X |  |
| RV_01_248 | Coleman River at Coleman River Rd near Clayton, GA | Savannah | Atlanta AMU | Trend Monitoring (SEMN) | 34.95203324 | -83.51659881 | X |  |  | X | X | X | X | X |  |
| RV_01_256 | Jacks Branch at Early Street, Springfield, GA | Savannah | Brunswick AMU | Targeted Monitoring (E coli) | 32.379418 | -81.309695 | X |  | X |  |  |  |  |  |  |
| RV_01_66 | Chattooga River at U.S. Hwy 76 near Clayton, GA | Savannah | Atlanta AMU | Trend Monitoring (USGS) | 34.813983 | -83.306433 | X |  | X | X | X |  |  |  |  |
| RV_01_87 | Savannah R BL Spirit Cr nr Augusta, GA | Savannah | Augusta AMU | Trend Monitoring (USGS) | 33.3306 | -81.9153 | X |  | X | X |  |  |  |  |  |
| RV_01_97 | Brier Creek at State Road 17 near Thomson, GA | Savannah | Augusta AMU | Targeted Monitoring (E coli) | 33.367416 | -82.467166 | X |  | X |  |  |  |  |  |  |
| RV_02_15811 | Hardin Canal D/S Pooler near Pine Barren Road near Pooler, GA | Ogeechee | Brunswick AMU | Targeted Monitoring (Salinity) | 32.090547 | -81.24629 | X |  |  |  |  |  |  |  |  |
| RV_02_16130 | Mile Branch at Hilley Rd near Crawfordville, GA | Ogeechee | Augusta AMU | Targeted Monitoring (Req-NH3) | 33.52855 | -82.90088 | X |  |  | X |  |  |  |  |  |
| RV_02_16131 | Mile Branch at Alexander Street near Crawfordville, GA | Ogeechee | Augusta AMU | Targeted Monitoring (Req-NH3) | 33.54648 | -82.90958 | X |  |  | X |  |  |  |  |  |
| RV_02_16132 | Trib to Mile Branch off of Asbury Heights near Crawfordville, GA | Ogeechee | Augusta AMU | Targeted Monitoring (Req-NH3) | 33.54842 | -82.89857 | X |  |  | X |  |  |  |  |  |
| RV_02_16832 | Goldens Creek off of N. Norwood St near North Pond | Ogeechee | Augusta AMU | Targeted Monitoring (Req-NH3) | 33.41416 | -82.669915 | X |  |  | X |  |  |  |  |  |
| RV_02_16833 | Goldens Creek at SR 16 / Macon Hwy | Ogeechee | Augusta AMU | Targeted Monitoring (Req-NH3) | 33.399944 | -82.672836 | X |  |  | X |  |  |  |  |  |
| RV_02_17701 | North Fork Ogeechee Brooks Rd | Ogeechee | Augusta AMU | Trend Monitoring | 33.588444 | -83.00933 | X |  | X | X | X | X | X | X |  |
| RV_02_17761 | Unnamed Canal at Louis Mills Blvd near Savannah, GA | Ogeechee | Brunswick AMU | Targeted Monitoring (E coli) | 32.035416 | -81.156263 | X |  | X |  |  |  |  |  |  |
| RV_02_17779 | North Fork Ogeechee Buffalo Lick Rd | Ogeechee | Augusta AMU | Targeted Monitoring (E coli) | 33.60066 | -83.05917 | X |  | X |  |  |  |  |  |  |
| RV_02_18183 | Goldens Creek at US 278/ GA Hwy 12 | Ogeechee | Augusta AMU | Targeted Monitoring (Req-NH3) | 33.407581 | -82.67259 | X |  |  | X |  |  |  |  |  |
| RV_02_18213 | Hardin Canal at Driveway to Pooler WPCP (upstream of outfall) near Pooler, GA | Ogeechee | Brunswick AMU | Targeted Monitoring (Salinity) | 32.11 | -81.261944 | X |  |  |  |  |  |  |  |  |
| RV_02_18214 | Hardin Canal at South Rogers Street (downstream of outfall) near Pooler, GA | Ogeechee | Brunswick AMU | Targeted Monitoring (Salinity) | 32.099462 | -81.254585 | X |  |  |  |  |  |  |  |  |
| RV_02_18228 | Boggy Gut Creek at SR 78 near Wadley, GA | Ogeechee | Augusta AMU | Probabilistic Monitoring | 32.87475 | -82.38636 | X |  | X | X | X |  |  | X |  |
| RV_02_280 | Little Ogeechee River at Shoals Rd near Culverton, GA | Ogeechee | Augusta AMU | Targeted Monitoring (E coli) | 33.2572 | -82.85781 | X |  | X |  | X |  |  |  |  |
| RV_02_291 | Ogeechee River at U.S. Highway 25 near Millen, GA | Ogeechee | Augusta AMU | Probabilistic Monitoring | 32.78883 | -81.96053 | X |  | X | X | X |  |  | X |  |
| RV_02_297 | Nevills Creek at Bulloch County Road 578 near Rocky Ford, GA | Ogeechee | Brunswick AMU | Targeted Monitoring (E coli) | 32.601871 | -81.794224 | X |  | X |  |  |  |  |  |  |
| RV_02_298 | Ogeechee River - Georgia Highway 24 nr Oliver, GA | Ogeechee | Augusta AMU | Trend Monitoring (USGS) | 32.49475 | -81.555833 | X |  | X | X |  |  |  |  |  |
| RV_02_416 | Casey Canal at DeRenne Ave | Ogeechee | Brunswick AMU | Targeted Monitoring (E coli) | 32.02271 | -81.089176 | X |  | X |  |  |  |  |  |  |
| RV_02_462 | Mill Creek at Bulloch County Road 386 Old River Road near Brooklet, GA | Ogeechee | Brunswick AMU | Trend Monitoring | 32.440012 | -81.579074 | X |  | X | X | X | X | X | X |  |
| RV_03_16203 | Calls Creek at Hwy 441 near Athens, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.860761 | -83.424908 | X |  | X |  |  |  |  |  |  |
| RV_03_16781 | Cedar Creek at Union Hill Church Rd near Hillsboro, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.16179 | -83.54283 | X |  | X |  |  |  |  |  |  |
| RV_03_18205 | Trib to Hard Labor Creek at Dial Road, near Monroe GA | Oconee | Atlanta AMU | Probabilistic Monitoring | 33.764707 | -83.696496 | X |  | X | X | X |  |  |  |  |
| RV_03_18206 | Rice Creek at Newborn Rd U/S Rutledge WPCP near Rutledge, GA | Oconee | Atlanta AMU | Targeted Monitoring (NH3) | 33.618368 | -83.61281 | X |  | X |  |  |  |  |  |  |
| RV_03_18207 | Rice Creek at Newborn Rd D/S Rutledge WPCP near Rutledge,GA | Oconee | Atlanta AMU | Targeted Monitoring (NH3) | 33.61592 | -83.60544 | X |  | X |  |  |  |  |  |  |
| RV_03_18208 | Unnamed trib to Big Indian Creek I-20 at mile 108, upstream of DOT rest area outfall near Rutledge, GA | Oconee | Atlanta AMU | Targeted Monitoring (NH3) | 33.579934 | -83.582182 | X |  | X |  |  |  |  |  |  |
| RV_03_18209 | Unnamed trib to Big Indian Creek I-20 at mile 108, downstream of DOT rest area outfall near Rutledge, GA | Oconee | Atlanta AMU | Targeted Monitoring (NH3) | 33.579841 | -83.582204 | X |  | X |  |  |  |  |  |  |
| RV_03_18225 | Fishing Creek at Callaway Road near Woodville ,GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.685166 | -83.237094 | X |  | X |  |  |  |  |  |  |
| RV_03_493 | West Fork Trail Creek US 29 near Athens, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.9795 | -83.35142 | X |  | X |  |  |  |  |  |  |
| RV_03_494 | East Fork Trail Creek Tributary (Carver Branch) at Olympic Drive near Athens, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.9777 | -83.32788 | X |  | X |  |  |  |  |  |  |
| RV_03_502 | Oconee River at Barnett Shoals Road near Athens, GA | Oconee | Augusta AMU | Trend Monitoring (USGS) | 33.85561 | -83.32684 | X |  | X | X | X |  |  |  |  |
| RV_03_503 | Oconee River at Georgia Highway 15 near Penfield, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.72074 | -83.29568 | X |  | X |  |  |  |  |  |  |
| RV_03_5061 | Unnamed Secondary Tributary to Limestone Creek at State Road 56 near Mt Vernon, GA | Oconee | Brunswick AMU | Targeted Monitoring (E coli) | 32.171549 | -82.591193 | X |  | X |  |  |  |  |  |  |
| RV_03_5121 | Pittman Branch @ Brookhollow Way DS Mansfield WPCP | Oconee | Atlanta AMU | Targeted Monitoring (E coli, NH3), revisit | 33.505505 | -83.71768 | X |  | X |  |  |  |  |  |  |
| RV_03_513 | Apalachee River at SR 186 / Snows Mill Road near Bishop ,GA | Oconee | Atlanta AMU | Probabilistic Monitoring | 33.81781 | -83.505839 | X |  | X | X | X |  |  |  |  |
| RV_03_532 | Little Sugar Creek at Kingston Road (County Road 127) near Buckhead, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.48995 | -83.378917 | X |  | X |  |  |  |  |  |  |
| RV_03_554 | Big Indian Creek at Georgia Highway 83 near Madison, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.525556 | -83.524444 | X |  | X |  |  |  |  |  |  |
| RV_03_563 | Murder Creek at Hillsborough Road near Eatonton ,GA | Oconee | Atlanta AMU | Probabilistic Monitoring | 33.268189 | -83.497299 | X |  | X | X | X |  |  |  |  |
| RV_03_640 | Oconee River at Interstate Highway 16 near Dublin, GA | Oconee | Tifton AMU | Trend Monitoring (USGS) | 32.480367 | -82.858217 | X | X |  | X |  |  |  |  |  |
| RV_03_658 | Limestone Creek - N. Old River Road near Vidalia, GA | Oconee | Brunswick AMU | Targeted Monitoring (E coli) | 32.151654 | -32.601829 | X |  | X |  |  |  |  |  |  |
| RV_03_659 | Peterson Creek - CR 58 near Glenwood, GA | Oconee | Brunswick AMU | Targeted Monitoring (E coli) | 32.16258 | -82.645677 | X |  | X |  |  |  |  |  |  |
| RV_03_668 | Rose Creek at Antioch Church Road | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.76797 | -83.32364 | X |  | X |  |  |  |  |  |  |
| RV_03_720 | West Fork Trail Creek at Hull Rd near Athens, GA | Oconee | Augusta AMU | Targeted Monitoring (E coli) | 33.98978 | -83.35078 | X |  | X | X |  |  |  |  |  |
| RV_03_730 | Big Creek Tributary at US Hwy 319 | Oconee | Tifton AMU | Targeted Monitoring (Req-Grease) | 32.556819 | -82.858865 | X |  |  |  |  |  |  |  |  |
| RV_03_739 | Noketchee Creek at Helican Springs Rd near Athens, GA | Oconee | Augusta AMU | Probabilistic Monitoring | 34.03456 | -83.33896 | X |  | X | X | X |  |  | X |  |
| RV_03_786 | Big Sandy Creek at Sandy Creek Road near Bostwick, GA | Oconee | Atlanta AMU | Targeted Monitoring (E coli) | 33.71775 | -83.559583 | X |  | X |  |  |  |  |  |  |
| RV_04_848 | South River at Georgia Hwy 81 @ Snapping Shoals | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 33.45265 | -83.927083 | X |  | X | X | X |  |  |  |  |
| RV_04_876 | Yellow River at Georgia Hwy 212 near Stewart, GA | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 33.454267 | -83.881333 | X |  | X | X | X |  |  |  |  |
| RV_04_888 | Alcovy River at Newton Factory Bridge Road near Stewart | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 33.4494 | -83.8283 | X |  | X | X | X |  |  |  |  |
| RV_04_892 | Tussahaw Creek at Fincherville Road near Jackson, GA | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 33.378867 | -83.9634 | X |  | X | X | X |  |  |  |  |
| RV_05_15800 | Jordan Creek at Porter Drive near Cochran, GA | Ocmulgee | Tifton AMU | Targeted Monitoring (Req - Wetland) | 32.393033 | -83.362516 | X |  |  | X |  |  |  |  |  |
| RV_05_17696 | Walnut Creek at Stagecoach Rd | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 32.895996 | -83.608888 | X |  | X |  |  |  |  |  |  |
| RV_05_17991 | Trib to Jordan Creek U/S WPCP near W. Railroad St. near Cochran, GA | Ocmulgee | Tifton AMU | Targeted Monitoring (Req - Wetland) | 32.383158 | -83.373371 | X |  |  | X |  |  |  |  |  |
| RV_05_18204 | Rum Creek at Dames Ferry Rd near Macon, GA | Ocmulgee | Atlanta AMU | Probabilistic Monitoring | 33.022972 | -83.74786 | X |  | X | X | X |  |  |  |  |
| RV_05_18216 | Little Creek at S. Union Church Road near Scotland, GA | Ocmulgee | Brunswick AMU | Probabilistic Monitoring | 32.086392 | -82.801332 | X |  | X | X | X |  |  |  |  |
| RV_05_18221 | Ocmulgee River at Halfmoon Rd./Half Moon Landing near Abbeville, Ga | Ocmulgee | Tifton AMU | Targeted Monitoring (Req-NH3, pH, and DO) | 32.001131 | -83.295561 | X | X |  |  |  |  |  |  |  |
| RV_05_2073 | Towaliga River at SR 36 near Jackson, GA | Ocmulgee | Atlanta AMU | Targeted Monitoring (pH) | 33.216389 | -84.049722 | X |  |  |  |  |  |  |  |  |
| RV_05_2165 | Ocmulgee River at New Macon Water Intake | Ocmulgee | Atlanta AMU | Trend Monitoring (USGS) | 32.899247 | -83.664064 | X |  | X | X | X |  |  |  |  |
| RV_05_2170 | Walnut Creek at McKay Rd | Ocmulgee | Atlanta AMU | Targeted Monitoring (E coli) | 32.958611 | -83.62 | X |  | X |  |  |  |  |  |  |
| RV_05_2203 | Ocmulgee River at Hawkinsville, GA | Ocmulgee | Tifton AMU | Trend Monitoring (USGS) | 32.281759 | -83.462773 | X | X |  | X |  |  |  |  |  |
| RV_05_2215 | Ocmulgee River - U.S. Highway 280 | Ocmulgee | Tifton AMU | Targeted Monitoring (Req-NH3, pH, and DO) | 31.996389 | -83.278611 | X | X |  |  |  |  |  |  |  |
| RV_05_2223 | Ocmulgee River - US Highway 341 at Lumber City, GA | Ocmulgee | Brunswick AMU | Trend Monitoring (USGS) | 31.919933 | -82.674267 | X |  | X | X |  |  |  |  |  |
| RV_06_18215 | Trib to Rocky Creek at US Hwy 280 near Higgins, GA | Altamaha | Brunswick AMU | Probabilistic Monitoring | 32.204901 | -82.482472 | X |  | X | X | X |  |  |  |  |
| RV_06_2846 | Altamaha River - 6 Miles Downstream From Doctortown | Altamaha | Brunswick AMU | Trend Monitoring (USGS) | 31.6233 | -81.7653 | X |  | X | X |  |  |  |  |  |
| RV_07_18212 | Satilla River Downstream of Warners Landing Boat Ramp near Horetense, GA *jon boat site* | Satilla | Brunswick AMU | Probabilistic Monitoring | 31.240185 | -81.867542 | X |  | X | X | X |  |  | X |  |
| RV_07_18217 | Satilla River at US Highway 1 near Dixie Union, GA | Satilla | Brunswick AMU | Probabilistic Monitoring | 31.302138 | -82.453567 | X |  | X | X | X |  |  | X |  |
| RV_07_18229 | Hatcher's Branch at Paxton Road near Folkston, GA | Satilla | Brunswick AMU | Targeted Monitoring (319 Request) | 30.839179 | -82.035086 | X |  | X |  |  |  |  |  |  |
| RV_07_2975 | Hurricane Creek at CR 552 near Nichols, GA | Satilla | Brunswick AMU | Targeted Monitoring (E coli) | 31.5087 | -82.63491 | X |  | X |  |  |  |  |  |  |
| RV_07_2977 | Dry Creek at CR 552 (Flying Hawk Dr) near Nichols, GA | Satilla | Brunswick AMU | Targeted Monitoring (E coli) | 31.48423 | -82.6314 | X |  | X |  |  |  |  |  |  |
| RV_07_2978 | Hog Creek at State Road 32 near Nichols, GA | Satilla | Brunswick AMU | Targeted Monitoring (E coli) | 31.520833 | -82.62556 | X |  | X |  |  |  |  |  |  |
| RV_07_2986 | Satilla River Georgia Highways 15 and 121 | Satilla | Brunswick AMU | Trend Monitoring (USGS), lead | 31.2167 | -82.1625 | X |  | X | X | X |  |  |  |  |
| RV_07_2995 | Hurricane Creek - U.S. Highway 1 near Alma, GA | Satilla | Brunswick AMU | Targeted Monitoring (E coli) | 31.566667 | -82.463889 | X |  | X |  |  |  |  |  |  |
| RV_07_3099 | Mill Creek at High Bluff Rock Rd nr Waycross, GA | Satilla | Brunswick AMU | Trend Monitoring | 31.189994 | -82.202803 | X |  | X | X | X | X | X | X |  |
| RV_08_16769 | Clay Branch @ Okefenokee Drive | St. Marys | Brunswick AMU | Targeted Monitoring (E coli) | 30.840673 | -82.014125 | X |  | X |  |  |  |  |  |  |
| RV_08_16770 | Long Branch @ Sardis/Spanish Creek Road | St. Marys | Brunswick AMU | Targeted Monitoring (E coli) | 30.813087 | -82.069331 | X |  | X |  |  |  |  |  |  |
| RV_09_16319 | Trib to Mill Creek at Amaco Rd near Rochelle, GA | Suwannee | Tifton AMU | Targeted Monitoring (Req-NH3, pH, and DO) | 31.955212 | -83.477801 | X |  |  |  |  |  |  |  |  |
| RV_09_16320 | Trib to Mill Creek at SR30 near Rochelle, GA | Suwannee | Tifton AMU | Targeted Monitoring (Req-NH3, pH, and DO) | 31.949891 | -83.480221 | X |  |  |  |  |  |  |  |  |
| RV_09_16321 | Mill Creek at SR233 near Rochelle, GA | Suwannee | Tifton AMU | Targeted Monitoring (Req-NH3, pH, and DO) | 31.932354 | -83.455066 | X |  |  |  |  |  |  |  |  |
| RV_09_16467 | Mill Creek at SR 215 near Rochelle, GA | Suwannee | Tifton AMU | Targeted Monitoring (Req-NH3, pH, and DO) | 31.94556 | -83.44503 | X |  |  |  |  |  |  |  |  |
| RV_09_18223 | Unnamed Trib to Warrior Creek at SR 112 near Sylvester, GA | Suwannee | Tifton AMU | Probabilistic Monitoring | 31.54931 | -83.8184 | X | X |  | X | X |  |  |  |  |
| RV_09_3181 | Suwannee River at US Highway 441 near Fargo, GA | Suwannee | Brunswick AMU | Trend Monitoring (USGS) | 30.6806 | -82.5606 | X |  | X | X |  |  |  |  |  |
| RV_09_3201 | Mud Creek at County Road 112 (Vann Road) near Valdosta, Ga | Suwannee | Tifton AMU | Targeted Monitoring (E coli) | 30.777778 | -83.18 | X | X |  |  |  |  |  |  |  |
| RV_09_3236 | Withlacoochee River at Clyattsville-Nankin Road near Clyattsville, GA | Suwannee | Tifton AMU | Trend Monitoring (USGS) | 30.674722 | -83.394722 | X | X |  | X |  |  |  |  |  |
| RV_09_3238 | Jumping Gully Creek - Jumping Gully Road SW of Lake Park, GA | Suwannee | Tifton AMU | Targeted Monitoring (E coli) | 30.634167 | -83.265833 | X | X |  |  |  |  |  |  |  |
| RV_09_3246 | Ty Ty Creek at U.S. Highway 82 at Ty Ty, GA | Suwannee | Tifton AMU | Targeted Monitoring (E coli) | 31.472778 | -83.663056 | X | X |  |  |  |  |  |  |  |
| RV_09_3355 | Trib to Withlacoochee River @ Clyattville Nankin Rd near Adel, GA | Suwannee | Tifton AMU | Targeted Monitoring (E coli) | 30.681843 | -83.379569 | X | X |  |  |  |  |  |  |  |
| RV_10_3384 | Tired Creek at County Road 151 near Reno, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.763611 | -84.229444 | X |  |  | X |  |  |  |  |  |
| RV_10_3386 | Ochlockonee River @ Hadley Ferry Rd. nr Calvary, GA | Ochlockonee | Tifton AMU | Trend Monitoring (USGS) | 30.731717 | -84.235533 | X | X |  | X |  |  |  |  |  |
| RV_10_3389 | Attapulgus Creek at U.S. Hwy 27 near Attapulgus, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.732778 | -84.453611 | X |  |  | X |  |  |  |  |  |
| RV_10_3390 | Swamp Creek at US Hwy 27 near Attapulgus, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.719444 | -84.411389 | X |  |  | X |  |  |  |  |  |
| RV_10_3422 | Little Attapulgus Creek at Faceville-Attapulgus Rd. near Attapulgus, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.750046 | -84.501333 | X |  |  | X |  |  |  |  |  |
| RV_10_3423 | Little Attapulgus Creek at State Rd 241 near Attapulgus, GA | Ochlockonee | Tifton AMU | Targeted Monitoring (Florida Stateline) | 30.718056 | -84.49 | X |  |  | X |  |  |  |  |  |
| RV_11_16371 | Sheep Rock Hollow at Cove Rd near Woodbury, GA | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 32.92279 | -84.53521 | X |  | X |  |  |  |  |  |  |
| RV_11_17559 | Trib to Perry Creek at Hwy 62 near Arlington, GA | Flint | Tifton AMU | Targeted Monitoring (Req-NH3) | 31.42638 | -84.74006 | X |  |  |  |  |  |  |  |  |
| RV_11_17560 | Trib to Perry Creek at Azalea St near Arlington, GA | Flint | Tifton AMU | Targeted Monitoring (Req-NH3) | 31.433889 | -84.727689 | X |  |  |  |  |  |  |  |  |
| RV_11_17572 | Trib to Mill Creek at Jewel Crowe Rd near Leesburg, GA | Flint | Tifton AMU | Targeted Monitoring (E coli) | 31.623937 | -83.879562 | X | X |  |  |  |  |  |  |  |
| RV_11_17657 | Po Joe Branch at Lamar Rd near Americus, Ga | Flint | Tifton AMU | Targeted Monitoring (E coli) | 32.0263 | -84.09152 | X | X |  |  |  |  |  |  |  |
| RV_11_17790 | Shoal Creek at Lower Fayetteville Rd near Peachtree City, GA | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 33.39425 | -84.63013 | X |  | X |  |  |  |  |  |  |
| RV_11_18016 | Lazer Creek at GA Hwy 36 near Woodland, GA | Flint | Atlanta AMU | Targeted Monitoring (E coli, lead) | 32.7597 | -84.5648 | X |  | X |  |  |  |  |  |  |
| RV_11_18220 | Slow Run Creek at GA Hwy 224 near Montezuma, GA | Flint | Tifton AMU | Targeted Monitoring (Request) | 32.36633 | -83.90658 | X | X |  | X |  |  |  |  |  |
| RV_11_18222 | Unnamed Trib to Mill Creek at Fowler Rd near Albany, GA | Flint | Tifton AMU | Probabilistic Monitoring | 31.63688 | -83.9164 | X | X |  | X | X |  |  |  |  |
| RV_11_18224 | Unnamed Trib to Little Pachitla Creek at SR 37 near Edison, GA | Flint | Tifton AMU | Probabilistic Monitoring | 31.5753 | -84.7792 | X | X |  | X | X |  |  |  |  |
| RV_11_18230 | Big Lazer Creek @ Big Lazer Creek WMA boat ramp near Thomaston, GA | Flint | Atlanta AMU | Probabilistic Monitoring | 32.807854 | -84.404274 | X |  | X | X | X |  |  |  |  |
| RV_11_3458 | Pennahatchee Creek at CR100(Slosheye Trl) West of Vienna, GA | Flint | Tifton AMU | Targeted Monitoring (E coli) | 32.095723 | -83.883787 | X | X |  |  |  |  |  |  |  |
| RV_11_3485 | Flint River at State Road 92 near Griffin, GA | Flint | Atlanta AMU | Trend Monitoring (USGS) | 33.3089 | -84.393056 | X |  | X | X | X |  |  |  |  |
| RV_11_3488 | Flat Creek at Georgia Highway 74, South Of Peachtree City | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 33.341111 | -84.538889 | X |  | X |  |  |  |  |  |  |
| RV_11_3489 | Line Creek At Georgia Highway 85 Near Senoia | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 33.319444 | -84.523611 | X |  | X |  |  |  |  |  |  |
| RV_11_3492 | White Oak Creek At Goodwyn Road | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 33.343333 | -84.714167 | X |  | X |  |  |  |  |  |  |
| RV_11_3503 | Bell Creek at Gordon School Road near Lincoln Park, GA | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 32.838056 | -84.358889 | X |  | X | X | X |  |  |  |  |
| RV_11_3507 | Flint River at State Roads 26/49 near Montezuma, GA | Flint | Tifton AMU | Trend Monitoring (Additional data) | 32.2981 | -84.043889 | X | X |  | X |  |  |  |  |  |
| RV_11_3511 | Flint River at SR 26 near Montezuma | Flint | Tifton AMU | Trend Monitoring (USGS) | 32.29295 | -84.044067 | X | X |  | X |  |  |  |  |  |
| RV_11_3513 | Turkey Creek - Georgia Highway 90 At Byromville | Flint | Tifton AMU | Probabilistic Monitoring | 32.195556 | -83.900833 | X | X |  | X | X |  |  |  |  |
| RV_11_3553 | Flint River at State Road 234 near Albany, GA | Flint | Tifton AMU | Trend Monitoring (USGS) | 31.5524 | -84.1463 | X | X |  | X |  |  |  |  |  |
| RV_11_3558 | Flint River at State Road 37 at Newton, GA | Flint | Tifton AMU | Trend Monitoring (USGS) | 31.309444 | -84.335 | X | X |  | X |  |  |  |  |  |
| RV_11_3563 | Flint River at U.S. Highway 27-B near Bainbridge, GA | Flint | Tifton AMU | Trend Monitoring (USGS) | 30.910946 | -84.580514 | X | X |  | X |  |  |  |  |  |
| RV_11_3574 | Pachitla Creek at State Road 37 near Edison, GA | Flint | Tifton AMU | Targeted Monitoring (E coli) | 31.554722 | -84.678611 | X | X |  |  |  |  |  |  |  |
| RV_11_3592 | Tributary to Potato Creek at Rocky Botton Rd nr Thomaston, GA | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 32.93523 | -84.28026 | X |  | X |  |  |  |  |  |  |
| RV_11_3627 | Turkey Creek at Turkey Creek Rd | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 33.343007 | -84.743763 | X |  | X |  |  |  |  |  |  |
| RV_11_3705 | Flint River at North Bridge Rd | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 33.41574 | -84.384485 | X |  | X |  |  |  |  |  |  |
| RV_11_3789 | Flint River at Sprewell Bluff Sprewell Bluff State Park | Flint | Atlanta AMU | Trend Monitoring | 32.855988 | -84.476812 | X |  | X | X | X | X | X |  |  |
| RV_11_3793 | Gum Creek at U.S. Highway 280 at Coney, GA | Flint | Tifton AMU | Targeted Monitoring (E coli) | 31.961111 | -83.883889 | X | X |  |  |  |  |  |  |  |
| RV_11_3800 | Lazar Creek at SR116 nr Shiloh, GA | Flint | Atlanta AMU | Targeted Monitoring (E coli) | 32.790847 | -84.606004 | X |  | X |  |  |  |  |  |  |
| RV_11_3804 | Lime Creek at Springhill Church Road east of Americus, GA | Flint | Tifton AMU | Trend Monitoring | 32.035 | -83.9925 | X | X |  | X | X |  |  |  |  |
| RV_11_3807 | Little Ichawaynochaway Creek at CR 3 near Shellman, Ga | Flint | Tifton AMU | Trend Monitoring | 31.803532 | -84.640013 | X | X |  | X | X |  |  |  |  |
| RV_11_3819 | Spring Creek @ SR90 at SR 90 nr Montezuma, GA | Flint | Tifton AMU | Targeted Monitoring (E coli) | 32.284784 | -84.007555 | X | X |  |  |  |  |  |  |  |
| RV_11_5101 | Branch Creek at Hartford St near Edison, GA | Flint | Tifton AMU | Targeted Monitoring (E coli) | 31.562284 | -84.718245 | X | X |  |  |  |  |  |  |  |
| RV_11_5107 | Mill Creek at GA Hwy 49 near Oglethorpe, GA | Flint | Tifton AMU | Targeted Monitoring (E coli) | 32.295999 | -84.052 | X | X |  |  |  |  |  |  |  |
| RV_12_15945 | Buttermilk Creek at Maxham Road near Austell, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 33.804837 | -84.619759 | X |  | X |  |  |  |  |  |  |
| RV_12_16354 | House Creek at Monument Rd | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.82465 | -85.04544 | X |  | X |  |  |  |  |  |  |
| RV_12_16370 | Sand Creek at Yates Rd near West Point, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.82918 | -85.09608 | X |  | X |  |  |  |  |  |  |
| RV_12_16527 | Flatrock Creek at Macon Rd near Columbus, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.519 | -84.878061 | X |  | X |  |  |  |  |  |  |
| RV_12_16773 | Trib to Mountain Creek at Callaway Gardens near Pine Mountain, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.828 | -84.861 | X |  | X |  |  |  |  |  |  |
| RV_12_17314 | White Sulfur Creek at Hubert Russell Rd near Greenville, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.92034 | -84.81318 | X |  | X |  |  |  |  |  |  |
| RV_12_17315 | Crawford Creek at Perry Mill Rd near Lagrange, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.93522 | -84.88933 | X |  | X |  |  |  |  |  |  |
| RV_12_17578 | Sweetwater Creek at Blairs Bridge Rd near Lithia Springs, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.77454 | -84.61455 | X |  | X | X | X | X | X |  |  |
| RV_12_18046 | Chattahoochee River at the 14th St Pedestrian Bridge in Columbus, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (pH) | 32.472642 | -84.996497 | X |  |  |  |  |  |  |  |  |
| RV_12_18103 | Big Creek at Grimes Bridge Rd (Roswell, GA) | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 34.018996 | -84.345133 | X |  | X | X | X | X | X |  |  |
| RV_12_18198 | Trib to Crawford Creek at Leisure Cir near Pine Mountain, GA | Chattahoochee | Atlanta AMU | Probabilistic Monitoring | 32.91256 | -84.871975 | X |  | X | X | X |  |  |  |  |
| RV_12_18199 | Baker Creek at Waverly Hall Rd near Geneva, GA | Chattahoochee | Atlanta AMU | Probabilistic Monitoring | 32.564479 | -84.666498 | X |  | X | X | X |  |  |  |  |
| RV_12_18200 | Tar River at Macon Rd near Geneva, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.54992 | -84.694003 | X |  | X |  |  |  |  |  |  |
| RV_12_18201 | Dozier Creek at Macon Rd near Columbus, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.54288 | -84.81583 | X |  | X |  |  |  |  |  |  |
| RV_12_18202 | Roaring Branch at Whitlesey Dr near Columbus, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.5397 | -84.96215 | X |  | X |  |  |  |  |  |  |
| RV_12_18203 | Cooper Creek at Macon Road near Columbus, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.4911 | -84.92086 | X |  | X |  |  |  |  |  |  |
| RV_12_18210 | Slaughterhouse Creek at Hollywood Ave near Gainesville, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.32068 | -83.8415 | X |  | X |  |  |  |  |  |  |
| RV_12_18211 | Tributary to West Fork Little River at Perry Parks Rd near Clermont, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.499 | -83.8323 | X |  | X |  |  |  |  |  |  |
| RV_12_18218 | Sweetwater Creek at Friendship Church Rd near Villa Rica, GA | Chattahoochee | Cartersville AMU | Probabilistic Monitoring | 33.779524 | -84.85867 | X |  | X | X | X |  |  |  |  |
| RV_12_3841 | Chattahoochee River at McGinnis Ferry Road | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 34.050556 | -84.097701 | X |  | X |  | X |  |  |  |  |
| RV_12_3854 | Johns Creek at Old Alabama Road near Alpharetta, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.011111 | -84.219722 | X |  | X |  |  |  |  |  |  |
| RV_12_3859 | Chattahoochee River - DeKalb County Water Intake | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.9731 | -84.2631 | X |  | X |  |  |  |  |  |  |
| RV_12_3870 | Chattahoochee River at Cobb County Water Intake near Roswell, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.9443 | -84.405 | X |  | X |  |  |  |  |  |  |
| RV_12_3891 | Chattahoochee River - Atlanta Water Intake | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.8278 | -84.455 | X |  | X |  | X |  |  |  |  |
| RV_12_3902 | Chattahoochee River at Belton Bridge Road near Lula, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 34.445145 | -83.68423 | X |  | X | X | X |  |  |  |  |
| RV_12_3917 | Tesnatee Creek at County Road 200 near Cleveland, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.583333 | -83.8225 | X |  | X |  |  |  |  |  |  |
| RV_12_3922 | Chestatee River at GA Hwy 52 nr Dahlonega, GA | Chattahoochee | Cartersville AMU | Targeted Monitoring (E coli) | 34.528056 | -83.939722 | X |  | X |  |  |  |  |  |  |
| RV_12_3925 | Chestatee River at GA 400, near Dahlonega, GA | Chattahoochee | Cartersville AMU | Trend Monitoring (USGS) | 34.466667 | -83.968889 | X |  | X | X | X |  |  |  |  |
| RV_12_3934 | Chattahoochee River at Bankhead Highway | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.795278 | -84.507778 | X |  | X |  | X |  |  |  |  |
| RV_12_3936 | Sandy Creek at Bolton Road near Atlanta, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 33.779267 | -84.499333 | X |  | X |  |  |  |  |  |  |
| RV_12_3950 | Pea Creek At Highway 70 | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 33.626389 | -84.724167 | X |  | X |  |  |  |  |  |  |
| RV_12_3960 | Chattahoochee River at Capps Ferry Road near Rico, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.5778 | -84.808611 | X |  | X |  |  |  |  |  |  |
| RV_12_3974 | Centralhatchee Creek at U.S. Highway 27 near Franklin, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 33.311111 | -85.104444 | X |  | X |  |  |  |  |  |  |
| RV_12_4003 | Flat Creek at McEver Road near Gainesville, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 34.265833 | -83.885 | X |  | X | X | X |  |  |  |  |
| RV_12_4039 | New River at State Road 100 near Corinth, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 33.235278 | -84.987778 | X |  | X | X | X |  |  |  |  |
| RV_12_4041 | Chattahoochee River at GA 100 at Franklin, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 33.278084 | -85.100936 | X |  | X | X | X |  |  |  |  |
| RV_12_4049 | Yellow Jacket Creek at Hammet Road near Hogansville, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (USGS) | 33.139167 | -84.975278 | X |  | X | X | X |  |  |  |  |
| RV_12_4075 | Mountain Oak Creek at State Road 103 near Hamilton, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 32.741111 | -85.068889 | X |  | X |  |  |  |  |  |  |
| RV_12_4094 | Chattahoochee River at Spur 39 near Omaha, Ga. | Chattahoochee | Tifton AMU | Trend Monitoring (USGS) | 32.1436 | -85.045278 | X | X |  | X |  |  |  |  |  |
| RV_12_4101 | Pataula Creek at State Road 50 near Georgetown, GA | Chattahoochee | Tifton AMU | Targeted Monitoring (E coli) | 31.818333 | -84.973889 | X | X |  |  |  |  |  |  |  |
| RV_12_4110 | Chattahoochee River at State Road 91 near Steam Mill, GA | Chattahoochee | Tifton AMU | Trend Monitoring (USGS) | 30.9775 | -85.005278 | X | X |  | X |  |  |  |  |  |
| RV_12_4123 | Hillabahatchee Creek at CR 210 near Frolona, GA | Chattahoochee | Atlanta AMU | Trend Monitoring | 33.311218 | -85.187675 | X |  | X | X | X | X | X | X |  |
| RV_12_4129 | Flowery Branch d/s from Mulberry St | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.179368 | -83.928094 | X |  | X |  |  |  |  |  |  |
| RV_12_4132 | Level Creek at Suwanee Dam Rd | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.096411 | -84.078158 | X |  | X |  |  |  |  |  |  |
| RV_12_4137 | Trib of Lake Lanier d/s of Pearl Nix Pkwy | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 34.298246 | -83.845248 | X |  | X |  |  |  |  |  |  |
| RV_12_4225 | Brush Creek at Bevis Rd near Franklin, GA | Chattahoochee | Atlanta AMU | Targeted Monitoring (E coli) | 33.201865 | -85.116664 | X |  | X |  |  |  |  |  |  |
| RV_12_4292 | Dicks Creek near Waters Creek, Neels Gap, GA | Chattahoochee | Cartersville AMU | Trend Monitoring (USGS) | 34.6797 | -83.937222 | X |  | X | X | X |  |  |  |  |
| RV_12_4293 | Drag Nasty Creek nr SR39, Georgetown, GA | Chattahoochee | Tifton AMU | Targeted Monitoring (E coli) | 31.768822 | -85.081764 | X | X |  |  |  |  |  |  |  |
| RV_12_4316 | Peachtree Creek at Northside Dr in Atlanta, GA | Chattahoochee | Atlanta AMU | Trend Monitoring (AWW) | 33.8194 | -84.407778 | X |  | X | X | X | X | X |  |  |
| RV_13_18219 | Kelley Creek at Beaver Run Rd near Tallapoosa, GA | Tallapoosa | Cartersville AMU | Probabilistic Monitoring | 33.669266 | -85.333465 | X |  | X |  |  |  |  |  |  |
| RV_13_4345 | Buck Creek at SR 16 nr Carrollton, GA | Tallapoosa | Cartersville AMU | Targeted Monitoring (E coli) | 33.592379 | -85.129296 | X |  | X |  |  |  |  |  |  |
| RV_13_4349 | Little Tallapoosa River near Bowdon, GA | Tallapoosa | Cartersville AMU | Trend Monitoring (USGS) | 33.4927778 | -85.279167 | X |  | X | X | X |  |  |  |  |
| RV_13_4353 | Tallapoosa River near Talllapoosa, GA | Tallapoosa | Cartersville AMU | Trend Monitoring (USGS) | 33.740833 | -85.336389 | X |  | X | X | X |  |  |  |  |
| RV_13_4393 | Mud Branch at North Van Wert Rd | Tallapoosa | Cartersville AMU | Targeted Monitoring (E coli) | 33.741289 | -84.960863 | X |  | X |  |  |  |  |  |  |
| RV_13_4402 | Little Tallapoosa River at Muses Bridge Rd nr Carrollton, GA | Tallapoosa | Cartersville AMU | Targeted Monitoring (E coli) | 33.648376 | -85.031462 | X |  | X |  |  |  |  |  |  |
| RV_14_15870 | Pinhook Creek at Hwy 411 | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.467173 | -84.700669 | X |  |  |  |  |  |  |  |  |
| RV_14_17477 | Ninetynine Branch at Irwin Mill Rd near Fairmount, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.42014 | -84.67966 | X |  |  |  |  |  |  |  |  |
| RV_14_17806 | Moore Creek at Ammons Dr nr Waleska, GA | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.32878 | -84.53471 | X |  | X |  |  |  |  |  |  |
| RV_14_18145 | Jacks Creek at Farmville Road | Coosa | Cartersville AMU | Targeted Monitoring (Revisit) | 34.45307 | -84.84482 | X |  | X | X | X |  |  |  |  |
| RV_14_4416 | Dry Creek at Pleasant Hill Road nr Redbud, GA | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.551944 | -84.779167 | X |  | X |  |  |  |  |  |  |
| RV_14_4438 | Conasauga River at US 76, near Dalton, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.783 | -84.873014 | X |  | X | X | X |  |  |  |  |
| RV_14_4460 | Conasauga River at Tilton, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.6667 | -84.9283 | X |  | X | X | X |  |  |  |  |
| RV_14_4466 | Swamp Creek @ Redwine Cove Rd | Coosa | Cartersville AMU | Probabilistic Monitoring | 34.64943 | -85.01358 | X |  | X | X | X |  |  |  |  |
| RV_14_4476 | Ellijay River at SR 52 (River Street) near Ellijay, GA | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.692702 | -84.479132 | X |  | X |  |  |  |  |  |  |
| RV_14_4518 | Mountaintown Creek at GA 282, near Ellijay, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.703378 | -84.539793 | X |  | X | X | X |  |  |  |  |
| RV_14_4520 | Coosawattee River Near Ellijay, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.6717 | -84.500164 | X |  | X | X | X |  |  |  |  |
| RV_14_4534 | Oostanaula River at Rome, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.2703 | -85.1733 | X |  | X | X | X |  |  |  |  |
| RV_14_4549 | Etowah River at Canton, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.239722 | -84.494423 | X |  | X | X | X |  |  |  |  |
| RV_14_4550 | Shoal Creek at GA 108, near Waleska, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.263333 | -84.595556 | X |  | X | X | X |  |  |  |  |
| RV_14_4555 | Little River at GA 5, near Woodstock, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.1222 | -84.504254 | X |  | X | X | X |  |  |  |  |
| RV_14_4586 | Etowah River at Hardin Bridge Road, near Euharlee, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.188859 | -84.925104 | X |  | X | X | X |  |  |  |  |
| RV_14_4608 | Beech Creek at Mays Bridge Rd SW nr Rome, GA | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.233157 | -85.293299 | X |  | X |  |  |  |  |  |  |
| RV_14_4610 | Kings Creek at SR 20 nr Coosa, GA | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.257222 | -85.426944 | X |  | X |  |  |  |  |  |  |
| RV_14_4622 | Coosa River at State line, AL/GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.1983 | -85.443928 | X |  | X | X | X |  |  |  |  |
| RV_14_4640 | Chattooga River at Chattoogaville, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.3356 | -85.4453 | X |  | X | X | X |  |  |  |  |
| RV_14_4735 | Cedar Creek on Littlefield Rd | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.393946 | -84.835649 | X |  |  |  |  |  |  |  |  |
| RV_14_4743 | Trib of Pettit Creek at Jones Mill Rd | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.194459 | -84.804813 | X |  | X |  |  |  |  |  |  |
| RV_14_4748 | Little Pine Log Creek at Perry Rd | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.39445 | -84.7845 | X |  |  |  |  |  |  |  |  |
| RV_14_4754 | Sugar Hill Creek at Knuckelsville Rd | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.360448 | -84.711863 | X |  |  |  |  |  |  |  |  |
| RV_14_4829 | Dykes Crk at Dykes Crk Xing nr Rome, GA | Coosa | Cartersville AMU | Trend Monitoring | 34.263568 | -85.08553 | X |  | X | X | X | X | X | X |  |
| RV_14_4831 | Flat Creek at SR 382 nr Ellijay, GA | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.639854 | -84.574449 | X |  | X |  |  |  |  |  |  |
| RV_14_4837 | Jones Creek near Jones Creek Rd, Dahlonega, GA | Coosa | Atlanta AMU | Trend Monitoring (SEMN) | 34.602401 | -84.150559 | X |  |  | X | X | X | X | X |  |
| RV_14_4841 | Lick Creek nr Langford Rd NE, Fairmount, GA | Coosa | Cartersville AMU | Targeted Monitoring (NWQI) | 34.534829 | -84.796003 | X |  |  |  |  |  |  |  |  |
| RV_14_4851 | Noonday Creek at GA 92, near Woodstock, GA | Coosa | Cartersville AMU | Trend Monitoring (USGS) | 34.08547 | -84.529354 | X |  | X | X | X |  |  |  |  |
| RV_14_4877 | Tails Creek at SR282/US Hwy 76 nr Ellijay, GA | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.686184 | -84.600248 | X |  | X |  |  |  |  |  |  |
| RV_14_5140 | Salacoa Creek at Knight Bottom Rd nr Calhoun, GA | Coosa | Cartersville AMU | Probabilistic Monitoring | 34.505 | -84.789 | X |  | X | X | X |  |  |  |  |
| RV_14_5150 | Pettit Creek at Jones Mill Road | Coosa | Cartersville AMU | Targeted Monitoring (E coli) | 34.19866 | -84.81178 | X |  | X |  |  |  |  |  |  |
| RV_15_4909 | Toccoa River 0.4 miles downstream of Blue Ridge Dam | Tennessee | Cartersville AMU | Probabilistic Monitoring | 34.887222 | -84.285278 | X |  | X | X | X |  |  |  |  |
| RV_15_4918 | West Chisckamauga Creek at GA 146, near Lakeview, GA | Tennessee | Cartersville AMU | Trend Monitoring (USGS) | 34.9572 | -85.2056 | X |  | X | X | X |  |  |  |  |
| RV_15_4921 | Peavine Creek at Old Dixie Hwy nr Graysville, GA | Tennessee | Cartersville AMU | Targeted Monitoring (E coli) | 34.964243 | -85.176007 | X |  | X |  |  |  |  |  |  |
| RV_15_4923 | Dry Creek at Houston Valley Road near Ringgold, GA | Tennessee | Cartersville AMU | Targeted Monitoring (E coli) | 34.858571 | -85.088254 | X |  | X |  |  |  |  |  |  |
| RV_15_4930 | Little Tennessee River at Georgia Hwy 246 | Tennessee | Atlanta AMU | Targeted Monitoring (E coli) | 34.984722 | -83.382222 | X |  | X |  |  |  |  |  |  |
| RV_15_4939 | Butternut Creek nr Meeks's Park | Tennessee | Cartersville AMU | Targeted Monitoring (E coli) | 34.867853 | -83.972174 | X |  | X |  |  |  |  |  |  |
| RV_15_4961 | East Chickamauga Crk at Lower Gordon Springs Rd nr Dalton, GA | Tennessee | Cartersville AMU | Trend Monitoring | 34.746923 | -85.12355 | X |  | X | X | X | X | X | X |  |
| RV_15_4963 | Hemptown Creek at SR 245 nr Mineral Bluff, GA | Tennessee | Cartersville AMU | Targeted Monitoring (E coli) | 34.915712 | -84.279382 | X |  | X |  |  |  |  |  |  |
| RV_15_4985 | Youngcane Creek at Byer's Road nr Blairsville, GA | Tennessee | Cartersville AMU | Targeted Monitoring (E coli) | 34.870248 | -84.076103 | X |  | X |  |  |  |  |  |  |

Routine field and chemical parameters include: gage height / tape down or discharge measurement, air temperature, water temperature, dissolved oxygen, pH, specific conductance, turbidity, 5-day BOD, alkalinity, hardness, suspended solids, ammonia, nitrate-nitrite, Kjeldahl nitrogen, total phosphorus, and total organic carbon.

Lakes/estuaries field, chemical and biological parameters include:  water depth, secchi disk transparency, photic zone depth, air temperature, depth profiles for dissolved oxygen, temperature, pH, and specific conductance, and chemical analyses for turbidity, specific conductance, 5-day BOD, pH, alkalinity, hardness, suspended solids, ammonia, nitrate-nitrite, Kjeldahl nitrogen, total phosphorus, total organic carbon, and chlorophyll a.

1 Sampling Organization:  Atlanta AMU = GAEPD Atlanta office; Augusta AMU =  GAEPD Phinizy Center for Water Sciences; Brunswick AMU = GAEPD Brunswick Regional Office; Cartersville AMU = GAEPD Cartersville Regional Office; Tifton AMU = GAEPD Tifton Regional office.

2 Routine field and chemical parameters include: gage height/tape down or discharge measurement, air temperature, water temperature, dissolved oxygen, pH, specific conductance, turbidity, 5-day BOD, alkalinity, hardness, suspended solids, ammonia, nitrate-nitrite, Kjeldahl nitrogen, total phosphorus, total organic carbon, and E. coli.

3 Biomonitoring: conducted for invertebrates and periphyton using Georgia EPD protocols.

- COASTAL SHELLFISH MONITORING STATIONS

| Station Number | Sampling Site | Latitude | Longitude |
| --- | --- | --- | --- |
| 1049 | Southernmost tributary off Romerly Marsh Creek | 31.92866 | -81.01839 |
| 1050 | Northern mouth of Habersham Creek | 31.92503 | -81.0086 |
| 1052 | Northernmost tributary off Romerly Marsh Creek | 31.94317 | -81.00914 |
| 1152 | Old Romerly Marsh Creek | 31.92557 | -80.9852 |
| 1153 | Romerly Marsh Creek Chatham | 31.92993 | -80.98919 |
| 1154 | Halfmoon River at Beard Creek | 31.97741 | -80.9679 |
| 1155 | Tybee Cut South | 31.95172 | -80.9853 |
| 1159 | Pa Cooper Creek | 31.96792 | -80.936 |
| 1200 | Mouth of House Creek Chatham | 31.946 | -80.93 |
| 1201 | North of House Creek/Wassaw Sound Chatham | 31.955 | -80.933 |
| 1222 | Cut Oyster Creek to Bull River Chatham | 32.015 | -80.924 |
| 1223 | North Fork Oyster Creek Chatham | 32.014 | -80.916 |
| 1224 | North Junction Lazaretto & Oyster Creeks Chatham | 31.998 | -80.912 |
| 1225 | South Junction Lazaretto & Oyster Creeks Chatham | 31.995 | -80.91 |
| 1337 | Bull River upstream of Betz Creek | 32.02829 | -80.9473 |
| 1338 | Betz Creek | 32.02005 | -80.94529 |
| 1339 | Bull River at Shad River | 32.01 | -80.93 |
| 1340 | Shad River | 32 | -80.95 |
| 1352 | Priest Landing Chatham | 31.96058 | -81.01186 |
| 1353 | Bull River | 32.01 | -80.92 |
| 3242 | Medway River Near Sunbury | 31.685 | -81.296 |
| 3249 | Halfmoon East | 31.686 | -81.277 |
| 3255 | Mouth of Jones Hammock Creek | 31.734 | -81.194 |
| 3273 | Bear River across from Newell Creek | 31.741 | -81.161 |
| 3275 | Bear River across from Kilkenny | 31.771 | -81.17 |
| 3285 | Dickinson Creek Mouth | 31.7568 | -81.2724 |
| 3286 | Jones Creek Mouth | 31.74765 | -81.2541 |
| 3288 | Medway River East of Sunbury Creek | 31.728 | -81.22028 |
| 3291 | Van Dyke Creek Mouth | 31.6894 | -81.194 |
| 3319 | Walburg Northwest | 31.68713 | -81.1563 |
| 4092 | Eagle Creek, McIntosh | 31.51 | -81.278 |
| 4100 | Back River at July Cut | 31.53 | -81.33 |
| 4120 | Mud River at Dog Hammock | 31.52777 | -81.25732 |
| 4122 | Little Mud River at Barbour Island River | 31.59343 | -81.2612 |
| 4123 | Sapelo Sound at Highpoint | 31.53432 | -81.2243 |
| 4175 | Old Teakettle Creek, McIntosh | 31.442 | -81.306 |
| 4177 | Shellbluff Creek, McIntosh | 31.476 | -81.332 |
| 4178 | Creighton Narrows, McIntosh | 31.488 | -81.323 |
| 4179 | New Teakettle Creek, McIntosh | 31.485 | -81.295 |
| 4180 | Front River, McIntosh | 31.523 | -81.291 |
| 4184 | Juliention River, McIntosh | 31.554 | -81.314 |
| 4185 | Little Mud River, McIntosh | 31.5636 | -81.25778 |
| 4186 | South Mouth Barbour Island River, McIntosh | 31.55775 | -81.2329 |
| 4187 | Middle Barbour Island River, McIntosh | 31.593 | -81.236 |
| 4188 | Middle Wahoo River, McIntosh | 31.615 | -81.214 |
| 4190 | South Swain River, McIntosh | 31.632 | -81.224 |
| 4191 | North Swain River, McIntosh | 31.634 | -81.237 |
| 4195 | Todd River, McIntosh | 31.56232 | -81.21815 |
| 4196 | Crescent River, McIntosh | 31.503 | -81.335 |
| 4197 | Crescent River, South-end of Creighton, McIntosh | 31.491 | -81.332 |
| 4304 | Julienton River mouth, McIntosh | 31.559 | -81.274 |
| 4305 | Julienton River middle, McIntosh | 31.548 | -81.308 |
| 4306 | Four Mile Island southwest, McIntosh | 31.539 | -81.302 |
| 4330 | Jolly Creek | 31.555 | -81.29 |
| 4333 | South end of Sapelo Island | 31.38741 | -81.28912 |
| 4400 | Julienton River, middle, McIntosh | 31.557 | -81.294 |
| 5069 | Jointer River Mouth, Glynn | 31.055 | -81.469 |
| 5105 | Jointer River - Mac’s Basin | 31.1 | -81.516 |
| 5198 | Mouth Cedar Creek, Glynn | 31.089 | -81.479 |
| 5199 | Jointer River, Glynn | 31.08 | -81.506 |
| 5200 | Cobb Creek, Glynn | 31.071 | -81.483 |
| 5322 | Jointer Island West, Glynn | 31.091 | -81.515 |
| 5357 | Jointer Creek at Sage Dock, Glynn | 31.102 | -81.527 |
| 5358 | Jointer Creek upstream of Sage Dock, Glynn | 31.106 | -81.533 |
| 5359 | Little Satilla River at Honey Creek, Glynn | 31.064 | -81.526 |
| 6201 | Little Satilla River Camden | 31.039 | -81.491 |
| 6210 | Cabin Bluff Camden | 30.892 | -81.512 |
| 6212 | North Brickhill River, Camden | 30.904 | -81.461 |
| 6213 | Delaroche Creek Mouth, Camden | 30.863 | -81.497 |
| 6214 | South Brickhill River, Camden | 30.85 | -81.477 |
| 6215 | Mouth Black Point Creek, Camden | 30.858 | -81.541 |
| 6216 | Crooked River Camden | 30.849 | -81.542 |
| 6217 | Crooked River South Camden | 30.841 | -81.521 |
| 6218 | South Crooked River Mouth Camden | 30.823 | -81.498 |
| 6300 | Cumberland River-Marker #39 Camden | 30.927 | -81.452 |
| 6317 | Cumberland River East Shellbine Camden | 30.910 | -81.485 |
| 6318 | Delaroche Creek Headwaters Camden | 30.861 | -81.508 |
| 6323 | Brickhill River Upstream 6214 Camden | 30.855 | -81.467 |
| 6343 | Brickhill River West Bend, Camden | 30.868 | -81.485 |
| 6344 | Mumford Creek at Brickhill River, Camden | 30.883 | -81.479 |
| 6360 | Maiden Creek | 31.0693 | -81.545 |
| 6361 | Honey Creek | 31.0547 | -81.539 |
| 6411 | Downstream from Cabin Bluff @ marker 51A, Camden | 30.881 | -81.511 |
| 6412 | Upstream from Delaroche ck @ marker 55, Camden | 30.87 | -81.499 |

- COASTAL BEACH MONITORING STATIONS

| Station ID | Beach Name | County | Frequency* |
| --- | --- | --- | --- |
| SIN | Saint Simons Island - North Beach at Goulds Inlet | Glynn | Weekly |
| SIM | Saint Simons Island - Middle Beach (aka East Beach Old Coast Guard Station) | Glynn | Weekly |
| SIMA | Saint Simons Island - Massengale Park Beach | Glynn | Weekly |
| SIF | Saint Simons Island - 5th Street Crossover Beach | Glynn | Weekly |
| SIS | Saint Simons Island - South Beach at Lighthouse | Glynn | Weekly |
| JICC | Jekyll Island - Clam Creek Beach | Glynn | Quarterly |
| JIDW | Jekyll Island – Driftwood Beach | Glynn | Weekly |
| JIN | Jekyll Island - North Beach at Dexter Lane | Glynn | Weekly |
| JIWY | Jekyll Island - Captain Wylly Road Crossover Beach | Glynn | Weekly |
| JIM | Jekyll Island - Middle Beach at Convention Center | Glynn | Weekly |
| JISD | Jekyll Island - South Dunes Picnic Area Beach | Glynn | Weekly |
| JIS | Jekyll Island - South Beach at 4-H Camp | Glynn | Weekly |
| JISA | Jekyll Island - St. Andrews Beach | Glynn | Quarterly |
| BIRP | Blythe Island Sandbar Beach | Glynn | Monthly |
| REIM | Reimolds Pasture Beach | Glynn | Monthly |
| SEN | Sea Island - North Beach | Glynn | Monthly |
| SES | Sea Island - South Beach | Glynn | Monthly |
| CNBF | Contentment Bluff Sandbar Beach | McIntosh | Monthly |
| DALL | Dallas Bluff Sandbar Beach | McIntosh | Monthly |
| TYP | Tybee Island - Polk Street Beach | Chatham | Weekly |
| TYN | Tybee Island - North Beach at Gulick Street | Chatham | Weekly |
| TYM | Tybee Island - Middle Beach at Center Terrace | Chatham | Weekly |
| TYST | Tybee Island - Strand Beach at Pier | Chatham | Weekly |
| TYS | Tybee Island - South Beach at Chatham Street | Chatham | Weekly |
| SKID | Skidaway Narrows County Park Beach (aka Butterbean Beach) | Chatham | Monthly |
| KING | Kings Ferry County Park Beach | Chatham | Quarterly |
| BOSS | Ossabaw Island – Bradley Point | Chatham | Monthly |
| SOSS | Ossabaw Island – South Beach | Chatham | Monthly |

*Stations sampled monthly are monitored April – October.

Stations sampled weekly are monitored March – October, every other week November-February.

Beaches Not monitored regularly

| CRD ID | Beach Name | County |
| --- | --- | --- |
| LCUM | Little Cumberland Island | Camden |
| PSPT | Pelican Spit | Glynn |
| RBOW | Rainbow Bar | Glynn |
| LSSI | Little St. Simons Island | Glynn |
| SAPN | Nanny Goat on Sapelo Island | McIntosh |
| SAPC | Cabretta on Sapelo Island | McIntosh |
| CATH | St. Catherines Island | Liberty |
| WILL | Williamson Island | Chatham |
| LTYB | Little Tybee Island | Chatham |

- DNR STATE PARKS LAKE BEACH MONITORING STATIONS

The following park beaches are sampled weekly from mid-April until Labor Day (Monday preceding Labor Day) each calendar year for E coli bacteria to calculate a geometric mean.  If the 30-day geometric mean exceeds 126 or a single sample exceeds 252, a beach swim advisory is issued.  The beach will be sampled further until the water quality standards are met.

| Station ID | Beach Name | Latitude | Longitude |
| --- | --- | --- | --- |
| FB_01_16574 | Elijah Clark State Park Beach | 33.856367 | -82.399955 |
| FB_12_16576 | F.D. Roosevelt State Park: Small Group Camp Beach | 32.829375 | -84.775682 |
| FB_14_16577 | Fort Mountain State Park Beach | 34.75784 | -84.707317 |
| FB_03_16579 | Fort Yargo State Park: Day Use Beach | 33.966848 | -83.730381 |
| FB_03_16580 | Hard Labor Creek State Park: Camp Rutledge Beach | 33.649489 | -83.586798 |
| FB_03_16581 | Hard Labor Cr. State Park: Camp Daniel Morgan Beach | 33.651146 | -83.593419 |
| FB_03_16582 | Hard Labor Creek State Park: Day Use Camp Beach | 33.644213 | -83.58583 |
| FB_01_16583 | Mistletoe State Park Beach | 33.649716 | -82.371765 |
| FB_14_16584 | Red Top Mountain State Park and Lodge Beach | 34.149863 | -84.715495 |
| FB_01_16585 | Richard B. Russell State Park Beach | 34.161213 | -82.744508 |
| FB_01_16586 | Tugaloo State Park Beach | 34.49139 | -83.063859 |
| FB_15_16587 | Vogel State Park Beach | 34.767508 | -83.922333 |
| FB_01_16588 | A.H. Stephens State Park Group Camp Beach | 33.571581 | -82.889775 |
| FB_12_16591 | George T. Bagby State Park and Lodge Beach | 31.666511 | -85.060356 |
| FB_11_16592 | Georgia Veterans State Park Beach | 31.955919 | -83.921094 |
| FB_09_16593 | Reed Bingham State Park Beach | 31.162559 | -83.547612 |
| FB_11_16594 | Seminole State Park Beach | 30.805162 | -84.876266 |
| FB_05_16595 | Little Ocmulgee State Lodge Park Beach | 32.088653 | -82.893332 |
| FB_12_16596 | Unicoi State Park Day Use Beach | 34.730395 | -83.725576 |
| FB_12_16598 | Don Carter Beach | 34.388519 | -83.740342 |
| FB_01_16599 | Tallulah Gorge Beach | 34.738925 | -83.398866 |
| FB_14_16600 | Rocky Mountain Public Fishing Area (State Park) Beach | 34.361424 | -85.314603 |
| FB_07_16601 | Laura Walker State Park Beach | 31.140241 | -82.213717 |
| FB_12_16602 | Kolomoki Mounds State Historic Park Beach | 31.467097 | -84.930566 |

- MERCURY IN FISH TREND MONITORING STATIONS

| Antioch Lake at Rocky Mtn. PFA | Flint River below Ichawaynochaway Creek |
| --- | --- |
| Oostanaula River at Georgia Hwy. 140 | Lake Kolomoki at Kolomoki State Park |
| Lake Acworth | Satilla River below U.S. Hwy. 82 |
| Lake Tugalo | Okefenokee Swamp National Wildlife Refuge |
| Bear Creek Reservoir | Banks Lake National Wildlife Refuge |
| Randy Pointer Lake (Black Shoals Reservoir) | Savannah River at U.S. Hwy. 301 |
| Chattahoochee River below Morgan Falls | Savannah River at I-95 |
| Chattahoochee River Below Franklin | Ogeechee River at GA Hwy. 204 |
| Lake Tobesofkee | Wassaw Sound |
| Ocmulgee River below Macon at GA Hwy. 96 | Altamaha Delta and Sound |
| Lake Andrews | St. Andrews Sound |

Parameters tested in the general contaminant program:

- PARAMETERS FOR FISH TISSUE TESTING

| Antimony | a-BHC | Heptachlor |
| --- | --- | --- |
| Arsenic | b-BHC | Heptachlor Epoxide |
| Beryllium | d-BHC | Toxaphene |
| Cadmium | g-BHC (Lindane) | PCB-1016 |
| Chromium, Total | Chlordane | PCB-1221 |
| Copper | 4,4-DDD | PCB-1232 |
| Lead | 4,4-DDE | PCB-1242 |
| Mercury | 4,4-DDT | PCB-1248 |
| Nickel | Dieldrin | PCB-1254 |
| Selenium | Endosulfan I | PCB-1260 |
| Silver | Endosulfan II | Methoxychlor |
| Thallium | Endosulfan Sulfate | HCB |
| Zinc | Endrin | Mirex |
| Aldrin | Endrin Aldehyde | Pentachloroanisole |
|  |  | Chlorpyrifos |

- GROUNDWATER MONITORING WELLS

| Well ID | Well Name | Owner | Address | Aquifer | Well Depth (ft.) | Years Monitored |
| --- | --- | --- | --- | --- | --- | --- |
| GW_05_2766 | Unadilla #3 | City of Unadilla | P.O. Box 307 Unadilla, GA 31091 | Claiborne | 315 | 2025-2026 |
| GW_11_2466 | Flint River Nursery Office Well | Flint River State Nursery | 9850 River Road Byromville, GA 31007 | Claiborne | 90 | 2025-2026 |
| GW_11_2673 | Plains Well #8 | Water and Sewer City of Plains | P.O. Box 190 Plains, GA 31780 | Claiborne | 230 | 2025-2026 |
| GW_11_2791 | Weathersby house well | Randy & Judi Weathersby |  | Clayton | 80 | 2025-2026 |
| GW_11_5032 | Briar Patch MHP Well | David Miller |  | Clayton | Currently Unknown | 2025-2026 |
| GW_11_5033 | City of Andersonville Well #1 | Jim Copeland |  | Clayton | 230 | 2025-2026 |
| GW_11_17617 | Dawson Crawford Street Well | City of Dawson | PO Box 190 Dawson, GA 39842 | Clayton | 367 | 2025-2026 |
| GW_11_17618 | Cuthbert Well #3 | City of Cuthbert | PO Box 100 Cithbert, GA 39840 | Clayton | 355 | 2025-2026 |
| GW_01_2523 | Hephzibah/Murphy Street Well | City of Hephzibah | Hephzibah City Hall P.O. Box 250 Hephzibah, GA 30815-0250 | Cretaceous | 484 | 2025-2026 |
| GW_01_15178 | City of Keysville Well #1 | City of Keysville | P.O. Box 159 Keysville, GA 30816-0159 | Cretaceous | Currently Unknown | 2025-2026 |
| GW_02_2704 | Sandersville Well #7B | City of Sandersville | Sandersville Annex Building 110 South Hospital Rd. Sandersville, GA 31082 | Cretaceous | 697 | 2025 |
| GW_02_15200 | Town of Mitchell Municipal Well #3 | Town of Mitchell | P.O. Box 32 Mitchell, GA 30820 | Cretaceous | Currently Unknown | 2025-2026 |
| GW_02_18180 | Sandersville Well #10 | City of Sandersville | Sandersville Annex Building 110 South Hospital Rd. Sandersville, GA 31082 | Cretaceous | 772 | 2026 |
| GW_03_17616 | Irwinton Well #4 | City of Irwinton | PO Box 359 Irwin, GA 31042 | Cretaceous | 400 | 2025-2026 |
| GW_05_2474 | Fort Valley Well #6 | Fort Valley Utility Commission | P.O. Box 1529 Fort Valley, GA 31030 | Cretaceous | 600 | 2025-2026 |
| GW_05_2560 | Jones County #4 | Jones County Water System | Jones County Water System 270 Highway 49 Macon, GA 31211 | Cretaceous | 128 | 2025-2026 |
| GW_05_2669 | Perry/Holiday Inn Well | City of Perry | ESG, Inc. P.O. Box 2030 Perry, GA 31069 | Cretaceous | 550 | 2025-2026 |
| GW_05_2778 | Warner Robins #2 | City of Warner Robins | ESG, Inc. 202 North Davis Dr., PMB 718 Warner Robins, GA 31093 | Cretaceous | ~540 | 2025-2026 |
| GW_11_2607 | Marshallville Well #2 | Marshallville Water and Sewer Dept. | 111 Main Street West Marshallville, GA 31057 | Cretaceous | 550 | 2025-2026 |
| GW_11_2672 | Plains Well #7 | Water and Sewer City of Plains | P.O. Box 190 Plains, GA 31780 | Cretaceous | 1000 | 2025-2026 |
| GW_11_5030 | Unimin Well #1 | Unimin Georgia Co., LLC | 1333 Sandpit Rd. Mauk, GA 31058 | Cretaceous | 150 | 2025-2026 |
| GW_11_5031 | Whitewater Creek Well | Whitewater Creek Park | 165 Whitewater Rd. Oglethorpe, GA 31068 | Cretaceous | Currently Unknown | 2025-2026 |
| GW_12_5037 | Camp Darby Well near Cussetta, GA | Columbus Water Works | P.O. Box 1600 Columbus, GA 31902-1600 | Cretaceous | Currently Unknown | 2025-2026 |
| GW_12_5046 | Louvale Community Well | Stewart County. Water. & Sewer Authority | P.O. Box 157 Lumpkin, GA 31815-0157 | Cretaceous | Currently Unknown | 2025-2026 |
| GW_12_17615 | Georgetown Well #3 | Quitman County | Georgetown Public Works PO Box 297 Georgetown, GA 31754 | Cretaceous | Currently Unknown | 2025-2026 |
| GW_01_2763 | Tybee Island #1 | City of Tybee Island | City of Tybee Island Water & Sewer Dept. Tybee Island, GA 31328 | Floridan | 402 | 2025-2026 |
| GW_02_2546 | Interstate Paper #1 | Interstate Paper, LLC | Interstate Paper, LLC 2366 Interstate Road Riceboro, GA 31323-3933 | Floridan | 810 | 2025 |
| GW_02_2526 | Hinesville #5 | City of Hinesville | CH2MHILL-OMI/Hinesville 613 E.G. Miles Parkway Hinesville, GA 31313 | Floridan | 806 | 2025-2026 |
| GW_02_2615 | Metter #2 | City of Metter | Metter Public Works Dept P.O. Box 74 Metter, GA 30439 | Floridan | 540 | 2025-2026 |
| GW_02_2620 | Millen #1 | City of Millen | 919 College Ave. Millen, GA 30442-1633 | Floridan | 500 | 2025-2026 |
| GW_02_2707 | Savannah #13 | City of Savannah | 208 Agonic Rd. Savannah, GA 31406 | Floridan | 1004 | 2025-2026 |
| GW_02_2736 | Statesboro #4 | City of Statesboro | Hill St. at Mulberry St. (office/shop) P.O. Box 348 Statesboro, GA 30459 | Floridan | 413 | 2025-2026 |
| GW_02_2741 | Swainsboro #7 | City of Swainsboro | (ofc) CH2M Hill 574 Industrial Way Swainsboro, GA 30401 | Floridan | 260 | 202565 |
| GW_02_5005 | Sapelo Gardens S/D #1 | South Atlantic Utilities, Inc. | P.O. Box 13705 Savannah, GA 31416-3705 | Floridan | 660 | 2025-2026 |
| GW_02_5006 | Hampton River Marina | Hampton River Marina | 1000 Hampton Pointe Drive St Simons Island GA 31522 | Floridan | 750 | 2025-2026 |
| GW_05_2450 | Eastman #4 | City of Eastman | Eastman City Hall 410 Main Street Eastman, GA 31023 | Floridan | 410 | 2025-2026 |
| GW_05_2611 | McRae Well #3 | City of McRae | McRae City Hall P.O. Box 157 McRae, GA 31055-0157 | Floridan | 600+ | 2025-2026 |
| GW_05_17478 | McRae Well #1 | City of McRae | McRae City Hall P.O. Box 157 McRae, GA 31055-0157 | Floridan | Currently Unknown | 2025-2026 |
| GW_05_17479 | McRae Well #2 | City of McRae | McRae City Hall P.O. Box 157 McRae, GA 31055-0157 | Floridan | Currently Unknown | 2025-2026 |
| GW_05_17480 | McRae Well #4 | City of McRae | McRae City Hall P.O. Box 157 McRae, GA 31055-0157 | Floridan | Currently Unknown | 2025-2026 |
| GW_05_17908 | Swainsboro #6 | City of Swainsboro | Ofc( CH2MHill 574 Industrial Way Swainsboro, GA 30401 | Floridan | NG | 2026 |
| GW_06_2772 | Vidalia #1 | City of Vidalia | ESG, Inc., 111 Brinson Rd. Vidalia, GA 30474 | Floridan | 808 | 2025-2026 |
| GW_07_2561 | Jowers Crossing (Well #2) | City of Ambrose | 96 Curtis Vickers Road Ambrose, GA 31512 | Floridan | 600 | 2025-2026 |
| GW_07_2623 | Miller Ball Park North East Well | Glynn County Board of Education | 200 Emory Dawson Road Brunswick, GA 31520 | Floridan | 1211 | 2025-2026 |
| GW_07_2785 | Waycross #3 | City of Waycross | ESG, Inc. P.O. Drawer 99/512 Alice Street. Waycross, GA 31502-0099 | Floridan | 775 | 2025-2026 |
| GW_07_5024 | Hofwyl-Broadfield Well | Hofwyl-Broadfield Plantation Historic Site | 5556 US Highway 17N Brunswick, GA 31525 | Floridan | Currently Unknown | 2025-2026 |
| GW_07_5025 | Jekyll Island | City of Jekyll Island | 100 James Road Jekyll Island GA 31527 | Floridan | 850 | 2025-2026 |
| GW_07_5026 | Ft. Morris Well | Ft. Morris Historic Site | 2559 Fort Morris Road Midway, GA 31320 | Floridan | 500 | 2025-2026 |
| GW_09_2308 | Adel #6 | City of Adel | City of Adel Water & Sewer Dept. 404 Poplar St. Adel, GA 31620 | Floridan | 405 | 2025-2026 |
| GW_09_2580 | Lakeland #2 | City of Lakeland | Lakeland City Hall 64 South Valdosta Road Lakeland, Georgia 31635 | Floridan | 340 | 2025-2026 |
| GW_09_2639 | Moultrie #1 | City of Moultrie | 2701 1st Ave. SE P.O. Box 3368 | Floridan | 750 | 2025-2026 |
| GW_09_2653 | Ocilla #3 | City of Ocilla | P.O. Box 626 Ocilla, GA 31774-0626 | Floridan | 637 | 2025-2026 |
| GW_09_2743 | Sycamore #2 | City of Sycamore | Sycamore City Hall 2529 US Highway 41 Sycamore, GA 31790-2201 | Floridan | 501 | 2025-2026 |
| GW_09_2746 | Sylvester #1 | City of Sylvester | Sylvester Water, Gas, & Light Dept. P.O. Box 370 Sylvester, GA 31791-0370 | Floridan | 196 | 2025-2026 |
| GW_09_2756 | Tifton #6 | City of Tifton | 80 Old Brookfield Rd P.O. Box 229 Tifton, GA 31793 | Floridan | 652 | 2025-2026 |
| GW_09_5015 | Ashburn #4 | City of Ashburn | Ashburn Water Department 291 Mill St. Ashburn, GA 31714 | Floridan | 600 | 2025-2026 |
| GW_10_2425 | Davis Ave. (Well #1) | City of Whigham | P.O. Box 71 Whigham, GA 39897 | Floridan | 604 | 2025-2026 |
| GW_10_2753 | Thomasville #6 | City of Thomasville | Mr. Bill Gerber 411 W. Jackson Street Thomasville, GA 31792 | Floridan | 400 | 2025-2026 |
| GW_10_5029 | Waverly/Four Corners #1 | City of Thomasville | P.O. Box 1540 Thomasville, GA 31799-1540 | Floridan | 900 | 2025-2026 |
| GW-10_17585 | Cairo #11 | City of Cario | Cairo City Hall P.O. Box 29 Cairo, GA 39828 | Floridan | 450 | 2025-2026 |
| GW_11_2376 | Camilla Ind. Pk. Well | City of Camilla | P.O. Box 328 Camilla, GA 31730 | Floridan | 360 | 2025-2026 |
| GW_11_2433 | Donalsonville / 7th St. Well | City of Donalsonville | P.O. Box 308 Donalsonville, GA 31745 | Floridan | 174 | 2025-2026 |
| GW_11_16636 | Smith House Well | Gerald Smith | 7983 Malone Drive Donalsonville, GA 31745 | Floridan | Currently Unknown | 2025-2026 |
| GW_11_16637 | Radium Spring | City of Albany | 2501 Radium Springs Rd Albany, GA 31705 | Floridan | Currently Unknown | 2025-2026 |
| GW_01_2384 | Cecchini Deep Well | Mr. Charles Cecchini |  | Piedmont/ Blue Ridge | 400 | 2025-2026 |
| GW_01_2465 | Fizer well | Mr. Alan Fizer | 1079 Oak Ct. Lincolnton, GA 30817 | Piedmont/ Blue Ridge | 220 | 2025-2026 |
| GW_01_2627 | Mistletoe SP Cottage Area Well | Ga. DNR Parks & Historic Sites | Mistletoe State Park 3725 Mistletoe Road Appling, GA 30802 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_01_2645 | Mt Airy City Hall Well | City of Mt Airy | P.O. Box 257 Mt Airy, GA 30563-0257 | Piedmont/ Blue Ridge | 500 | 2025-2026 |
| GW_01_2655 | O’Connor house well | Dr. Bruce O'Connor |  | Piedmont/ Blue Ridge | 150 | 2025-2026 |
| GW_01_4993 | Beaverdam MHP #1 | Mr. Tom Cleveland |  | Piedmont/ Blue Ridge | 250 | 2025-2026 |
| GW_01_4994 | Victoria Bryant SP #101 | Victoria Bryant State Park | 1105 Bryant Park Road Royston, GA 30662 | Piedmont/ Blue Ridge | 320 | 2025-2026 |
| GW_01_4997 | City of Ila Well #1 | City of Ila | P.O. Box 46 Ila, GA 30647-0046 | Piedmont/ Blue Ridge | 650 | 2025-2026 |
| GW_01_4999 | Windy Acres MHP #1 | Windy Acres Mobile Home Park | 630 South Old Belair Rd. Lot 30 Grovetown, GA 30813 | Piedmont/ Blue Ridge | 180 | 2025-2026 |
| GW_01_5000 | Lake Harbor Shores #4 | Lake Harbor Shores | 433 Seminole Trail Martin, GA 30557 | Piedmont/ Blue Ridge | 380 | 2025-2026 |
| GW_01_5003 | City of Rayle Well #1 | Town of Rayle | PO Box 67, Rayle GA 30660-0067 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_01_15196 | Grovetown Municipal Well #1 | City of Grovetown | PO Box 120 Grovetown, GA 30813-0120 | Piedmont/ Blue Ridge | 600 | 2025 |
| GW_01_15198 | Tradewinds Marina well | Tradewinds Marina | 5577 Marina Parkway Appling, GA 30802 | Piedmont/Blue Ridge | 60 | 2025-2026 |
| GW_01_15732 | Wilson Family Well | Roger Wilson |  | Piedmont/ Blue Ridge | 80 | 2025-2026 |
| GW_01_17637 | Gold Mine Landing Well | Gold Mine Landing | Clayton, GA 30525 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_02_5008 | Hamburg State Park | Hamburg State Park | 6071 Hamburg State Park Road Mitchell, GA 30820 | Piedmont/ Blue Ridge | 340 | 2025-2026 |
| GW_03_2357 | Bragg Well | City of Gray | Gray City Hall P.O. Box 443 Gray, GA 31032-0443 | Piedmont/ Blue Ridge | 405 | 2025-2026 |
| GW_04_2047 | Siloam #2 | City of Siloam | P.O. Box 9 Siloam, GA 30665 | Piedmont/ Blue Ridge | 300 | 2025-2026 |
| GW_05_2540 | Indian Spring | Ga. DNR Parks & Historic Sites | Indian Springs State Park 678 Lake Clark Road Flovilla , GA 30216 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_05_2541 | Indian Springs New Main Well | Ga. DNR Parks & Historic Sites | Indian Springs State Park 678 Lake Clark Road Flovilla , GA 30216 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_05_5017 | Jarrell Plantation Staff House Well | Ga. DNR Parks & Historic Sites | 695 Jarrell Plantation Road Juliette, GA 31046 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_05_17465 | Reeves House Well | Ms. Collie Reeves | 1129 Crawford Road Barnesville, GA 30204 | Piedmont/ Blue Ridge | 445 | 2025 |
| GW_11_2487 | Gay #1 | City of Gay | 18762 Highway 85 P.O. Box 257 Gay, GA 30218-0257 | Piedmont/ Blue Ridge | 600 | 2025-2026 |
| GW_11_2600 | Well #3 | City of Luthersville | 104 Wortham Rd. P.O. Box 10 Luthersville, GA 30251-0010 | Piedmont/ Blue Ridge | 185 | 2025-2026 |
| GW_11_2748 | The Gates #1 | Mr. Derek Bunch |  | Piedmont/ Blue Ridge | 705 | 2025-2026 |
| GW_11_5035 | Country Village SD Well#13 | SOS Enterprises | 205 East Gordon Street Thomaston GA 30266 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_11_16635 | Lone Oak Well | Mr. Derek Bunch |  | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_11_17619 | Warm Spring at FD Roosevelt SP | Ga. DNR Parks & Historic Sites | FD Roosevelt State Park BOX 2970 Hwy 190 East Pine Mountain, GA 31822 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_12_2468 | Flowery Branch Well #1 | City of Flowery Branch | Flowery Branch Water & Sewer Dept. P. O. Box 757 Flowery Branch, GA 30542 | Piedmont/ Blue Ridge | 240 | 2025-2026 |
| GW_12_2532 | Rahbar house well | Mr. Bijan Rahbar |  | Piedmont/ Blue Ridge | 200 | 2025-2026 |
| GW_12_2700 | Roopville #1 | City of Roopville | 284 S. Old Highway 27 P.O. Box 165 Roopville, GA 30170 | Piedmont/ Blue Ridge | 230 | 2025-2026 |
| GW_12_2740 | Suwanee #1 | Suwanee Public Works Division | 330 Town Center Avenue Suwanee, GA 30024 | Piedmont/ Blue Ridge | 600 | 2025 |
| GW_12_5041 | Well #1 Leisure Lake Village | Leisure Lake Condo Association | PO Box 1706 Gainesville, GA 30503-1706 | Piedmont/ Blue Ridge | 380 | 2025-2026 |
| GW_12_5042 | Valley Inn and RV Park Well | VIOH, LLC | 524 South Main Avenue Pine Mountain, GA 31822 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_12_5043 | FD Roosevelt Spring | FD Roosevelt State Park | 2970 Highway 190 East Pine Mountain, GA 31822 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_12_5049 | Sweetwater Coffeehouse | Sweetwater Coffeehouse | P.O. Box 381 Sautee Nacoochee, GA 30571 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_12_18119 | Wedgewood Circle Well | Mr. Derek Bunch | P.O. Box 768771 Roswell, GA 30076 | Piedmont/Blue Ridge | NG | 2026 |
| GW_14_2650 | Nix Spring | Chatsworth Water Works Commission | P.O. Box 100 Chatsworth, GA 30705 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_14_5050 | Willow Court Well | Mr. Derek Bunch |  | Piedmont/ Blue Ridge | 220 | 2025-2026 |
| GW_14_17589 | Jasper Spring | Public |  | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_14_17638 | Jacobs House Well | Tommy Jacobs |  | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_14_17739 | Voudy House Well | Christine Voudy | 335 Bethesda Trail Ball Ground GA 30107 | Piedmont/ Blue Ridge | 525 | 2025-2026 |
| GW_15_2806 | Young Harris Swanson Road Well | Young Harris Water Department | P.O. Box 122 Young Harris, GA 30582 | Piedmont/ Blue Ridge | 265 | 2025-2026 |
| GW_15_5052 | Brasstown Bald Spring | USFS Brasstown Ranger District | 2042 Highway. 515 W, Blairsville, GA 30512 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_15_5053 | Bryant Cove SD Well #2 | Appalachian Water Inc | PO Box 2381 Blairsville GA 30514 | Piedmont/ Blue Ridge | 605 | 2025-2026 |
| GW_15_17462 | Young Harris College Well | Young Harris Water Department | P.O. Box 122 Young Harris, GA 30582 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_15_17742 | Young Harris Main St. Well | City of Young Harris | PO Box 122 Young Harris, GA 305 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_15_17759 | Willer House Well | Gary Willer | 81 Dallas Collins Rd Blairsville, GA 30512 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_15_18043 | Willer House Well | Gary Willer | 81 Dallas Collins Rd Blairsville, GA 30512 | Piedmont/ Blue Ridge | Currently Unknown | 2025-2026 |
| GW_01_2801 | Wrens #4 | City of Wrens | 415 W. Walker Street Wrens, GA 30833 | Jacksonian | 200 | 2025-2026 |
| GW_01_2803 | Wrightsville #4 | City of Wrightsville | 2566 East Elm Street Wrightsville, GA 31096 | Jacksonian | 520 | 2025-2026 |
| GW_02_2562 | Kahn House Well | Lee and Thelma Kahn |  | Jacksonian | 40 | 2025-2026 |
| GW_02_2610 | McNair House Well | Bob and Ann McNair |  | Jacksonian | ~90 | 2025-2026 |
| GW_02_15202 | City of Bartow Municipal Well #1 | City of Bartow | PO Box 248 Bartow, GA 30413 | Jacksonian | 345 | 2025-2026 |
| GW_02_17261 | Henley 1 Louisville | Geneda Henley | 1082 Darisaw Circle Louisville, GA 30434 | Jacksonian | ~90 | 2025-2026 |
| GW_02_17262 | Henley 2 Bartow | Geneda Henley | 1082 Darisaw Circle Louisville, GA 30434 | Jacksonian | ~90 | 2025-2026 |
| GW_05_2398 | Cochran #3 | City of Cochran | Cochran City Hall 108 NE Dyke Street Cochran, Georgia 31014 | Jacksonian | 307 | 2025-2026 |
| GW_06_5019 | City of Harrison Well #1 | Town of Harrison | P.O. Box 31 Harrison, GA 31035-0031 | Jacksonian | Currently Unknown | 2025-2026 |
| GW_06_5020 | City of Riddleville Well #1 | City of Riddleville | 9019 Highway 242 Harrison, GA 31035 | Jacksonian | 330 | 2025-2026 |
| GW_01_2730 | Springfield Egypt Road Test Well | Ga. DNR & Effingham County Engineer | 601 North Laurel Street Springfield, GA 31329 | Miocene | 120 | 2025-2026 |
| GW_02_18182 | Liberty Coiunty Fire Station 20 | Liberty County | 4305 Islands Highway Midway, GA 31320 | Miocene | NG | 2026 |
| GW_06_5021 | Raintree TP Main Well | Raintree Trailer Park | 669 Spring Grove Rd. Jesup, GA 31545 | Miocene | 400 | 2025-2026 |
| GW_09_2310 | McMillan House Well | Mr. Willie McMillan |  | Miocene | 220 | 2025-2026 |
| GW_09_2354 | Boutwell House Well | Mr. Stacey Boutwell |  | Miocene | 70 | 2025-2026 |
| GW_10_2373 | Calhoun House Well | Ms. LaRue Calhoun |  | Miocene | 150 | 2025-2026 |
| GW_11_2350 | Blakely Well #4 | City of Blakely | Blakely Water Treatment Dept. P.O. Box 350 Blakely, GA 39823 | Providence | 1025 | 2025-2026 |
| GW_11_2676 | Preston Well #4 | Unified Government of Webster County | P.O. Box 29 Preston, GA 31824 | Providence | 205 | 2025-2026 |
| GW_11_5036 | Weston Well #1 | Chris Shannon |  | Providence | Currently Unknown | 2025-2026 |
| GW_12_2473 | Fort Gaines Well #2 | City of Ft. Gaines | Fort Gaines City Hall P.O. Box 251 Fort Gaines, GA 39851-0251 | Providence | 456 | 2025-2026 |
| GW_12_5047 | Providence Canyon SP well | Providence Canyon State Park | 218 Florence Rd. Omaha, GA 31821 | Providence | Currently Unknown | 2025-2026 |
| GW_14_2385 | Cedartown Spring | Cedartown Water/Wastewater Dept. | P.O. Box 65 Cedartown, GA 30125-0065 | Ridge & Valley | Currently Unknown | 2025-2026 |
| GW_14_2460 | Eton Spring | Chatsworth Water Works Commission | P.O. Box 100 Chatsworth, GA 30705 | Ridge & Valley | Currently Unknown | 2025-2026 |
| GW_14_2570 | Kingston Rd. Well | Floyd County Water Dept. | Floyd County Water Dept. P.O. Box 1169 Rome, GA 30162-1169 | Ridge & Valley | 280 | 2025-2026 |
| GW_14_2576 | LaFayette Lower Big Spring | Lafayette Water Department | Lafayette Water Department P.O. Box 89 Lafayette, GA 30728 | Ridge & Valley | Currently Unknown | 2025-2026 |
| GW_14_2725 | South Well | Chemical Products Corp. | Chemical Products Corp. P.O. Box 2470 Cartersville, GA 30120 | Ridge & Valley | 300 | 2025 |
| GW_14_17588 | Cave Spring | City of Cave Spring | PO Box 365 Cave Springs, GA 30124 | Ridge & Valley | Currently Unknown | 2025-2026 |
| GW_15_2414 | Crawfish Spring | City of Chickmauga | Water Dept., City of Chickamauga P.O. Box 369 Chickamauga, GA 30707 | Ridge & Valley | Currently Unknown | 2025-2026 |

Standard field parameters include: water temperature, dissolved oxygen, pH, specific conductance.

Standard chemical parameters include: VOCs, chloride, sulfate, nitrate-nitrite, phosphorus, chromium, nickel, copper, zinc, arsenic, selenium, molybdenum, silver, cadmium, tin, antimony, barium, thallium, lead, uranium, aluminum, beryllium, calcium, cobalt, iron, potassium, magnesium, manganese, sodium, titanium, vanadium, fluorine.

*Sampled in 2021

Appendix B

WATER USE CLASSIFICATIONS AND WATER QUALITY STANDARDS APPROVED BY EPA

FOR SPECIFIC DETAILS – REFER TO THE LASTEST EPA APPROVED VERSION OF GEORIGA’S WATER QUALITY STANDARDS ON THE GAEPD WEBSITE (https://epd.georgia.gov/watershed-protection-branch/georgia-water-quality-standards)

DESIGNATED USES

|  | Bacteria |  | Dissolved Oxygen1 (other than trout streams)2 |  |  | pH | Temperature (other than trout streams)2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Designated Use | 30-Day Geometric Mean2 (#/100 mL) | Maximum (#/100 mL) | Daily Average (mg/L) |  | Minimum (mg/L) | Std. Units | Maximum Rise (°F) | Maximum (°F) |
| Drinking Water | 126 (May-Oct) 265 (Nov-Apr) Freshwater- E coli | STV 410 (May-Oct) STV 811 (Nov-Apr Freshwater- E coli | 5.0 |  | 4.0 | 6.0-8.5 | 5 | 90 |
| Recreation | 126 Freshwater- E coli 35 Coastal-Enterococci | STV 410 Freshwater- E coli STV 130 Coastal-Enterococci | 5.0 |  | 4.0 | 6.0-8.5 | 5 | 90 |
| Fishing | 126 (May-Oct) 265 (Nov-Apr) Freshwater- E coli 35 (May-Oct) 74 (Nov-Apr) Coastal-Enterococci | STV 410 (May-Oct) STV 811 (Nov-Apr Freshwater- E coli STV 130(May-Oct) STV 273 (Nov-Apr Coastal-Enterococci | 5.0 |  | 4.0 | 6.0-8.5 | 5 | 90 |
| Coastal Fishing4 | 126 (May-Oct) 265 (Nov-Apr) Freshwater- E coli 35 (May-Oct) 74 (Nov-Apr) Coastal-Enterococci | STV 410 (May-Oct) STV 811 (Nov-Apr Freshwater- E coli STV 130(May-Oct) STV 273 (Nov-Apr Coastal-Enterococci | 5.0 | 4.0 |  | 6.0-8.5 | 5 | 90 |
|  |  |  | If it is determined that the “natural condition” in the waterbody is less than the values stated above, then the criteria will revert to the “natural condition” and the water quality standard will allow for a 0.1 mg/L deficit from the “natural” dissolved oxygen value. Up to a 10% deficit will be allowed if it is demonstrated that resident aquatic species shall not be adversely affected. |  |  |  |  |  |
| Wild River | No alteration of natural water quality |  |  |  |  |  |  |  |
| Scenic River | No alteration of natural water quality |  |  |  |  |  |  |  |

- The dissolved oxygen criteria as specified in individual water use classifications shall be applicable at a depth of one meter below the   water surface; in those instances where depth is less than two meters, the dissolved oxygen criterion shall be applied at a mid-depth. On a case specific basis, alternative depths may be specified.

- Standards for Trout Streams for dissolved oxygen are an average of 6.0 mg/L and a minimum of 5.0 mg/L. No temperature alteration is allowed in Primary Trout Streams, and a temperature change of 2 deg. F is allowed in Secondary Trout Streams.

- Geometric means should be “based on at least four samples collected from a given sampling site over a 30-day period at intervals not less than 24 hours.” The geometric mean of a series of N terms is the Nth root of their product. Example: the geometric mean of 2 and 18 is the square root of 36.

- Standards are the same as fishing with the exception of dissolved oxygen, which is site specific.

NARRATIVE WATER QUALITY STANDARDS (excerpt from Georgia Rules and Regulations for Water Quality Control Chapter 391-3-6-.03 - Water Use Classifications and Water Quality Standards) (5) General Criteria for All Waters. The following criteria are deemed to be necessary and applicable to all waters of the State: (a) All waters shall be free from materials associated with municipal or domestic sewage, industrial waste or any other waste which will settle to form sludge deposits that become putrescent, unsightly or otherwise objectionable. (b) All waters shall be free from oil, scum and floating debris associated with municipal or domestic sewage, industrial waste or other discharges in amounts sufficient to be unsightly or to interfere with legitimate water uses. (c) All waters shall be free from material related to municipal, industrial or other discharges, which produce turbidity, color, odor or other objectionable conditions, which interfere with legitimate water uses. (d) All waters shall be free from toxic, corrosive, acidic and caustic substances discharged from municipalities, industries or other sources, such as nonpoint sources, in amounts, concentrations or combinations which are harmful to humans, animals or aquatic life. (e) All waters shall be free from turbidity, which results in a substantial visual contrast in a water body due to man-made activity. The upstream appearance of a body of water shall be observed at a point immediately upstream of a turbidity-causing man-made activity. The upstream appearance shall be compared to a point, which is located sufficiently downstream from the activity so as to provide an appropriate mixing zone. For land disturbing activities, proper design, installation and maintenance of best management practices and compliance with issued permits shall constitute compliance with [this] Paragraph....