OWENS CORNING

ROOFING AND ASPHALT PLANT

ATLANTA, GEORGIA

TITLE V PERMIT AMENDMENT APPLICATION

INTRODUCTION

Owens Corning operates a roofing and asphalt facility in Atlanta, GA and is located in Fulton county, which is in attainment for all regulated pollutants except ozone.  Any facility in an attainment area, which has a potential to emit 250 tons per year or more of any regulated pollutant (except nitrogen oxides and volatile organic compounds) is subject to PSD.  The Owens Corning facility in Atlanta has limited their potential to emit to less than 250 tons per year of any regulated pollutant.  Therefore, this facility is not subject to a PSD review at this time.

The Owens Corning facility currently operates under Title V Permit No. 2952-121-0334-V-01-0 dated May 25, 2000 and Part 70 Operating Permit Amendment No. 2952-121-0334-V-01-1 dated May 23, 2001.

The proposed Title V administrative amendment application includes operational flexibility scenarios to avoid uncontrolled emissions in the event of a control device malfunction, compliance testing strategies for the proposed flexible operating scenarios, clarification of VOC emission limits for asphalt facility RACT sources, and proposed clarifications for reporting and record keeping requirements in order to assure operational flexibility.

Operating Scenarios

The various operational scenarios will not change total emissions; however, the emissions will be vented through a different stack.  The Atlanta Trumbull facility proposes the following scenarios to be listed in the permit as follows: (1) Fume Incinerator (IB01) controlling emissions from Convertors #2 and #3 with RTO #3 controlling emissions from Convertor #1, (this scenario is a normal operating procedure);  (2)  Fume Incinerator (IB01) controlling emissions from Convertors #1 and #3 or Convertors #2 and #3 and RTO #1 controlling emissions from the  loading station (limited to only load through one of the three loading arms)  in addition to the other sources permitted to be controlled by RTO #1, when RTO #3 is down;  (3) RTO #3 controlling emissions from Convertor #1 or Convertor #2 or Convertor #3 and the loading station fumes when the Fume Incinerator is down. The emission controls for these scenarios are reflected in the attached Block Flow Diagrams.

A new exhaust stack fan will be installed for the Fume Incinerator.  It will be located adjacent to the building that houses the Fume Incinerator and will not effect emissions.

# Compliance Testing

Owens Corning proposes the following tests to demonstrate compliance for all flexible operating scenarios proposed in the amendment:

Testing Parameter	Control Device		Worst Case Scenario

Opacity and TSP	Fume Incinerator (IB01)	Convertors 2 and 3

Simultaneously Operating

Opacity and TSP	RTO #3			Convertor 2 and Three

Paving Loading Stations

Simultaneously Operating

Opacity		RTO #1			Tanks 1-11 and 1 Paving

Loading Arm

Simultaneously Operating

These testing scenarios are based on the worst-case conditions for each control device.  Convertor #2 is larger than Convertor #1 or Convertor #3.  Therefore, any scenarios that involve Convertor #1 or Convertor #3 are in compliance when Convertor #2 testing demonstrates compliance.  These proposed conditions are included in Section 3.30.

VOC Limits for RACT Sources

Owens Corning requests the following emission limits for the VOC-RACT emission sources:

Emission Sources				Annual VOC Limit

# Tanks 0-10, SBS, 20-24			7.9 tons per year total

Tanks 12 and 17				1.3 tons per year total

Paving Loading Station			1.04 tons* per year

Hot Side/Finished Product 			7.72 tons* per year

Rack Loading

*  Note:  These emissions include both captured and uncaptured emissions.

These proposed conditions are included in Section 3.30 of this application.

Reporting and Recordkeeping Clarification

Owens Corning requests that the following permit conditions be modified.

Condition 5.2.6 Sections i, ii and iii include the statement “while control device is in operation controlling emissions”.

Condition 5.2.6 Section vi. from the original permit was deleted when the amendment to the Title V was issued in May 2001.  The old Condition 5.2.6 c vi was related to Condition 5.2.3 for the dust collector DC5.  The new condition 5.2.6 c vi relates to the coater.  Owens Corning requests clarification to determine if this deletion was intentional.

The administrative revisions in this application are prepared in accordance with the Georgia Air Regulations, the Georgia Department of Natural Resources, Environmental Protection Division (EPD), Air Protection Branch’s Federal Operating Permit Application Forms and Instructions.

| Section 0.00 - Application Form - Summary of Pages Submitted |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. FACILITY NAME: |  | Owens Corning Atlanta Roofing and Asphalt Plant |  |  |
| 2. After completing all other sections in the application, indicate the total number of pages for each section in the last column of the table |  |  |  |  |
| Section Number | Section Name |  | Uncompleted Pages | Pages Submitted |
| 0.00 | Application - Summary of Pages Submitted (this form) |  | 1 | 1 |
| 1.10 | Facility Information |  | 3 | 3 |
| 1.20 | Facility Information - Required Documents |  | 1 | 4 |
| 2.10 | Part 70 Site Potential to Emit (PTE) |  | 1 |  |
| 2.20 | Facility Wide Emission Estimates |  | 1 | 3 |
| 3.10 | Facility Wide Rule Applicability |  | 9 |  |
| 3.11 | Facility Wide Title VI Applicability |  | 1 |  |
| 3.20 | Current Air Quality Permit Conditions Analysis |  | 1 |  |
| 3.21 | Justification for Permit Condition Elimination |  | 1 |  |
| 3.30 | Proposed Title V Permit Conditions |  | 1 | 2 |
| 4.10 | Insignificant Activities Checklist |  | 3 |  |
| 4.20 | Generic Emissions Grouping |  | 1 | 3 |
| 4.30 | Fugitive Emissions Associated with Leak Detection and Repair (LDAR) |  | 1 |  |
| 4.40 | Short Term Activities |  | 1 |  |
| 4.50 | Insignificant Activities based on Emission Levels |  | 1 |  |
| 5.00 | Emission Units - Summary |  | 1 |  |
| 5.10 | Emission Units - Boilers, Furnaces, and other Indirect Contact Heat Generating Equip. |  | 1 |  |
| 5.11 | Emission Units - Internal Combustion Engines and Combustion Turbines |  | 1 |  |
| 5.20 | Emission Units - Liquid Storage Vessels |  | 1 |  |
| 5.30 | Emission Units - Crushing, Milling, and Grinding |  | 1 |  |
| 5.31 | Emission Units - Dryers, Calciners, Kilns and Ovens |  | 1 |  |
| 5.40 | Emission Units - Solid/Liquid Waste Destruction (Incineration) |  | 1 |  |
| 5.41 | Emission Units - Landfills |  | 1 |  |
| 5.50 | Emission Units - Painting and Coating Operations |  | 1 |  |
| 5.51 | Emission Units - Printing Operations |  | 1 |  |
| 5.52 | Emission Units - Non-Reactive Bulk Mixing |  | 1 |  |
| 5.60 | Emission Units - Perchloroethylene Dry Cleaners |  | 1 |  |
| 5.61 | Emission Units - Solvent Cleaning Machines |  | 2 |  |
| 5.62 | Emission Units - Chromium Electroplating and Chromium Anodizing Tanks |  | 1 |  |
| 5.70 | Emission Units - Fugitive Emissions |  | 1 |  |
| 5.90 | Emission Units - Miscellaneous |  | 2 | 26 |
| 6.10 | Emission Units as Part of a Process Group [Related to Rules (e),(p),(q),(r) or (fff)] |  | 2 |  |
| 6.20 | Emission Units as Part of an Equipment Group |  | 1 | 4 |
| 6.30 | Emission Units as Part of an Operations Group |  | 1 |  |
| 7.10 | Unit, Process or Plant Wide Emissions Summary |  | 1 |  |
| 7.11 | Work Practice and Design Standards |  | 1 |  |
| 8.10 | Air Pollution Control Devices |  | 1 | 4 |
| 9.10 | Stack and Process Vent - Summary |  | 1 | 2 |
| 10.10 | Compliance Determination Procedures - Reference Test Methods |  | 1 | 7 |
| 10.20 | Compliance Determination Procedures - Monitoring |  | 1 | 8 |
| 11.10 | Compliance Plan for Non-compliant Emission Unit or Process |  | 1 |  |
| 12.10 | Accidental Release Prevention Program |  | 5 |  |
| 13.00 | Certifications and Signatures |  | 1 |  |
| Totals: |  |  | 62 | 67 |

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For EPD Use Only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section 1.10 - Facility Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TV- |  |  |  |  |
| 1. PARENT/HOLDING COMPANY NAME: |  |  |  | Owens Corning |  |  |  |  |  | 2. FACILITY NAME: |  |  |  |  | Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |
| 3. FACILITY LOCATION: |  |  |  | Street |  | 4795 Frederick Drive |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | City |  | Atlanta |  |  |  |  | County |  | Fulton |  |  |  | ZIP | 30336 |  |
| 4. CONTACT A: |  |  |  | Name |  | Willie Peete- Roofing |  |  |  |  |  |  | Title |  |  | Plant Manager |  |  |  |
| (PLANT CONTACT) |  |  |  | Telephone |  | (404) 691-3910 ext. 3501 |  |  |  |  | Fax |  |  |  | (404) 699-9520 |  |  |  |  |
| 5. CONTACT B: |  |  |  | Name |  | Paul Lazarek - Asphalt (Plant Contact, also) |  |  |  |  |  |  | Title |  |  | Operations Manager |  |  |  |
|  |  |  |  | Telephone |  | (404) 691-3910 ext. 3502 |  |  |  |  | Fax |  |  |  | (404) 699-9520 |  |  |  |  |
| 6. CONTACT C: |  |  |  | Name |  | Jack Hepner |  |  |  |  |  |  | Title |  |  | Envir & Technical Specialist |  |  |  |
|  |  |  |  | Telephone |  | (404) 836-3506 |  |  |  |  | Fax |  |  |  | (404) 699-9520 |  |  |  |  |
| 7. CONTACT D: |  |  |  | Name |  |  |  |  |  |  |  |  | Title |  |  |  |  |  |  |
|  |  |  |  | Telephone |  |  |  |  |  |  | Fax |  |  |  |  |  |  |  |  |
| 8. MAILING ADDRESS 1: |  |  |  | Company Name / Department |  |  |  |  | Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  |  |  |
|  | (Plant Mail) |  |  | Street / P.O. Box |  |  | 4795 Frederick Drive |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | City |  | Atlanta |  |  |  |  |  | State |  |  | GA |  | ZIP | 30336 |  |
| 9. MAILING ADDRESS 2: |  |  |  | Company Name / Department |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Street / P.O. Box |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | City |  |  |  |  |  |  |  | State |  |  |  |  | ZIP |  |  |
| 10. MAILING ADDRESS 3: |  |  |  | Company Name / Department |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Street / P.O. Box |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | City |  |  |  |  |  |  |  | State |  |  |  |  | ZIP |  |  |
| 11. REASON FOR APPLICATION SUBMITTAL: (Answer all that apply) |  |  |  |  |  | Update to Initial Application: X |  |  |  |  |  |  |  |  | Existing Major Source Requesting Synthetic Minor Permit for: |  |  |  |  |
|  |  | Initial Title V Operating Permit |  |  |  | Initial App. Date |  |  |  |  |  |  |  |  |  | All Pollutants |  |  |  |
|  |  |  |  |  |  | EPD Log # |  |  | TV - |  |  |  |  |  |  | Specific Pollutant(s) |  |  |  |
| 12. APPLICATION SUBMITTED FOR: (All applications for one Part 70 site must be submitted at the same time.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | All facilities under common control at a Part 70 site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Title V application for part of a Part 70 site. Describe: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Section 1.10 - Facility Information (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13. MAILING INFORMATION: |  |  |  |  | Using the contact and mailing address information given on page 1 of this section, identify to whom and where the following correspondence should be sent (only one contact/address per line). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CORRESPONDENCE RELATING TO: |  |  |  |  |  |  |  | Primary Contact |  |  |  |  |  | Mailing Address |  |  |  |  |
|  |  |  |  |  |  |  |  |  | (A, B, C, OR D) |  |  |  |  |  | (1,2, OR 3) |  |  |  |  |
|  | Facility (mail that must reach the plant site) |  |  |  |  |  |  |  |  | A |  |  |  |  |  | 1 |  |  |  |
|  | Parent Company / Legal Owner (legal actions, etc.) |  |  |  |  |  |  |  |  | A |  |  |  |  |  | 1 |  |  |  |
|  | Permits (granted permits, permit amendments, etc.) |  |  |  |  |  |  |  |  | A |  |  |  |  |  | 1 |  |  |  |
|  | Permit Applications (blank forms, requests for additional information, etc.) |  |  |  |  |  |  |  |  | A |  |  |  |  |  | 1 |  |  |  |
|  | Surveys, questionnaires (emission inventories, etc.) |  |  |  |  |  |  |  |  | A |  |  |  |  |  | 1 |  |  |  |
|  | Enforcement Actions (non-compliance letters, notices of violations, etc.) |  |  |  |  |  |  |  |  | A |  |  |  |  |  | 1 |  |  |  |
|  | Fees (fee manuals, fee forms, audit notices, etc.) |  |  |  |  |  |  |  |  | A |  |  |  |  |  | 1 |  |  |  |
|  | Monitoring (CEM certification applications, requests for monitoring testing information, etc.) |  |  |  |  |  |  |  |  | A |  |  |  |  |  | 1 |  |  |  |
| 14. SIC OF MAJOR PRODUCT |  |  |  |  | 2952; 2951 |  |  |  | 15. FEI NUMBER |  |  | 34-4323452 |  |  |  |  |  |  |  |
| 16. AIRS NUMBER |  |  |  | 04-13- |  | 12100020 |  |  | 17. DUN AND BRADSTREET NUMBER |  |  |  |  |  |  | 086-961844 |  |  |  |
| 18. LOCATION OF CENTER OF PRODUCTION AREA (use either UTM or Latitude/Longitude): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | UTM Zone |  |  |  | 0016 |  |  |  |  | Latitude |  |  |  |  |  | ' |  | " |  |
|  | UTM Vertical Meters |  |  |  | 3738606 |  |  | or |  | Longitude |  |  |  |  |  | ' |  | " |  |
|  | UTM Horizontal Meters |  |  |  | 727312 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19. LIST ALL CURRENT STATE AND FEDERAL AIR QUALITY PERMITS, AND DATES (ORIGINAL ISSUE AND AMENDMENT): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Air Quality Permit |  |  |  |  |  |  |  | Original Issue and Amendment Date |  |  |  |  |  |  |  |  |  |  |  |
| 2952-121-0334-V-01-0 |  |  |  |  |  |  |  | May 25, 2000; |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | May 23, 2001 |  |  |  |  |  |  |  |  |  |  |  |
| Section 1.10 - Facility Information (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20. FACILITY PRODUCT(S): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Primary Product(s) of this Facility |  |  |  |  |  |  | SIC Code |  |  | Brief Description of Manufacturing Process |  |  |  |  |  |  |  |  |  |
| Trumbull Plant - Oxidized Asphalt |  |  |  |  |  |  | 2952 |  |  | Asphalt Felt and Coatings |  |  |  |  |  |  |  |  |  |
| Roofing Plant - Roofing Products-shingles/rolls |  |  |  |  |  |  | 2952 |  |  | Asphalt Felt and Coatings |  |  |  |  |  |  |  |  |  |
| Trumbull Plant - Asphalt Paving Mixtures |  |  |  |  |  |  | 2951 |  |  | Asphalt Paving Mixtures |  |  |  |  |  |  |  |  |  |
| 21. COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

State of Georgia

Department of Natural Resources

Environmental Protection Division

Air Protection Branch

Major Source Operating Permit Application

Atlanta Tradeport, Suite 120

4244 International Parkway

Atlanta, Georgia 30354-3906

404/363-7000

Rev. 4/15/96 Page 1 of 1 Section 1.20

Section 1.20 - Facility Information - Required Documents

- FACILITY NAME:   Owens Corning

2. SUBMIT THE FOLLOWING INFORMATION WITH APPLICATION:

(Indicate number of corresponding attachments in space provided.)

Building layout (overhead view), indicating location of Emission Units and Stacks.

(Only stacks that are listed in Section 9.10 need to be located on the layout.)

Plant site map detailing property lines, latitude/longitude or UTM location from Section 1.10, any outdoor storage piles (indicating material), roads (indicate paved or unpaved), and areas of company property to which the public has unrestricted access.

General area map showing specific location of plant in relation to surrounding area.

XX  Process flow diagram(s) for the plant, including emission flow.

(These do not have to include material input/output flow rates.)

(Requested documents may be combined or seperated so long as all required information is included)

3. (optional) RECORDKEEPING DOCUMENTATION:

Attach all current and proposed forms required to comply with applicable recordkeeping. (optional)

4. KEEP THE FOLLOWING INFORMATION AT THE PLANT, TOGETHER, IN A FORM SUITABLE FOR INSPECTION, REVIEW OR

SUBMITTAL: (Place ‘X’ in box when completed.)

All calculations and records on which emissions information is based.

(e.g. Material Safety Data Sheets, emission calculations)

5. COMMENTS:

SCENARIO #1

SCENARIO #2

SCENARIO #3

| Section 3.30 - Proposed Title V Permit Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. FACILITY NAME: |  | Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  |
| 2. LIST PROPOSED TITLE V PERMIT CONDITIONS AND THEIR APPLICABLE AUTHORITY: |  |  |  |  |  |  |  |  |  |  |
| Emission Unit, or Group ID # | Requested Permit Condition |  | Indicate Type of Condition1 |  |  |  | Applicable Authority |  | Permit Shield (Y/N) | Reason for Requested Permit Condition |
|  |  |  |  |  |  |  | State / Federal Rule (list rule) | Existing Permit Condition(s) (List Permit#, Amend. Date, and Cond#) |  |  |
|  |  |  | E | M | H | N |  |  |  |  |
| IB01 | Fume Incinerator (IB01) controlling emissions from Convertors #2 and #3 with RTO #3 controlling Convertor #1, (this scenario is a normal operating procedure). |  |  |  |  | x | N/A |  | Y | This needs to be changed due to better control of emissions in the event of a control device malfunction. |
| IB01 | Fume Incinerator (IB01) controlling emissions from Convertors #1 and #3 or Convertors #2 and #3 and RTO #1 controlling emissions from loading station in addition to other sources permitted to be controlled by RTO #1. (Scenario #2) |  |  |  |  | X | N/A |  | Y | This needs to be changed due to better control of emissions in the event of a control device malfunction. |
| RTO#3 | RTO #3 controlling emissions from Convertor #1 or Convertor #2 or Convertor #3 and the loading rack fumes when the Fume Incinerator is down. (Scenario #3) |  |  |  |  | # X | N/A |  | Y | This needs to be changed due to better control of emissions in the event of a control device malfunction. |
| EU06 | Conduct stack testing for opacity and particulate matter while operating Convertors #2 and #3 and venting emissions to IB01 (Stack 082) |  |  |  |  | # X | N/A |  | Y | To demonstrate compliance with 40 CFR Part 60 Subpart UU requirements. |
| EU06 EU07 | Conduct stack testing for opacity and particulate matter while operating Convertor #2 and loading asphalt from the 3 loading arms and venting emissions to RTO3 (Stack 083) |  |  |  |  | # X | N/A |  | Y | To demonstrate compliance with 40 CFR Part 60 Subpart UU requirements. |
| EU03 EU07 | Conduct stack testing for opacity while loading asphalt from one loading arm and venting emissions to RTO1 (Stack A105) |  |  |  |  | # X | N/A |  | Y | To demonstrate compliance with 40 CFR Part 60 Subpart UU requirements. |

| Emission Unit, or Group ID # | Requested Permit Condition | Indicate Type of Condition1 |  |  |  | Applicable Authority |  | Permit Shield (Y/N) | Reason for Requested Permit Condition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | State / Federal Rule (list rule) | Existing Permit Condition(s) (List Permit#, Amend. Date, and Cond#) |  |  |
|  |  | E | M | H | N |  |  |  |  |
| EU03 | 7.90 tons per year VOC total for emission unit |  |  |  | # X | N/A |  | Y | Per GADNR request |
| EU 05.1 and 05.3 | 1.3 tons per year VOC total |  |  |  | # X | N/A |  | Y | Per GADNR request |
| 07.1 | 1.04 tons per year VOC |  |  |  | # X | N/A |  | Y | Per GADNR request |
| 08.1 | 7.72 tons per year VOC total |  |  |  | # X | N/A |  | Y | Per GADNR request |
| RTO1, RTO2, RTO3, IB01 | Add “while control device is in operation controlling emissions” to each section listed |  | # X |  |  | 391-3-1-02(6)(b)1 and 40 CFR 70.6(a) (3)(i) | 2952-121-0334-V-01, 5/25/01, 5.2.6 c I through iii | Y | Per GADNR request |
| 1Indicate origin of Requested Permit Condition, Existing permit condition, Modified permit condition, Hybrid (combination of several existing permit conditions), or New permit condition) |  |  |  |  |  |  |  |  |  |

| Section 5.90 - Emission Units - Miscellaneous |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative Operating Scenario |  |  |  |  |  |  |  |  |  |  |  |  | #1 (Normal) |  |  |
| 1. FACILITY NAME: | Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. EMISSION UNIT ID NUMBER: |  |  | 3. FACILITY’S NAME FOR THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |
| 06.1 |  |  | # Convertor No. 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. BRIEF DESCRIPTION OF THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This unit is used to oxidize asphalt. - Located in Asphalt Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. NAME FOR ASSOCIATED AIR POLLUTION CONTROL DEVICE: |  |  |  |  |  |  | 6. AIR POLLUTION CONTROL DEVICE ID NUMBER: |  |  |  |  |  |  |  |  |
| Regenerative Thermal Oxidizer No. 3 |  |  |  |  |  |  | RTO3 |  |  |  |  |  |  |  |  |
| 7. EMISSION UNIT MANUFACTURER AND MODEL NUMBER: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unknown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. EMISSION UNIT DATE OF MANUFACTURE / RECONSTRUCTION: |  |  |  |  |  | 9. EMISSION UNIT EFFECTIVE DATE OF INSTALLATION: |  |  |  |  |  |  |  |  |  |
| 1983 |  |  |  |  |  | December 1983 |  |  |  |  |  |  |  |  |  |
| 10. DESCRIBE ANY FUGITIVE EMISSIONS ASSOCIATED WITH THIS UNIT, SUCH AS OUTDOOR STORAGE PILES, UNPAVED ROADS, OPEN CONVEYORS, AND EQUIPMENT LEAKS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| There are no fugitive emissions associated with this emission unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11. LIST THE TYPES AND AMOUNTS OF SIGNIFICANT INPUT MATERIAL TO THIS UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material |  | CAS Number |  |  | Maximum Hourly Input |  |  | Maximum Annual Input |  |  |  |  |  | Average Free Moisture Content (% mass) |  |
|  |  |  |  |  |  | Units |  |  |  |  | Units |  |  |  |  |
| 12. LIST PRODUCTS FROM THIS UNIT: (Note: this information is optional if it is not needed to determine applicability and regulatory requirements.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product |  | CAS Number |  |  | Maximum Hourly Output |  |  | Maximum Annual Output |  |  |  |  |  | Average Free Moisture Content (% mass) |  |
|  |  |  |  |  |  | Units |  |  |  |  | Units |  |  |  |  |
| Asphalt-High softening pt |  | 8502-42-4 |  |  | 93,264 | tons/hr |  | 381,826 |  |  | tons/yr |  |  | N/A |  |
| Asphalt-Low softening pt |  | 8502-42-4 |  |  | 220,200 | tons/hr |  | 925,029 |  |  | tons/yr |  |  | N/A |  |
| Section 5.90 - Emissions Units - Miscellaneous (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternative Operating Scenario # |  |  |  |  |  |  |  |  |  |  |  |  |  | # 1 (Normal) |  |
| FACILITY NAME: |  |  |  |  |  |  |  |  | EMISSION UNIT ID NUMBER: |  |  |  |  |  |  |
| Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  | 06.1 |  |  |  |  |  |  |
| 13. FUELS AND FIRING CONDITIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Type and Grade |  | Max Hourly Fuel Consumption |  |  | Max Heat Input (Million Btu/hr) | Max Annual Fuel Consumption |  |  |  | Maximum Heating Value |  |  |  |  | Max Allowable % Sulfur |
|  |  |  |  | Units |  |  |  | Units |  |  |  | Units |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. GIVE A BRIEF DESCRIPTION OF THIS EMISSION UNIT: [Include any additional information necessary to determine applicability and regulatory requirements not specifically asked for in this section (5.90).] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This unit is used to oxidize asphalt. – Asphalt Plant This unit is included in Scenario #1 – Fume Incinerator controls Convertors #2 and #3 and RTO #3 controls Convertor #1 and Loading Station fumes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The maximum hourly and annual rates indicated are for Convertors No. 1, No. 2 and No. 3.

| Section 5.90 - Emission Units - Miscellaneous |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative Operating Scenario |  |  |  |  |  |  |  |  |  |  |  |  | #1 (Normal) |  |  |
| 1. FACILITY NAME: | Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. EMISSION UNIT ID NUMBER: |  |  | 3. FACILITY’S NAME FOR THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |
| 06.2 |  |  | Convertor No. 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. BRIEF DESCRIPTION OF THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This unit is used to oxidize asphalt. - Located in Asphalt Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. NAME FOR ASSOCIATED AIR POLLUTION CONTROL DEVICE: |  |  |  |  |  |  | 6. AIR POLLUTION CONTROL DEVICE ID NUMBER: |  |  |  |  |  |  |  |  |
| Fume Incinerator/Waste Heat Boiler |  |  |  |  |  |  | IB01 |  |  |  |  |  |  |  |  |
| 7. EMISSION UNIT MANUFACTURER AND MODEL NUMBER: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unknown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. EMISSION UNIT DATE OF MANUFACTURE / RECONSTRUCTION: |  |  |  |  |  | 9. EMISSION UNIT EFFECTIVE DATE OF INSTALLATION: |  |  |  |  |  |  |  |  |  |
| 1983 |  |  |  |  |  | December 1983 |  |  |  |  |  |  |  |  |  |
| 10. DESCRIBE ANY FUGITIVE EMISSIONS ASSOCIATED WITH THIS UNIT, SUCH AS OUTDOOR STORAGE PILES, UNPAVED ROADS, OPEN CONVEYORS, AND EQUIPMENT LEAKS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| There are no fugitive emissions associated with this emission unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11. LIST THE TYPES AND AMOUNTS OF SIGNIFICANT INPUT MATERIAL TO THIS UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material |  | CAS Number |  |  | Maximum Hourly Input |  |  | Maximum Annual Input |  |  |  |  |  | Average Free Moisture Content (% mass) |  |
|  |  |  |  |  |  | Units |  |  |  |  | Units |  |  |  |  |
| 12. LIST PRODUCTS FROM THIS UNIT: (Note: this information is optional if it is not needed to determine applicability and regulatory requirements.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product |  | CAS Number |  |  | Maximum Hourly Output |  |  | Maximum Annual Output |  |  |  |  |  | Average Free Moisture Content (% mass) |  |
|  |  |  |  |  |  | Units |  |  |  |  | Units |  |  |  |  |
| Asphalt-High softening pt |  | 8502-42-4 |  |  | 93,264 | tons/hr |  | 381,826 |  |  | tons/yr |  |  | N/A |  |
| Asphalt-Low softening pt |  | 8502-42-4 |  |  | 220,200 | tons/hr |  | 925,029 |  |  | tons/yr |  |  | N/A |  |
| Section 5.90 - Emissions Units - Miscellaneous (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternative Operating Scenario # |  |  |  |  |  |  |  |  |  |  |  |  |  | #1 (Normal) |  |
| FACILITY NAME: |  |  |  |  |  |  |  |  | EMISSION UNIT ID NUMBER: |  |  |  |  |  |  |
| Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  | 06.2 |  |  |  |  |  |  |
| 13. FUELS AND FIRING CONDITIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Type and Grade |  | Max Hourly Fuel Consumption |  |  | Max Heat Input (Million Btu/hr) | Max Annual Fuel Consumption |  |  |  | Maximum Heating Value |  |  |  |  | Max Allowable % Sulfur |
|  |  |  |  | Units |  |  |  | Units |  |  |  | Units |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. GIVE A BRIEF DESCRIPTION OF THIS EMISSION UNIT: [Include any additional information necessary to determine applicability and regulatory requirements not specifically asked for in this section (5.90).] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This unit is used to oxidize asphalt. – Asphalt Plant This unit is included in Scenario #1 – Fume Incinerator controls Convertors #2 and #3 and RTO #3 controls Convertor #1 and Loading Station fumes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The maximum hourly and annual rates indicated are for Convertors No. 1, No. 2 and No. 3.

| Section 5.90 - Emission Units - Miscellaneous |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative Operating Scenario |  |  |  |  |  |  |  |  |  |  |  |  | #1 (Normal) |  |  |  |
| 1. FACILITY NAME: | Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. EMISSION UNIT ID NUMBER: |  |  | 3. FACILITY’S NAME FOR THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 06.3 |  |  | Convertor No. 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. BRIEF DESCRIPTION OF THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This unit is used to oxidize asphalt. - Located in Asphalt Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. NAME FOR ASSOCIATED AIR POLLUTION CONTROL DEVICE: |  |  |  |  |  |  | 6. AIR POLLUTION CONTROL DEVICE ID NUMBER: |  |  |  |  |  |  |  |  |  |
| Fume Incinerator/Waste Heat Boiler |  |  |  |  |  |  | IB01 |  |  |  |  |  |  |  |  |  |
| 7. EMISSION UNIT MANUFACTURER AND MODEL NUMBER: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unknown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. EMISSION UNIT DATE OF MANUFACTURE / RECONSTRUCTION: |  |  |  |  |  | 9. EMISSION UNIT EFFECTIVE DATE OF INSTALLATION: |  |  |  |  |  |  |  |  |  |  |
| December 1998 |  |  |  |  |  | January 29, 1999 |  |  |  |  |  |  |  |  |  |  |
| 10. DESCRIBE ANY FUGITIVE EMISSIONS ASSOCIATED WITH THIS UNIT, SUCH AS OUTDOOR STORAGE PILES, UNPAVED ROADS, OPEN CONVEYORS, AND EQUIPMENT LEAKS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| There are no fugitive emissions associated with this emission unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11. LIST THE TYPES AND AMOUNTS OF SIGNIFICANT INPUT MATERIAL TO THIS UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material |  | CAS Number |  |  | Maximum Hourly Input |  |  | Maximum Annual Input |  |  |  |  |  |  | Average Free Moisture Content (% mass) |  |
|  |  |  |  |  |  | Units |  |  |  |  | Units |  |  |  |  |  |
| 12. LIST PRODUCTS FROM THIS UNIT: (Note: this information is optional if it is not needed to determine applicability and regulatory requirements.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product |  | CAS Number |  |  | Maximum Hourly Output |  |  | Maximum Annual Output |  |  |  |  |  |  | Average Free Moisture Content (% mass) |  |
|  |  |  |  |  |  | Units |  |  |  |  | Units |  |  |  |  |  |
| Asphalt-High softening pt |  | 8502-42-4 |  |  | 93,264 | tons/hr |  | 381,826 |  |  | tons/yr |  |  |  | N/A |  |
| Asphalt-Low softening pt |  | 8502-42-4 |  |  | 220,200 | tons/hr |  | 925,029 |  |  | tons/yr |  |  |  | N/A |  |
| Section 5.90 - Emissions Units - Miscellaneous (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternative Operating Scenario |  |  |  |  |  |  |  |  |  |  |  |  |  | #1 (Normal) |  |  |
| FACILITY NAME: |  |  |  |  |  |  |  |  | EMISSION UNIT ID NUMBER: |  |  |  |  |  |  |  |
| Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  | 06.3 |  |  |  |  |  |  |  |
| 13. FUELS AND FIRING CONDITIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Type and Grade |  | Max Hourly Fuel Consumption |  |  | Max Heat Input (Million Btu/hr) | Max Annual Fuel Consumption |  |  |  | Maximum Heating Value |  |  |  |  |  | Max Allowable % Sulfur |
|  |  |  |  | Units |  |  |  | Units |  |  |  | Units |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. GIVE A BRIEF DESCRIPTION OF THIS EMISSION UNIT: [Include any additional information necessary to determine applicability and regulatory requirements not specifically asked for in this section (5.90).] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This unit is used to oxidize asphalt. – Asphalt Plant This unit is included in Scenario #1 – Fume Incinerator controls Convertors #2 and #3 and RTO #3 controls Convertor #1 and Loading Station fumes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The maximum hourly and annual rates indicated are for Convertors No. 1, No. 2 and No. 3.

| Section 5.90 - Emission Units - Miscellaneous |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative Operating Scenario |  |  |  |  |  |  |  |  |  |  |  |  | #1 (Normal) |  |  |
| 1. FACILITY NAME: | Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. EMISSION UNIT ID NUMBER: |  |  | 3. FACILITY’S NAME FOR THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |
| 07.1 |  |  | Loading Station |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. BRIEF DESCRIPTION OF THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This loading rack is used to load asphalt into trucks. - Located in Asphalt Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. NAME FOR ASSOCIATED AIR POLLUTION CONTROL DEVICE: |  |  |  |  |  |  | 6. AIR POLLUTION CONTROL DEVICE ID NUMBER: |  |  |  |  |  |  |  |  |
| Regenerative Thermal Oxidizer No. 3 |  |  |  |  |  |  | RTO3 |  |  |  |  |  |  |  |  |
| 7. EMISSION UNIT MANUFACTURER AND MODEL NUMBER: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unknown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. EMISSION UNIT DATE OF MANUFACTURE / RECONSTRUCTION: |  |  |  |  |  | 9. EMISSION UNIT EFFECTIVE DATE OF INSTALLATION: |  |  |  |  |  |  |  |  |  |
| 1962 |  |  |  |  |  | 1962 |  |  |  |  |  |  |  |  |  |
| 10. DESCRIBE ANY FUGITIVE EMISSIONS ASSOCIATED WITH THIS UNIT, SUCH AS OUTDOOR STORAGE PILES, UNPAVED ROADS, OPEN CONVEYORS, AND EQUIPMENT LEAKS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fugitive emissions are contributed from the collection hood. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11. LIST THE TYPES AND AMOUNTS OF SIGNIFICANT INPUT MATERIAL TO THIS UNIT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material |  | CAS Number |  |  | Maximum Hourly Input |  |  | Maximum Annual Input |  |  |  |  |  | Average Free Moisture Content (% mass) |  |
|  |  |  |  |  |  | Units |  |  |  |  | Units |  |  |  |  |
| 12. LIST PRODUCTS FROM THIS UNIT: (Note: this information is optional if it is not needed to determine applicability and regulatory requirements.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product |  | CAS Number |  |  | Maximum Hourly Output |  |  | Maximum Annual Output |  |  |  |  |  | Average Free Moisture Content (% mass) |  |
|  |  |  |  |  |  | Units |  |  |  |  | Units |  |  |  |  |
| Asphalt |  | 8502-42-4 |  |  | 30,000 | lbs/hr |  | 285,568 |  |  | tons/yr |  |  | N/A |  |
| Section 5.90 - Emissions Units - Miscellaneous (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternative Operating Scenario # |  |  |  |  |  |  |  |  |  |  |  |  |  | #1 (Normal) |  |
| FACILITY NAME: |  |  |  |  |  |  |  |  | EMISSION UNIT ID NUMBER: |  |  |  |  |  |  |
| Owens Corning, Atlanta Roofing and Asphalt Plant |  |  |  |  |  |  |  |  | 07.1 |  |  |  |  |  |  |
| 13. FUELS AND FIRING CONDITIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Type and Grade |  | Max Hourly Fuel Consumption |  |  | Max Heat Input (Million Btu/hr) | Max Annual Fuel Consumption |  |  |  | Maximum Heating Value |  |  |  |  | Max Allowable % Sulfur |
|  |  |  |  | Units |  |  |  | Units |  |  |  | Units |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. GIVE A BRIEF DESCRIPTION OF THIS EMISSION UNIT: [Include any additional information necessary to determine applicability and regulatory requirements not specifically asked for in this section (5.90).] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This loading rack is used to transfer asphalt to trucks for off-site shipment. This unit is included in Scenario #1 – Fume Incinerator controls Convertors #2 and #3 and RTO #3 controls Convertor #1 and Loading Station fumes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |