| Section 5.10 - Emission Unit - Boilers, Furnaces, and Other Indirect Contact Heat Generating Equipment |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative Operating Scenario # |  |  |  |  |  |  |  |  |  |  |  |
| 1. FACILITY NAME: | Inland Paperboard and Packaging, Inc. - Rome Linerboard Mill |  |  |  |  |  |  |  |  |  |  |
| 2. EMISSION UNIT ID NUMBER: | 3. FACILITY’S NAME FOR THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |
| PB4 | Power Boiler 4 |  |  |  |  |  |  |  |  |  |  |
| 4. DESCRIPTION OF EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |
| Power Boiler 4 - used for process steam |  |  |  |  |  |  |  |  |  |  |  |
| 5. NAME FOR ASSOCIATED AIR POLLUTION CONTROL DEVICE: |  |  |  | 6. AIR POLLUTION CONTROL DEVICE ID NUMBER: |  |  |  |  |  |  |  |
| Multiclone 4, precipitator 6 |  |  |  | PB4M, ESP6 |  |  |  |  |  |  |  |
| 7. EMISSION UNIT MANUFACTURER AND MODEL NUMBER: |  |  |  |  |  |  |  |  |  |  |  |
| Combustion Engineering 19463 |  |  |  |  |  |  |  |  |  |  |  |
| 8. EMISSION UNIT DATE OF MANUFACTURE / RECONSTRUCTION: |  |  |  | 9. EMISSION UNIT EFFECTIVE DATE OF INSTALLATION: |  |  |  |  |  |  |  |
| 1962 |  |  |  | 1962 |  |  |  |  |  |  |  |
| 10. DESCRIBE FUEL BURNING CONFIGURATION: |  |  |  |  |  |  |  |  |  |  |  |
| Low NOx, Pulverized coal, and fuel oil burner |  |  |  |  |  |  |  |  |  |  |  |
| 11. EMISSION UNIT HEAT INPUT CAPACITY: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 565 |  | million Btu/hr |  |  |  |  |  |  |
| 12. FUELS AND FIRING CONDITIONS: |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Type and Grade | Max Hourly Fuel Consumption |  | Max Heat Input (Million Btu/hr) |  |  | Max Annual Fuel Consumption |  |  | Maximum Heating Value |  | Max Allowable % Sulfur |
|  |  | Units |  |  |  |  | Units |  |  | Units |  |
| Coal (1.0% Sulfur) | 21.899 | tons | 565 |  |  | 191,837 | tons |  | 12,900 | Btu/lb | 1 |
| Coal (1.29% Sulfur) | 21,899 | Tons | 565 |  |  | 148,711 | Tons |  | 12,900 | Btu/gal | 1.29 |
| No. 2/5 Fuel Oil | 4,035 | gal | 565 |  |  | 35,352,857 | gal |  | 140,000 | Btu/gal | 0.5 |
| 13. COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |

| Section 5.10 - Emission Unit - Boilers, Furnaces, and Other Indirect Contact Heat Generating Equipment |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative Operating Scenario # |  |  |  |  |  |  |  |  |  |  |  |
| 1. FACILITY NAME: | Inland Paperboard and Packaging, Inc. - Rome Linerboard Mill |  |  |  |  |  |  |  |  |  |  |
| 2. EMISSION UNIT ID NUMBER: | 3. FACILITY’S NAME FOR THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |
| WF | Waste Fuel Boiler |  |  |  |  |  |  |  |  |  |  |
| 4. DESCRIPTION OF EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |  |
| The Waste Fuel Boiler is used to produce steam for power production and for process use. |  |  |  |  |  |  |  |  |  |  |  |
| 5. NAME FOR ASSOCIATED AIR POLLUTION CONTROL DEVICE: |  |  |  | 6. AIR POLLUTION CONTROL DEVICE ID NUMBER: |  |  |  |  |  |  |  |
| Electrostatic Precipitator 3 |  |  |  | ESP3 |  |  |  |  |  |  |  |
| 7. EMISSION UNIT MANUFACTURER AND MODEL NUMBER: |  |  |  |  |  |  |  |  |  |  |  |
| Combustion Engineering 19457 |  |  |  |  |  |  |  |  |  |  |  |
| 8. EMISSION UNIT DATE OF MANUFACTURE / RECONSTRUCTION: |  |  |  | 9. EMISSION UNIT EFFECTIVE DATE OF INSTALLATION: |  |  |  |  |  |  |  |
| 2004 |  |  |  | 2004 |  |  |  |  |  |  |  |
| 10. DESCRIBE FUEL BURNING CONFIGURATION: |  |  |  |  |  |  |  |  |  |  |  |
| Fluidized Bed |  |  |  |  |  |  |  |  |  |  |  |
| 11. EMISSION UNIT HEAT INPUT CAPACITY: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 856 |  | million Btu/hr |  |  |  |  |  |  |
| 12. FUELS AND FIRING CONDITIONS: |  |  |  |  |  |  |  |  |  |  |  |
| Fuel Type and Grade | Max Hourly Fuel Consumption |  | Max Heat Input (Million Btu/hr) |  |  | Max Annual Fuel Consumption |  |  | Maximum Heating Value |  | Max Allowable % Sulfur |
|  |  | Units |  |  |  |  | Units |  |  | Units |  |
| Bark / Wood Waste (as received) | 120.70 | tons | 856 |  |  | 1,057,327 | tons |  | 4,433 | Btu/lb | 0 |
| Tire-Derived Fuel (as received) | 6,667 | lb | 113 |  |  | 29,200 | tons |  | 17,000 | Btu/lb | 2.5 |
| Wastewater Sludge | 2,500 | dry lb | 21.25 |  |  | 10,950 | dry tons |  | 8,500 | Btu/lb (dry) | 1.5 |
| Fuel Oil (as received) | 6,114 | gal | 856 |  |  | 1,100,000 | gal |  | 140,000 | Btu/gal | 0.5 |
| 13. COMMENTS: The maximum hourly and annual fuel consumption of bark is based on the minimum heat content of bark (3,546 Btu/lb). |  |  |  |  |  |  |  |  |  |  |  |

| Section 5.10 - Emission Unit - Boilers, Furnaces, and Other Indirect Contact Heat Generating Equipment |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alternative Operating Scenario # |  |  |  |  |  |  |  |  |  |  |
| 1. FACILITY NAME: | Inland Paperboard and Packaging, Inc. - Rome Linerboard Mill |  |  |  |  |  |  |  |  |  |
| 2. EMISSION UNIT ID NUMBER: | 3. FACILITY’S NAME FOR THIS EMISSION UNIT: |  |  |  |  |  |  |  |  |  |
| PK2 | Package Boiler |  |  |  |  |  |  |  |  |  |
| 4. DESCRIPTION OF EMISSION UNIT: |  |  |  |  |  |  |  |  |  |  |
| Package Boiler used for the production of steam |  |  |  |  |  |  |  |  |  |  |
| 5. NAME FOR ASSOCIATED AIR POLLUTION CONTROL DEVICE: |  |  |  | 6. AIR POLLUTION CONTROL DEVICE ID NUMBER: |  |  |  |  |  |  |
| N/A |  |  |  | N/A |  |  |  |  |  |  |
| 7. EMISSION UNIT MANUFACTURER AND MODEL NUMBER: |  |  |  |  |  |  |  |  |  |  |
| Nationwide Boiler Inc. |  |  |  |  |  |  |  |  |  |  |
| 8. EMISSION UNIT DATE OF MANUFACTURE / RECONSTRUCTION: |  |  |  | 9. EMISSION UNIT EFFECTIVE DATE OF INSTALLATION: |  |  |  |  |  |  |
| 2001 |  |  |  | 2001 |  |  |  |  |  |  |
| 10. DESCRIBE FUEL BURNING CONFIGURATION: |  |  |  |  |  |  |  |  |  |  |
| Low NOx Burner |  |  |  |  |  |  |  |  |  |  |
| 11. EMISSION UNIT HEAT INPUT CAPACITY: |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 192 |  | million Btu/hr |  |  |  |  |  |
| 12. FUELS AND FIRING CONDITIONS: |  |  |  |  |  |  |  |  |  |  |
| Fuel Type and Grade | Max Hourly Fuel Consumption |  | Max Heat Input (Million Btu/hr) | Max Annual Fuel Consumption |  |  | Maximum Heating Value |  |  | Max Allowable % Sulfur |
|  |  | Units |  |  |  | Units |  |  | Units |  |
| Natural Gas | 184,000 | scf | 184 | 1,612 |  | Mmscf | 1,000 |  | Btu/scf | -- |
| No. 2 Fuel Oil | 1,371 | Gal | 192 | 3,000,000 |  | gal | 140,000 |  | Btu/gal | 0.5 |
| 13. COMMENTS: |  |  |  |  |  |  |  |  |  |  |