Georgia SIP Air Permit Application

# Instructions for the completion of Form 3.00 Air Pollution Control Device Information

This form is to be completed and submitted with any Georgia SIP Air Permit application that involves the construction, operation, and/or modification of an Air Pollution Control Device (APCD).  The latest version of Form 3.00 is located at http://epd.georgia.gov/air/documents/ssppsip-app-form-300-air-pollution-control-device-information.  Do not submit instructions with the SIP application.

This form provides general information about APCDs and should be used for all devices including those APCDs with unique forms.  Additional forms are necessary for scrubbers (Form 3.01), baghouses/filters (Form 3.02), and electrostatic precipitators (Form 3.03).

| Air Pollution Control Devices – Part A: General Equipment Information |  |
| --- | --- |
| Part A – Equipment Information. Complete one row for each air pollution control device. |  |
| APCD Unit ID: | The identification of an APCD unit is an alpha-numeric code with a maximum of four characters (e.g. 27, D1, BH, BH1, ESP1). APCD IDs are unique to each piece of equipment at a facility; emission units, air pollution control devices, and stacks cannot share the same ID. |
| Emission Unit ID: | Form 2.00 Emission Unit List contains a list of the emission units for the facility. Enter the corresponding emission unit that is being controlled by the APCD. If the APCD controls emissions from multiple emission units, enter the Emission Unit IDs for all of the emission units controlled by this APCD. |
| APCD Type: | Indicate the general type of air pollution control device. More detailed information for each APCD should be entered on the corresponding Forms 3.01 through 3.03 for that type of control device. |
| Construction Date: | This should be the date that the APCD was constructed or installed on the emissions unit or units that it currently controls. |
| Make & Model Number: | Enter the make and model number of the APCD. Any available literature, including manufacturer’s specifications, that provides detailed information on the APCD, should be attached to or included in the list in Form 1.00 General Information, Item 16. Indicate if the APCD was not commercially manufactured but was custom fabricated. |
| Unit Modified from Mfg Specifications: | If the APCD was commercially manufactured, indicate whether or not the device has been modified from the original manufacturer’s specifications. If the answer is yes, attached an explanation of the modification, why the modification was made, and the effect the modification has on air pollution collection efficiency. Include any attachments and list them in Form 1.00 General Information, Item 16. |
| Gas Temperature: | This is the inlet and outlet gas temperature in degree Fahrenheit to and from the APCD. |
| Inlet Gas Flow Rate: | Inlet gas flow rate to the APCD in actual cubic feet per minute. |
| Air Pollution Control Devices – Part B: Emission Information |  |
| Part B – Emission Information. Some air pollution control devices are used to reduce multiple pollutants. If this is the case, complete one row for each pollutant controlled by each APCD. |  |
| APCD Unit ID: | This should correspond to the APCD(s) listed on Part A of this form. |
| Pollutants Controlled: | List each air pollutant that is controlled by the APCD on a separate line. |
| Percent Control Efficiency: | List the design and actual control efficiency (in percent) for each pollutant controlled by the APCD. |
| Inlet Stream to APCD: | For each pollutant, enter the uncontrolled mass rate, in pounds per hour, of the pollutant as it enters the APCD. Specify the method that was used to determine this rate, e.g., stack test, AP-42 calculation, mass balance, etc. |
| Exit Stream from APCD: | For each pollutant, enter the controlled mass emissions rate, in pounds per hour as it exits the APCD. Specify the method that was used to determine this rate, e.g., stack test, AP-42 calculation, mass balance, etc. |
| Pressure Drop Across Unit: | If applicable, enter the gas phase pressure drop, in inches of water, across the APCD. |

| Facility Name: |  |  |  |  |  |  | Date of Application: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Form 3.00 – AIR POLLUTION CONTROL DEVICES - PART A: GENERAL EQUIPMENT INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| APCD Unit ID | Emission Unit ID |  | APCD Type (Baghouse, ESP, Scrubber etc) | Date Installed | Make & Model Number (Attach Mfg. Specifications & Literature) | Unit Modified from Mfg Specifications? |  | Gas Temp. F |  |  | Inlet Gas Flow Rate (acfm) |
|  |  |  |  |  |  |  |  | # Inlet |  | Outlet |  |

| Facility Name: |  |  |  |  |  |  | Date of Application: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Form 3.00 – AIR POLLUTION CONTROL DEVICES – PART B: EMISSION INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| APCD Unit ID | Pollutants Controlled |  | Percent Control Efficiency |  | Inlet Stream To APCD |  |  | Exit Stream From APCD |  |  | Pressure Drop Across Unit (Inches of water) |
|  |  |  | # Design | Actual | lb/hr | Method of Determination |  | lb/hr | Method of Determination |  |  |